3GPP TSG-SA WG6 Meeting #60	S6-241400

Changsha, China 15th-19th  April 2024	(revision of S6-241300)

Source:	Ericsson

Title:	Update to KI#3

Spec:	3GPP TR 23.700-82 v0.3.0

Agenda item:	8.3

Document for:	Approval

Contact:	Ashish S Sharma <ashish.sanjay.sharma@ericsson.com>

- Introduction

This paper updates the KI #3 to cover missing aspects of the FL.

2. Reason for Change

The existing KI#3 does not capture the different network topologies of the FL, FL clients incentive exposure, FL optimization. These aspects are necessary because FL is used in different domains with different devices like IoT, edge nodes, consumer devices etc. for different use cases. Each use case has its requirements like training in energy-efficient ways for power constraint devices, reduce communication overhead for bandwidth-constrained devices, etc., in addition to the heterogeneity in devices and FL model training.

Removal of the repeated paragraph.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.3.0

Reference:

[1]	Nguyen, Dinh C. et al. “Federated Learning for Internet of Things: A Comprehensive Survey.” IEEE Communications Surveys & Tutorials 23 (2021): 1622-1658.

[2]	Kairouz, Peter et al. “Advances and Open Problems in Federated Learning.” Found. Trends Mach. Learn. 14 (2019): 1-210.

* * * First Change * * * *

## 5.3	Key issue #3: Support for federated learning

Federated Learning (FL) is a machine learning technique that enables multiple FL clients to train a model by exchanging the parameters instead of exchanging/sharing local data set. FL servers perform management of FL operations by maintaining and updating a global ML model, selecting and managing FL clients, performing aggregation strategies, scheduling training in a federated manner, and communicating with FL clients. FL clients provide various aspects of data for FL operations, such as data collection, data preparation, and using data for training and/or inferencing while communicating updates of ML models to FL servers.

Further, Considering the fact that the FL/ML clients can be deployed on end devices like UEs, there is potentially a very large number of FL/ML clients that can participate in machine learning operations. Out of these potentially large number for FL/ML clients, a set of FL/ML clients can be selected for performing various machine learning tasks. Due to various factors like mobility, changing capabilities, varying resource conditions, etc, the set of UEs (hosting FL/ML clients) that can participate in federated / distributed learning changes over time and may change more frequently. Also, the model information that is exchanged between FL/ML clients and the machine learning servers could also be huge, considering the large machine learning models. So, there will be frequent need for changing the member FL/ML clients that can participate in federated/distributed machine learning operations and also frequently the need to distribute the model information.

The FL process depends on the FL client's participation. No training can be done unless the client participates. Since the FL process is subject to communication and clients can drop off during the process due to various reasons like high communication costs, no benefit for participation, etc. The FL client drop-off during the process has a negative impact on the FL operation like training. [1],[2] describes the requirement for an incentive mechanism to encourage client participation. Therefore, it is necessary to incentivise clients to avoid client drop-offs and encourage client participation for AIML operations like FL training.

This key issue will study:

1.	How to support federated learning at application enablement layers?

2. 	Identify procedures for supporting FL at the application enablement layer, including FL entity discovery, registration, communication, reporting of the FL analytics

3. 	Whether and how to support the data collection and preparation for FL in the application enablement layer?

4. 	Whether and how to support the management of FL groups (e.g., how to create and manage a group of FL members) during FL operations (e.g., training)?

5.	Whether and how to support the application server for the distribution of the model information to the FL members/FL clients/ML clients for model training, considering the dynamically changing potential FL members/FL clients/ML clients for model training?

6.	Whether and how to encourage the participation of AIML clients like FL client/server in AIML operations like FL?

* * * End of Change * * * *