3GPP TSG-SA WG6 Meeting #60	S6-241444

Changsha, China 15th April – 19th April 2024	(revision of S6-241256)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0060 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve Editor's notes about SEALDD query procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For obtaining the stored data from SEALDD server, it is better to use the SEALDD fetch message instead of using SEALDD query message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to solve the Editor’s notes in SEALDD query procedure, to correct the SEALDD query message into SEALDD fetch message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current name of SEALDD query message/API is possible to cause the misunderstanding for its functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.2.3, 9.5.2.5, 9.5.3.4, 9.5.4.1, 9.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.5.2.3	Data storage fetch

Pre-conditions:

1.	VAL client/server has stored data in SEALDD server.

2.	The consumer (e.g. SEALDD client/VAL server/other SEALDD server) requesting the stored data has got the identifier of the stored data (e.g. via application layer signalling or other mechanisms).

Figure 9.5.2.3-1: Access to stored data in SEALDD server

1.	The consumer (e.g. SEALDD client/VAL server/other SEALDD server) sends a Sdd_DataStorage_Fetch request to the SEALDD server with the identifier of the stored data.

NOTE:	The consumer requesting for the stored data can be the data storage creator or other network functions.

2.	Upon receiving the request, the SEALDD server performs an authorization check to verify if the requested data (according to the data access control policy) can be accessed by the consumer. The SEALDD server will also check whether the stored data is expired. If the requested data fetchedby SEALDD client is not be stored by the SEALDD server, the SEALDD server may use the downlink pull mode if possible, to retrieve the data/content from the VAL server over SEALDD-S interface, as defined in clause 9.1.

3.	If the verification was successful, the SEALDD server responds with the requested data.

* * * Next Change * * * *

9.5.2.5	Stored data transfer between VAL servers via SEALDD server

Pre-conditions:

1.	The source VAL server already obtains the serving target SEALDD server of the target VAL server.

2.	The source VAL server may have stored data to the source SEALDD server and obtained the identifier of the stored data by invoking Sdd_DataStorage_Creation API in clause 9.5.2.1.

Figure 9.5.2.5-1: Stored data transfer procedure between VAL servers

1.	The VAL server sends a SEALDD data storage delivery subscription request to its serving SEALDD server with the VAL server information, and the VAL server address to receive the stored data delivery.

2.	The SEALDD sever responds to the requested VAL server with the subscription result, if successful, the response includes the subscription ID, expiration time.

3.	The source VAL server determines to use the stored data transfer service provided by the serving SEALDD server. The source VAL server invokes a SEALDD data storage delivery request, the request includes the identifier of the stored data needed to be delivered, or the stored data (i.e., the source VAL server does not use data storage creation to store data to the source SEALDD server before), the target VAL server information and the target SEALDD server information.

4.	The source SEALDD server sends a SEALDD delivery connection establish request towards the target SEALDD server with the source SEALDD server information, the target VAL server information, and the supporting transport layer protocols for data delivery.

5.	The target SEALDD server responds to the source SEALDD server with the traffic descriptor of target SEALDD server (e.g. address/port, transport layer protocol). The target SEALDD server may allocate the storage for the target VAL server.

NOTE: 	The transport layer protocol between SEALDD server connection can be different depending on the specific application scenarios, compared with the connection between SEALDD client and SEALDD server in clause 9.2.

6.	Upon receiving the traffic descriptor of target SEALDD server, the source SEALDD server sends the requested data corresponding to the identifier of the stored data to the target SEALDD server.

7.	The source SEALDD server responds to the source VAL server with the result of data delivery.

8.	The target SEALDD server sends the received data to the target VAL server via the notification message. Optionally, the target SEALDD server may only send the identifier of the received data to the target VAL server in the notification message, the target VAL server can obtain the stored data originated from the source VAL server, by invoking the Sdd_DataStorage_Fetch API to the target SEALDD server, as specified in clause 9.5.2.2.

* * * Next Change * * * *

9.5.3.4	SEALDD data storage fetch request

Table 9.5.3.4-1 describes the information flow from the requesting consumer (e.g., SEALDD client, VAL server, other SEALDD server) to the SEALDD server for requesting the data storage fetch.

Table 9.5.3.4-1: SEALDD data storage fetch request

| Information element | Status | Description |
| --- | --- | --- |
| Identifier of the stored data | M | Identify the stored data queried by the requesting consumer |

9.5.3.5	SEALDD data storage fetch response

Table 9.5.3.5-1 describes the information flow from the SEALDD server to the requesting consumer (e.g., SEALDD client, VAL server, other SEALDD server) for responding the data storage fetch request.

Table 9.5.3.5-1: SEALDD data storage fetch response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| The fetchedapplication data | O | The fetched application data returned to the requesting consumer |

* * * Next Change * * * *

9.5.4.1	General

Table 9.5.4.1-1 illustrates the APIs exposed by SEALDD server for data storage.

Table 9.5.4.1-1: List of SEALDD server APIs for data storage

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_ DataStorage_Creation | Request | Request/Response | SEALDD client, VAL server |
| Sdd_ DataStorage_Fetch | Request | Request/Response | SEALDD client, VAL server, other SEALDD server |
| Sdd_ DataStorage_Management | Request | Request/Response | SEALDD client, VAL server |
| Sdd_DataStorage_Delivery_Subscription | Request | Request/Response | VAL server |
| Sdd_DataStorage_Delivery_notification | Notify | Subscribe/notify | VAL server |
| Sdd_DataStorage_Delivery | Request | Request/Response | VAL server |

* * * Next Change * * * *

9.5.4.3	Sdd_DataStorage_Fetch Request operation

API operation name: Sdd_DataStorage_Fetch Request

Description: The consumer requests for one time for data storage Fetch.

Inputs: See clause 9.5.3.4.

Outputs: See clause 9.5.3.5.

See clause 9.5.2.3 for details of usage of this operation.