3GPP TSG-SA WG6 Meeting #60	S6-241446

Changsha, China 15th – 19th April 2024	(revision of S6-241279)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0068 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct SEALDD connection status event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IE tables needs to be corrected for SEALDD connection status event, e.g. report id can re-use event id. |  |  |  |  |  |  |  |  |
| , st |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove report id - Clarify the IE applicability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IE redundancy; unclarity for the IE related to SEALDD client connection status. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.6.2.1, 9.2.3.7, 9.2.3.8, 9.2.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.2.6.2.1	SEALDD connection status procedure

Figure 9.2.2.6.2.1-1 illustrates the procedure for SEALDD connection status from the VAL server to the SEALDD server.

Figure 9.2.2.6.2.1-1: SEALDD connection status procedure

1.	The VAL server sends the SEALDD connection status subscribe request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identity, and the SEALDD client connection status check periodicity under "SEALDD client connection status" event as described in Table 9.2.3.7-1.

2.	The SEALDD server subscribes to the NEF UE reachability, Application Detection and Loss of connectivity events using the procedure defined in clause 4.15.3.2.3b TS 23.502. It can also use the NRM Event monitoring procedure defined in clause 14.3.6.2.2 TS 23.434.

3.	The SEALDD server also sends a SEALDD client connection status reporting configuration request to the SEALDD client. The request message consists of reporting interval, flow ID, and method of reporting. The SEALDD client configures the connection status reporting configuration and sends a SEALDD client connection status reporting configuration response. The flow ID identifies the application traffic flow for which the reporting notification is configured. The SEALDD client monitors the application using flow ID. The method of reporting is defined as regular or irregular. The regular reporting method uses the reporting interval to send the notification. The irregular reporting method sends the notification if the current connection status state is different from the previous connection status state or if the application state changes (like crash, close, stop).

4.	The SEALDD client configures the reporting configuration and provides the response to the SEALDD server.

5.	The SEALDD server sends the connection status subscription response to the VAL server.

6a-6b. The SEALDD server receives the notification for the UE connection status from the subscribed NEF, NRM. It can also receive a notification from the SEALDD client regarding the connection status. If the UE is in power saving mode, then the SEALDD client sends the connection status reporting notification with the status as sleeping to the SEALDD server and suspends the connection status periodic reporting.

7.	Based on the NRM, NEF event subscription response and SEALDD client connection status reporting notification message with "SEALDD client connection status" event as described in Table 9.2.3.9-1, the SEALDD server processes the responses and sends the SEALDD client connection status of unreachable or sleeping status notification to the VAL server.

NOTE:	If the VAL server is not aware of the connection status like VAL UE reachability and may continue sending the application traffic, results in packet losses and degradation of the QoS of the VAL UEs.

#### 9.2.3.7	SEALDD connection status subscription request

Table 9.2.3.7-1 describes the information flow from the VAL server to SEALDD server to subscribe to SEALDD connection status information.

Table 9.2.3.7-1: SEALDD connection status subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| Event ID list | M | Identifies a list of events such as establishment, release, congestion report, SEALDD client connection status. |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of VAL UE or VAL user. |
| SEALDD-S Data transmission connection information | M | Address/port of the VAL server to send/receive the application packets to/from the SEALDD server. |
| Immediate reporting flag | O | Indicates the immediate reporting of connection status notification |
| SEALDD client connection status check periodicity | O | Indicates the frequency to perform SEALDD client connection status check |

#### 9.2.3.8	SEALDD connection status subscription response

Table 9.2.3.8-1 describes the information flow from the SEALDD server to VAL server for responding SEALDD connection status subscription request.

Table 9.2.3.8-1: SEALDD connection status subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Subscription ID | O (NOTE) | Subscription identifier corresponding to the subscription. |
| Expiration time | O (NOTE) | Indicates the expiration time of the subscription. |
| NOTE: These IEs shall be present when the result is success. |  |  |

#### 9.2.3.9	SEALDD connection status notification

Table 9.2.3.9-1 describes the information flow from the SEALDD server to the VAL server to notify SEALDD connection status.

Table 9.2.3.9-1: SEALDD connection status notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | M | Identifies event such as establishment, release, congestion report, SEALDD client connection status. |
| Identity | M | Identifier of VAL UE or VAL user. |
| VAL service ID | M | Identity of the VAL service. |
| SEALDD connection establishment data | O (See NOTE 1) | Data related to SEALDD connection establishment. |
| > SEALDD-S Data transmission connection information | M | Address/port of the SEALDD server to send/receive the application packets to/from the VAL server. |
| > SEALDD communication lifetime | O | Identifies the DD communication lifetime. |
| Congestion level | O (See NOTE 2) | The congestion level of the VAL service |
| SEALDD client connection status | O (See NOTE 3) | Indicates the connection status of VAL UE. E.g. reachable, unreachable, sleeping |
| NOTE 1: This IE is used for the establishment event. NOTE 2: This IE is used for the congestion report event. NOTE 3: This IE is used for the SEALDD client connection status event. |  |  |

* * * End of Changes * * * *