3GPP TSG-SA WG6 Meeting #60	S6-241491

Changsha, China 15th – 19th April 2024	(revision of S6-241119)

Source:	China Mobile, ZTE, Lenovo, Samsung

Title:	New KI on support the tethered UE

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution proposes a new KI on support the tethered UE.

2. Reason for Change

Different from other types of AR UE, the end-to-end path for the tethered UE includes one more wireless/wireline tethering link between AR Glasses and the tethering 5G Phone. In order to fulfil the end-to-end QoS requirements for the AR glass session, the consumer need to acquire the tethering link status via measurement tests or empirical values, and takes it into account when determining the QoS for the 5G system link. With the tethering link status, the application enablement layer may communicate with AF for dynamic QoS policy adjustment accordingly.

The 5G WireLess Tethered AR UE is introduced in TR26.998 [x], and is further studied in TR 26.806[y]. they provides the Segment-by-segment delay measurement solution based on the ICMP, and End-to-end delay measurement based on RTP. The End-to-end delay measurement relies on tethered device to do the RTP based measurement. While it successfully identifies non-5G delays, distinguishing delays from the tethered link and those after N6 remains challenging. The Segment-by-segment delay measurement utilizes the UPF and tethered device for ICMP based measurement, however how UPF retrieves the RTT between the UPF and the application server and further exposes the latency results to the AF are not supported in SA2 in current release. And all the measurement results are sent to the AF, indicating that SA4 does not play a role in optimizing latency based on the measurements.

The above study provides good foundation for this KI. This KI can further study how to identify tethered links and optimize transmission.

It is proposed to study the open issues:

-	Whether and how the application enabling layer to identify traffic flows from the tethered devices behind the UE.

-	Whether and how the application enabling layer to support the acquisition of tethering link status via measurement tests or analysis and provide a transporting optimization, considering different type of the tethered UE (stand along tethered device, or wire/wireless tethered device).

-	Whether and how the PINAPP could be utilized to support the tethered UE.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL);Data Delivery enabler for vertical applications;  Stage 2".

…

[x]	3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[y]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

* * * Next Change * * * *

4.x	Key issue #x: Support the tethered UE

Different from other types of AR UE, the end-to-end path for the tethered UE includes one more wireless/wireline tethering link between AR Glasses and the tethering 5G Phone. In order to fulfil the end-to-end QoS requirements for the AR glass session, the consumer needs to acquire the tethering link status via measurement and analysis, and takes it into account when determining the QoS for the 5G system link. With the tethering link status, the application enablement layer may communicate with AF for dynamic QoS policy adjustment accordingly.

The 5G WireLess Tethered AR UE is introduced in TR 26.998 [x], and is further studied in TR 26.806[y]. they provides the Segment-by-segment delay measurement solution based on the ICMP, and End-to-end delay measurement based on RTP. The End-to-end delay measurement relies on tethered device to do the RTP based measurement. While it successfully identifies non-5G delays, distinguishing delays from the tethered link and those after N6 remains challenging. The Segment-by-segment delay measurement utilizes the UPF and tethered device for ICMP based measurement, however how UPF retrieves the RTT between the UPF and the application server and further exposes the latency results to the AF are not supported in SA2 in current release. And all the measurement results are sent to the AF, indicating that SA4 does not play a role in optimizing latency based on the measurements.

The above study provides good foundation for this KI. This KI can further study how to identify tethered links and optimize transmission.

It is proposed to study the open issues:

-	Whether and how the application enabling layer to identify traffic flows from the tethered devices behind the UE.

-	Whether and how the application enabling layer to support the acquisition of tethering link status via measurement and analytic and provide a transporting optimization, considering different type of the tethered UE (stand along tethered device, or wire/wireless tethered device).

-	Whether and how the PINAPP could be utilized to support the tethered UE.

* * * Next Change * * * *

<Proposed change in revision marks>