3GPP TSG-SA WG6 Meeting #60	S6-241553

Changsha, China 15th – 19th April 2024	(revision of S6-241290)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0630 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AC and EEC interaction for ACR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The AC and EEC interaction over EDGE-5 is not specified for ACR triggering by the EEC via AC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following are the changes proposed: 1. EEC Notification to AC: EEC triggers AC to perform DNS query 2. ACR trigger request: to include CAS information when AC performs DNS query 3. New procedure DNS trigger status reporting between AC and EEC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AC and EEC interaction specification would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.3.10, 8.14.2.x (new), 8.14.3.x1 (new), 8.14.3.x2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

#### 8.14.3.10	ACR trigger request

Table 8.14.3.10-1 describes information elements of the ACR trigger request sent by the AC to the EEC.

Table 8.14.3.10-1: ACR trigger request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile for EEC to determine the target EAS. |
| Security credentials | M | Security credentials of the AC |
| Requested ACR action | M | One of ACR detection or ACR initiation request actions |
| S-EAS information | M | ID of the EAS to which the ACR request relates. |
| T-EAS information (NOTE 1) (NOTE 2) | O | AC provided target EAS information or CAS information received from DNS query result. |
| NOTE 1: This IE may be included if the request is to initiate the ACR. NOTE 2: This IE may be included if the request is to initiate the ACR and CAS information is requested by AC. |  |  |

* * * Next Change * * * *

#### 8.14.2.x	Get CAS information

The procedure is for EEC to get CAS information as a result of DNS query from AC. The get CAS information request is triggered by EEC, when EEC notifies AC about unavailablity of T-EES or T-EAS.

Pre-conditions:

1.	EEC decides to get CAS information due to unavailability of suitable T-EES and/or unavailability of suitable T-EAS as specified in clause 8.14.2.5.3.

Figure 8.14.2.x-1: Get CAS information request procedure

1.	EEC sends get CAS information request to AC, due to unavailability of suitable T-EES and/or unavailability of suitable T-EAS. The request includes ACID corresponding to the S-EAS as described in clause 8.14.3.x1.

2.	The AC identifies the CAS information correcponding to ACID received in step 1, which may trigger AC to perform DNS query (due to unavailability of suitable T-EES and/or unavailability of suitable T-EAS).

2.	The AC provides CAS information, if available or failure response back to EEC as described in clause 8.14.3.x2.

* * * Next Change * * * *

#### 8.14.3.x1	Get CAS information request

Table 8.14.3.x1-1 describes information elements of the Get CAS information request sent by the EEC to the AC.

Table 8.14.3.x1-1: Get CAS information request

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | ID of the AC corresponding to the S-EAS, for which EEC is expecting to get CAS information. |
| S-EAS information | M | ID of the EAS to which the T-EES or T-EAS is not available |

* * * Next Change * * * *

#### 8.14.3.x2	Get CAS information response

Table 8.14.3.x2-1 describes information elements of the Get CAS information response sent by the AC to the EEC.

Table 8.14.3.x2-1: Get CAS information response

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | ID of the AC corresponding to the S-EAS, for which EEC is expecting to get CAS information. |
| S-EAS information | M | ID of the EAS received in the get CAS information request |
| Successful response (NOTE) | O | Indicates that the CAS information is available. |
| > CAS information | O | AC provides target CAS information received from DNS query result. |
| Failure response (NOTE) | O | Indicates that getting the CAS information failed |
| > Cause | O | Indicates the cause of request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * End of Changes * * *