3GPP TSG-SA WG6 Meeting #60	S6-241575

Changsha, China, 15th– 19th April 2024

Source:	Ericsson

Title:	Solution on New ADAE Analytics Based on Ranging/Sidelink Positioning Information Exposure

Spec:	3GPP TS 23.700-72 V0.4.0

Agenda item:	8.1

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

As described in Key Issue #5, the study aspects on support for ranging/sidelink positioning services include:

-	Whether and how the enabler layer can support exposing ranging information to 3rd party consumers?

The ranging/sidelink positioning information can be used in many applications with the support of enabler layer, e.g., be used by ADAE Server to generate analytics for various applications. For example, ADAE Server can use the ranging/sidelink positioning information of UEs from 5GC, location information of UEs from SEAL LM server and/or LM client (moving devices like UAS devices, V2X devices, robots, and/or people with UE), and the location information about environment (static devices like infrastructures) from the 3rd party LM server, to generate predictions on collision detection. The analytics output includes list of UEs with the alleged collision location and times for the given time horizon. The predictions on potential collisions can be used:

- to minimum effect and collisions between the moving robots in factory enviornment.

- to avoid collisions between vehicles and pedestrains for intelligent driving.

- for planning best moving route of robots in smart warehouse enviornment.

- by UAE Server and/or UAS application specific server for DAA, planning flight route of UAVs, and determining propoer control entity of a UAV.

This paper proposes a new ADAE analytics on collision detection among UEs (static devices, moving devices) by using the ranging/sidelink positioning information from 5GC, location information from ADAE client, LM server, LM client, and 3rd party LM server.

Different from NWDAF, ADAE Server is able to collect data directly from the data producers (e.g. ADAE client, LM server, LM client, 3rd party LM server), and generate analytics with the application information as inputs, which make the generated analytics easier be used in various applications. In detail, different from the existing NWDAF Analytics ID e.g., “Relative Proximity”, the new ADAE analytics:

- Takes inputs from wider range of data producer, e.g., from 5GC for ranging/sidelink positioning information of UEs, from ADAE client, SEAL LM server, LM client, and 3rd party LM server for location information of UEs. Based on information available in the application layer (e.g., VAL Service ID), the ADAE server can determine the shape and size of the UEs that will increase the collision detection accuracy.

- Considers wider range of objects for collision detection, e.g., moving devices (e.g., UAS devices, V2X devices, robots, and/or people with UE), static devices (e.g., infrastructures), and UEs in 5G network. The NWDAF analytics only focus on the UEs which are registered in 5G network.

For specific applications e.g. DAA, the existing solution in TR 23.700-59 clause 6.5.3.1 requires that information of the UE is known when request service from 5GC, e.g., identifier of the UAV(s) (e.g., GPSI(s), CAA-Level UAV ID(s)) as inputs to request 5GC for GMLC service on Ranging/Sidelink Positioning location and/or for NWDAF service on Relative Proximity predictions on collision via NEF. The new ADAE analytics does not have such limitations.

2. Reason for Change

To support exposure of ranging/sidelink positioning information, new ADAE analytics on potential collisions among UEs for DAA can be studied.

3. Conclusions

This pCR proposes new ADAE analytics on potential collisions among UEs by using the ranging/sidelink positioning information.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72 V0.4.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	3GPP TS 29.572: "Location Management Services; Stage 3".

[9]	3GPP TS 23.434: Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows.

[10]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs".

[12]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[13]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[14]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains ".

[15]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service ".

[16]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services ".

[Y]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # x |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |
| Sol #2 | X |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |
| Sol #5 |  |  |  | X |  |  |
| Sol #6 |  |  | X |  |  |  |
| Sol #X |  |  |  |  | X |  |

* * * Next Change * * * *

## Solution #X: New ADAE Analytics Based on Ranging/Sidelink Positioning Information Exposure

### 6.X.1	General

The following clauses specify procedures, information flows and APIs for KI#5 to support new ADAE analytics on collision detection among UEs by using ranging/sidelink positioning information exposured from 5GC, the location information of devices from SEAL LM server, LM client, and/or 3rd party LM server.

Different from existing NWDAF, ADAE Server is able to collect data directly from the wider range of data producers (e.g., from 5GC for ranging/sidelink positioning information of UEs, from ADAE client, SEAL LM server, LM client, and 3rd party LM server for location information of UEs.), and generate analytics with the application information as inputs, which make the generated analytics easier be used in various applications. Based on information available in the application layer (e.g., VAL Service ID), the ADAE server can determine the shape and size of the UEs that will increase the collision detection accuracy.

The new ADAE analytics considers wider range of objects for collision detection, e.g., moving devices (e.g., UAS devices, V2X devices, robots, and/or people with UE), static devices (e.g., infrastructures), and UEs in 5G network. Furthermore, supports 5G non-registered UEs (i.e., not limited to PC5 ProSe), or even the NSWO UEs as defined in clasue 5.42 of 3GPP TS 23.501 [6].

This solution is applicable for ranging and non-ranging services.

### 6.X.2	Procedure

Pre-conditions:

-	Information about environment, e.g. static devices, are available from the 3rd party LM server.

-	The location information about the UEs is available at SEAL LM server and/or LM client.

-	The Ranging/Sidelink Positioning information exposure of the UEs is allowed at 5GC.

#### 6.X.2.1	Subscribe-notify model

Figure 6.X.2.1-1: ADAES support for collision detection analytics

- The analytics consumer (e.g. VAL Server, LM server, UAE server, UAS application specific server) sends analytics subscription request for collision detection analytics to ADAE server. The Analytics ID in the request message is set to "Collision detection analytics". For analytics subscription request, the request contains message as defined in table 6.X.3.2-1.

- Upon receiving the event subscription request from the consumer, the ADAE server checks for the relevant authorization for the event subscription. If the authorization is successful, the ADAE server stores the request information. The ADAE server sends a service API event subscription response indicating successful subscription.

- The ADAE server sends a subscription request to the Data Producer (e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data) with the respective Data Collection Event ID and the requirement for collection of ranging and/or sidelink positioning related data or analytics of UEs, and location information of UEs. Data collection at the UE(s) reuses the SA4 mechanism based on EVEX study (3GPP TS 26.531 [Y]).

- The Data Producer sends subscription response as a positive or negative acknowledgement to the ADAE server.

- The ADAE server based on subscription receive ranging and/or sidelink positioning related data/analytics of Ues, and location information of UEs.

NOTE:	The procedures for UE related data collection need to take user consent into account.

- The ADAE server performs analytics relevant operations to generate the analytics for the collision detecton between any target VAL UEs, collision detecton between any UEs and target VAL UEs, or collision detection between any UE within the Area of Interest and based on the request provided in step 1 and data received from the Data Producer (e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data).

- The ADAE server sends notifications to the consumer with the required collision detection analytics.

#### 6.X.2.2	Request-response model

Pre-conditions:

-	ADAE Server already have the analytics data derived from steps 3-6 in the procedure introduced in clause 8.X.2.1.

Figure 6.X.2.2-1: ADAES support for collision detection analytics

1.	The analytics consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) sends a request message to the ADAE server to receive analytics data for collision detection analytics with Analytics ID in the request message setting to "Collision detection analytics". The request contains message as defined in table 6.X.3.8-1.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer.

3.	If the analytics consumer is authorized, the ADAE server sends a response message including the analytics data (statistical and/or predictive) of the collision detection analytics.

### 6.X.3	Information flows

#### 6.X.3.1	General

The following information flows are specified for collision detection analytics based on clause 6.X.2.

#### 6.X.3.2	Collision detection analytics subscription request

Table 6.X.3.2-1 describes the information flow from the consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) as a request or update request for the collision detection analytics.

Table 6.X.3.2-1: Collision detection analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer. |
| Analytics ID | O (NOTE) | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions, |
| Analytics filter information | M | Filter information for the analytics event |
| >Target UE(s) | O | The UE(s) for which the analytics subscription applies. |
| > Any UE(s) | O | The list of any UE(s) collision with that shall be analysed. |
| > Collision between any target UE indicator | O | Indicates the request for the collision detection analytics between any UE provided in the Target VAL UE(s) IE. |
| > Collision between any UE(s) and target UE(s) | O | Indicates the request for the collision detection analytics between any any UE and any target UE. |
| > Collision detection between any UE within the Area of Interest | O | Indicates the request for the collision detection analytics between any UEs within the Area of Interest. |
| >Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies. |
| >Area of Interest | O | The geographical or service area for which the subscription request applies |
| Attributes for collision detection | O | The parameters for collision detection analytics apply. |
| >Distance | O | The minimum distance allowed for UEs. The collision is detected if the distance between the UEs is less than the provided value. |
| >Velocity | O | The maximum velocity of the UEs. |
| >Direction | O | The expected direction of the UEs moving. |
| >Movement track | O | The parameters for Movement track (e.g., the list of geographical points) of UEs. |
| >Granularity | O | The granularity of the movement track. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: This information element shall not be updated. |  |  |

#### 6.X.3.3	Collision detection analytics subscription response

Table 6.X.3.3-1 describes the information elements for the collision detection analytics subscription response from the ADAE server to the consumer.

Table 6.X.3.3-1: Collision detection analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |

#### 6.X.3.4	Collision detection analytics notification

Table 6.X.3.4-1 describes the information flow from the ADAES to the consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) as a response for the collision detection analytics.

Table 6.X.3.4-1: Collision detection analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Outputs | O | The reported analytics for the collision detection analytics. |
| >VAL UE IDs | M | The VAL UE identifiers for which the analytics outputs apply. |
| >>Time of potential collisions | M | Time of the potential collisions between the UEs. |
| >>Location of potential collisions | M | Location of the potential collisions between the UEs. |
| >>Direction of moving | O | Direction of the UEs moving. |
| >>Velocity of moving | O | Velocity of the UEs moving. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

#### 6.X.3.5	Ranging/SL positioning data and location information collection subscription request

Table 6.X.3.5-1 describes information elements for the Ranging/SL positioning data and location information collection subscription request from the ADAE server to the Data Producer, e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data.

Table 6.X.3.5-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. This ID can be for example "Collision Detection analytics". |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| >Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| >VAL UE IDs | O | The VAL UE(s) identifiers for which the data/analytics apply. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |

#### 6.X.3.6	Ranging/SL positioning data and location information collection subscription response

Table 6.X.3.6-1 describes information elements for the Data collection subscription response from the Data Producer, e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data.

Table 6.X.3.6-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the Ranging/SL positioning data and location information collection subscription request (positive or negative acknowledgement). |

#### 6.X.3.7	Data Notification

Table 6.X.3.7-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 6.X.3.7-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. Such data can be ranging/sidelink positioning information of UEs, location information of UEs (moving devices like UAS devices, V2X devices, robots, and/or people with UE), and the location information about environment (static devices like infrastructures) for a given time and area of interest, Relative Proximity analytics. |

#### 6.X.3.8	Get analytics data request

Table 6.X.3.8-1 describes information elements for the collision detection analytics request from the analytics consumer to the ADAE server.

Table 6.X.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions, |
| Analytics filter information | M | Filter information for the analytics event |
| >Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies |
| >Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies. |
| >Area of Interest | O | The geographical or service area for which the subscription request applies |
| Attributes for collision detection | O | The parameters for collision detection analytics apply. |
| >Distance | O | The minimum distance allowed for two UEs. |
| >Velocity | O | The maximum velocity of the UEs. |
| >Direction | O | The expected direction of the UEs moving. |
| >Granularity | O | The granularity of the movement track. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |

#### 6.X.3.9	Get analytics data response

Table 6.X.3.9-1 describes information elements for the Get collision detection analytics response from the ADAE server to the consumer.

Table 6.X.3.9-1: Get analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Outputs | O | The reported analytics for the collision detection analytics. |
| >VAL UE IDs | M | The VAL UE identifiers for which the analytics outputs apply. |
| >>Time of potential collisions | M | Time of the potential collisions between the UEs. |
| >>Location of potential collisions | M | Location of the potential collisions between the UEs. |
| >>Direction of moving | O | Direction of the UEs moving. |
| >>Velocity of moving | O | Velocity of the UEs moving. |
| >>Timestamp | O | Time stamp of the collected analytics data. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

# *** End of Changes ***