3GPP TSG-SA WG6 Meeting #60	S6-241515

Changsha, China 15th – 19th April 2024	(revision of S6-241247)

Source:	KPN N.V.

Title:	Solution #14 update: member selection and traffic QoS adjustment

Spec:	3GPP TR 23.700-82 v0.3.0

Agenda item:	8.3

Document for:	Approval

Contact:	Yonatan Shiferaw, yonatan.shiferaw@tno.nl

1. Introduction

This contribution updates Solution #14 to clarify how the list of AI/ML traffic QoS adjustment policies is used together with list of member selection and re-selection polices.

2. Reason for Changes

In clause 8.14.3 list of AI/ML traffic QoS adjustment policies and list of member selection and re-selection polices are defined. The AI/ML member selection and re-selection policies are used to select AI/ML members based on conditions defined in table 8.14.3.1-2. The QoS adjustment is then applies to the selected members and the action is revereted for the members that are de-selected. In this way, it is guranteed that only members that are going to participate in the AI/ML operation are adjusted. The contribution adds this clarification in clause 8.14.2.1. This contribution additionaly clarifies the List of AI/ML member participation configurations IE in Table 8.14.3.1-2 by adding “conditions” indicate this among the other conditions listed in the table which is used for member selection and re-selection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.3.0.

* * * First Change * * * *

## 8.14	Solution #14: AI/ML policies provisioning and management

### 8.14.1	General

The following clauses specify procedures, information flows and APIs for Key issue#7 to provision and manage the AI/ML policies for the AI/ML Enablement services.

### 8.14.2	Procedures

#### 8.14.2.1	AI/ML policies provisioning and management

Figure 8.14.2-1: AI/ML policies provisioning and management procedure

1.	The VAL server sends an AI/ML service request with policies provisioning and management information as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enablement Server performs an authorization check of the VAL server.

3.	If the VAL server is authorised, the AI/ML Enablement Server stores the AI/ML policies and applies the provisioned policies to ongoing and further the AI/ML operations (e.g., ML model training). The AI/ML traffic QoS adjustment policies are applied for the selected AI/ML members as per the member selection and re-selection policy after obtaining the identifiers of the AI/ML members that are selected as a result of the enforcement of the policy. The AI/ML Enablement server can use the SEALDD or the NEF services ( AFSessionWithQoS) to configure the QoS. When the AI/ML members are de-selected the QoS adjustment is revesed as they no longer participate in the AI/ML operation.

4.	AI/ML Enablement Server provides the response to the VAL server with a status via AI/ML member selection policies provisioning and management response defined in clause 8.14.3.2.

NOTE:	The service operation defined in clause 8.A.2.1 can be utilized for AI/ML member selection policies management (i.e., update/delete) functionality.

Editor's Note:	The related service operation request (e.g., member selection) is to be updated in the evaluation stage.

### 8.14.3	Information flows

#### 8.14.3.1	AI/ML service request with policies provisioning and management information

Table 8.14.3.1-1 describes the information flow from the VAL server to the AI/ML Enablement Server as a request that contains the AI/ML policies provisioning and management information.

Table 8.14.3.1-1: AI/ML policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (see NOTE 1) | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of member selection and re-selection polices | O (see NOTE) | Identifies the list of member selection and re-selection policies, e.g., the AI/ML member shall be re-selected when the PLR is above the given value or leaves the certain area. See details in table 8.14.3.1-2. |
| List of AI/ML traffic QoS adjustment policies | O (see NOTE) | Identifies the list of QoS adjustment policies, e.g., the AI/ML traffic QoS shall be adjusted when the conditions are met. See details in table 8.14.3.1-3. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Editor's note: Whether additional AI/ML policies are needed is FFS.

Table 8.14.3.1-2: Member selection and re-selection policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., select member, not select member, or re-select member. |
| List of AI/ML traffic QoS conditions | O (see NOTE) | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |
| List of AI/ML member mobility conditions | O (see NOTE) | Provides the list of mobility conditions for the AI/ML member associated with the Action, e.g., low mobility, high mobility, stationary. |
| List of AI/ML member participation configuration conditions | O (see NOTE) | Provides the list of the AI/ML member participation configuration conditions associated with the Action. The AI/ML member participation configurations provided by the AI/ML members as in table 8.10.3.1-1 are matched with the provided configuration conditions for member selection and re-selection. |
| List of AI/ML compute capability conditions | O (see NOTE) | Provides the list of the AI/ML compute capability conditions associated with the Action. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Editor's note: The AI/ML compute capability conditions are FFS.

Table 8.14.3.1-3: AI/ML traffic QoS adjustment policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., adjust AIML traffic QoS. |
| List of the required AI/ML traffic QoS values | M | Represents the required QoS values that shall be fulfilled when the conditions are met. |
| List of AI/ML traffic QoS conditions | M | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |

#### 8.14.3.2	AI/ML service response with policies provisioning and management information

Table 8.14.3.2-1 describes the information flow from the AI/ML Enablement Server to the VAL server as a response that contains the AI/ML policies provisioning and management result.

Table 8.14.3.2-1: AI/ML member selection policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * End of Change * * * *