3GPP TSG-SA WG6 Meeting #60	S6-241587

Changsha, China, 15th – 19th April 2024	(revision of S6-241418)

Source:	Ericsson

Title:	Pseudo-CR on considerations for MC services over non-terrestrial network

Spec:	3GPP TR 23.700-01 v 0.2.0

Agenda item:	8.6

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR provides some considerations related to utilizing non-terrestrial networks for MC services.

2. Reason for Change

This pCR provides some considerations related to the type of satellite system which is most beneficial for MC services.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v 0.2.0

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 22.822: "Study on using satellite access in 5G; Stage 1".

[4]	3GPP TR 22.865: "Study on satellite access - Phase 3".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[9]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications"

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[38811]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks"

[38821]	3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks (NTN)"

[23501]	3GPP TS23.501: "System architecture for the 5G system"

* * * First Change * * * *

# Annex X:	Considerations for MC services over non-terrestrial networks

## X.1	Description

The following clauses provides considerations on the satellite type in order for MC services to benefit from non-terrestrial networks. Clause X.2 discusses the different type of satellite systems and the type which are suitable, and most beneficial for MC services.

## X.2	Type of satellite systems

Satellite systems or deployments can be divided into two main categories with respect to its altitude: Geostationary Equatorial Orbit (GEO), and Non-Geostationary Equatorial Orbit. The GEO is orbiting at highest altitudes comparing to the non-GEO, e.g., orbiting at 35786 km altitude in the equatorial plan as described in 3GPP TR 38.811 [33811], where its position can be considered fixed with respect to the earth. This provides the benefit of having a large area being covered via one satellite system.

Furthermore, the non-GEO satellite comprises of two constellations: Low Earth Orbit (LEO), and Medium Earth Orbit (MEO). The LEO can be at altitude between 600 – 1500 km, and MEO at 10000 km, as described in 3GPP TR 38.811 [33811].

Satellite systems of relatively low altitude, e.g., LEO, can offer relatively lower latency, which can be the most beneficial choice latency sensitive services such as MC services. The different characteristics related to the delay, e.g., propagation delay, differential delay, etc., are mentioned in 3GPP TR 38.811 [33118].

Unlike GEO, non-GEO satellite beams are moving comparing to the ground. Therefore, it is necessary to have a constellation of several non-GEO satellites associated with the MC service under consideration for handover mechanisms to ensure proper service continuity over the non-terrestrial networks. This results in having the same cell being covered by different beams, i.e., different non-GEO satellites over time.

* * * End of Changes * * * *