3GPP TSG-SA WG6 Meeting #60	S6-241613

Changsha, China, 15th – 19th April, 2024

Source:	Ericsson

Title:	Solution on re-selection policies for background AIML data transfer

Spec:	3GPP TR 23.700-82 V0.3.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

The re-selection procedure may require the data relocation from the dis-selected member to the newly selected member. For example:

Given: the VAL server has specific requirements for the data transfer from dis-selected member to the selected member, e.g., initiate background AI/ML data transfer  or configure a temporary AI/ML data transfer with QoS during AI/ML member re-selection procedure.

When: the re-selection conditions are met, the AI/ML Enablement Server shall configure the data transfer session with the requested characteristics and initiate the data transfer.

Then: the AI/ML process can continue on the new AI/ML client.

2. Reason for Change

The VAL server may have specific requirements for the AIML data transfer (e.g., the current state of the trained ML model) during the re-selection procedure.

The following use-case examples are supported by the policy:

- The VAL server performs the time-sensitive learning that should be completed as soon as possible. The VAL server can request the AI/ML Enablement server to configure a temporary AI/ML data transfer session with QoS during AI/ML member re-selection procedure, in order to transfer the current trained data within the minimal time.

- The VAL server performs the non-time-sensitive learning and would like to reduce the cost of the AI/ML operation. The VAL server can request the AI/ML Enablement server to use background data transfer with lowest costs as BDT policy guidance.

3. Conclusions

This pCR proposes a new AI/ML policy for data transfer in order to address the possible requirements from the VAL server.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.3.0.

* * * First Change * * * *

### 8.14.1	General

The following clauses specify procedures, information flows and APIs for Key issue#7 to provision and manage the AI/ML policies for the AI/ML Enablement services.

The AI/ML data transfer policies allow the VAL server to provide requirements for the AI/ML data (e.g., the current status of the trained ML model) from the re-selected to the selected client. The VAL server may have a network QoS requirements for AI/ML data transfer to use Background Data Transfer to save cost for the AI/ML data transfer.

The AI/ML data transfer policies can be used for both time-sensitive and non-time-sensitive AI/ML data transfer. For example:

- VAL server performs the time-sensitive learning that should be completed as soon as possible. The VAL server can request the AI/ML Enablement server to configure a temporary AI/ML data transfer session with QoS during AI/ML member re-selection procedure, in order to transfer the current trained data within the minimal time.

- The VAL server performs the non-time-sensitive learning and would like to reduce the cost of the AI/ML operation. The VAL server can request the AI/ML Enablement server to use background data transfer with lowest costs as BDT policy guidance.

* * * Next Change * * * *

#### 8.14.3.1	AI/ML service request with policies provisioning and management information

Table 8.14.3.1-1 describes the information flow from the VAL server to the AI/ML Enablement Server as a request that contains the AI/ML policies provisioning and management information.

Table 8.14.3.1-1: AI/ML policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (see NOTE 1) | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of member selection and re-selection polices | O (see NOTE) | Identifies the list of member selection and re-selection policies, e.g., the AI/ML member shall be re-selected when the PLR is above the given value or leaves the certain area. See details in table 8.14.3.1-2. |
| List of AI/ML traffic QoS adjustment policies | O (see NOTE) | Identifies the list of QoS adjustment policies, e.g., the AI/ML traffic QoS shall be adjusted when the conditions are met. See details in table 8.14.3.1-3. |
| AI/ML data transfer policy for AI/ML member re-selection | O (see NOTE) | Identifies the AI/ML data transfer policy that shall be applied for the AI/ML data transfer from dis-selected to the selected AI/ML member during the re-selection procedure. See details in table 8.14.3.1-4. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Editor's note: Whether additional AI/ML policies are needed is FFS.

Table 8.14.3.1-2: Member selection and re-selection policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., select member, not select member, or re-select member. |
| List of AI/ML traffic QoS conditions | O (see NOTE) | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |
| List of AI/ML member mobility conditions | O (see NOTE) | Provides the list of mobility conditions for the AI/ML member associated with the Action, e.g., low mobility, high mobility, stationary. |
| List of AI/ML member participation configurations | O (see NOTE) | Provides the list of the AI/ML member participation configurations associated with the Action as defined in table 8.10.3.1-1. |
| List of AI/ML compute capability conditions | O (see NOTE) | Provides the list of the AI/ML compute capability conditions associated with the Action. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Editor's note: The AI/ML compute capability conditions are FFS.

Table 8.14.3.1-3: AI/ML traffic QoS adjustment policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., adjust AIML traffic QoS. |
| List of the required AI/ML traffic QoS values | M | Represents the required QoS values that shall be fulfilled when the conditions are met. |
| List of AI/ML traffic QoS conditions | M | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |

Table 8.14.3.1-4: AI/ML data transfer policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., initiate background AI/ML data transfer or configure a temporary AI/ML data transfer with QoS during AI/ML member re-selection procedure. |
| List of the required AI/ML traffic QoS values | O (see NOTE) | Represents the required network QoS values that shall be enforced for the AI/ML data transfer with QoS. This IE is applicable for the "configure a temporary AI/ML data transfer with QoS". |
| Background data transmission details and policy guidance | O (see NOTE) | Provides the background data transmission details and policy guidance as described in clause 14.3.2.58 of 3GPP TS 23.434 [7]. This IE is applicable for the "initiate background AI/ML data transfer" action. |
| NOTE: Only one of these information elements shall be provided. |  |  |

NOTE:	The AI/ML data transfer policies can be used for e.g. AI/ML model distribution and transfer. In detail, for example, the model transfer process mentioned in step 2 of Solution #5 can consider using AI/ML data transfer policies. The AI/ML data transfer policies can also be used by the solutions for KI#5 on in-time transfer of AI/ML models.

* * * Next Change * * * *

### 8.5.3	AI/ML model information discovery procedure

Figure 8.5.3-1: AI/ML model information discovery procedure

1.	The model repository consumer (e.g., VAL server) sends an AI/ML model information discovery request to the model repository. The request contains filter information of the AI/ML model such as the analytics ID(s) associated with the model, requirements of the model (e.g., required computing power), information of the training data, etc. If ADAE is the AI/ML model consumer, then step 1 is skipped.

2.	Upon receiving the discovery request, the model repository processes the request and sends the response message to the model repository consumer. The response includes information of the discovered AI/ML model. Optionally, the model repository can initiate a model transfer process to the consumer.

NOTE 1:	It is to be determined in the evaluation phase which entities are can have direct interaction with the model repository, e.g., AI/ML enablement server/client and/ or MTME-enhanced ADAE server/client, etc. This procedure is assumed to also enable other AI/ML service consumers to access this functionality indirectly, via these enablers with direct access.

NOTE 2:	The model transfer process mentioned in step 2 can use the AI/ML data transfer policies.

# *** End of Changes ***