3GPP TSG-SA WG6 Meeting #60	S6-241625

Changsha, China April 15th – 19th April 2024	(revision of S6-241484,S6-241287)

Source:	Ericsson

Title:	Enhancement to the update solution

Spec:	3GPP TR 23.700-21 v0.2.0

Agenda item:	8.4

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This pCR proposes a solution to KI#1

2. Reason for Change

The existing solution does not support the update of the linked spatial anchor. The VAL server can modify the existing linked spatial anchor, delink the existing spatial anchor, and update the prioritization of the spatial anchor.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v0.2.0.

* * * First Change * * * *

#### 7.2.3.2	Updating spatial anchor

Figure 7.2.3.2-1 depicts the procedure for updating spatial anchor association for metaverse applications. The update operation includes modifying existing spatial anchor or deleting the spatial anchor. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.2-1: Update spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update the existing spatial anchor association. The request is used to modify or delete the spatial anchor. The request message includes the identity of the spatial anchor and the application service identifier. To modify the spatial anchor, the request may include service information of the product to associate it with the spatial anchor, spatial anchor service area and access control rules.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server checks whether the spatial anchor as identified by the identity of the spatial anchor exists or not. If the spatial anchor does not exists then the SEAL LM server sends failure response. Otherwise, the SEAL LM server updates the spatial anchor details (for update request) or deletes the spatial anchor details (for delete request). The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including reason for failure for failure case.

* * * First Change * * * *