3GPP TSG-SA WG6 Meeting #61	S6-242036

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Deutsche Telekom AG

Title:	Pseudo-CR on Verify UE location

Spec:	3GPP TR 23.700-72 v0.5.0

Agenda item:	8.1

Document for:	Approval

Contact:	Vasil Aleksiev, vasil.aleksiev@magenta.at

1. Introduction

The study item on enhanced application layer support for location services has a KI#4: Enhancement to support location information exposure. This pCR proposes new solution to KI#4.

2. Reason for Change

Currently the Application in the UE (LMC) does not have possibility to check directly its location. UE side is considered untrusted and with possibility the location to be spoofed or phone jailbraked and to commit different fraud with it. For example, there is a need to confirm the location to various applications on UE where this is important. Uber, Lyft, shopping apps, ordering food apps would be able to know if the respective UE/customer is at the service location where they intend to send the order or to initially approve the customer for a specific service. The ordering food application would not initiate the customer order if the location is not confirmed to save false orders/time and resources of related suppliers. Bank application will not allow registration if the customer is not within specific country. At the same time this functionality provides solution where the UE would not receive exact location from the LMS/network but only indications confirming the cell, country, city, home operator, roaming operator. Such indication would be enough to confirm the location to various applications where this is important.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72 v0.5.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |
| Sol #2 | X |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |
| Sol #5 |  |  |  | X |  |  |
| Sol #6 |  |  | X |  |  |  |
| Sol #7 |  |  |  |  | X |  |
| Sol #8 |  |  |  |  | X |  |
| Sol#X |  |  |  | X |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: Verify UE location

### 6.x.1	Description

This solution addresses the key issue #4: Enhancement to support location information exposure.

### 6.x.2	Procedures

#### 6.x.2.1	Confirm location service subscription

The application is running on a VAL server. The application would like to use confirm location service and subscribes to LMS for it using the following procedure:

Figure 6.x.2.1-1: Procedure of confirm location service subscription

- The VAL server subscribes to LMS to receive notification every time UE using specific application is requesting confirmation of location.

- The LMS shall check if the VAL server is authorized to subscribe for the confirm location service notifications for a specific application identifier.

- If the VAL server and specific application are authorized to subscribe for confirm location service, the LMS creates subscription based on provided parameters in the request.

4.	The LMS sends to VAL server confirm location service subscription response.

#### 6.x.2.2	Confirm location verification

The location management client (LMC)/application in the UE gets from GPS in the UE its location. The application in the UE wants to check if this is really the correct location.

Pre-conditions:

1.	The LMC is authorized to discover and to use Confirm Location API provided by the LMS;

2.	LMS is authorized to use Nnef Event Exposure API, based on SLA with NEF/5GS of MNO;

3.	It is assumed there is user consent given in the UE for exposing GPS location information from UE to LMS.

Figure 6.x.2.2-1: Procedure of confirm location verification

1.	The UE sends Confirm Location request from the LMC towards LMS. It contains GPS location measured from the UE GPS. Additional attributes: vertical accuracy, timestamp, course/direction, longtitude, speed, latitude, horizontal accuracy, altitude, carrier name, verify location token, cell id (known to the UE from the network signalling), application identifier.

2.	 The LMS shall check if there is subscription (based on VAL subscription above) for confirm location service with the specific application identifier received in step1.

3.	If such subscription is present, then requests UE location from 5GC/LMF via NEF (LMS send determine Location operation via NEF (N33 interface) and using Nlmf_Location API defined in TS29.572 – parameters available from the location reporting are described in the TS). It also requests accuracy to be a parameter present in the response. LMS can also perform a location information request to GMLC directly or via NEF (as defined in TS 23.273), acting as AF. LMS can also perform a location information request to 3rd party location servers via LM-3P interface. LMS can also use the T8 interface to obtain UE location. Or the LMS can have already the UE location based on the methods described in TS23.434. When the response comes, it contains the UE location from the network. LMS checks it and provides reply to the LMC – with location status:

4.	Verify location response - Different possibilities for the reply of the location: unknown, verified, mismatch in the same country, mismatch in other country, roaming country mismatch, error unauthorized, error other. Cell status Different possibilities for the reply: connected to specified cell, not connected to specified cell.

5.	The LMS sends notification towards the VAL server with the UE identifier and application using the service.

As the LMS matches the request from the LMC with the response from the Nlmf_Location API, this saves additional efforts if the network has to do the matching (the efforts are in the LMS). Also saves additional computing power at the UE (the efforts are in the LMS).

Such verify location API could be used by multiple applications used in the UE to monetize the location services provided from the network. The respective application logic would first use this API before dealing with customer requests and respectively would provide its services after the confirmed location.

### 6.x.3	Information flows

#### 6.x.3.1	General

The following information flows are specified for confirm location service based on clause 6.x.2.

#### 6.x.3.2	Confirm location service subscription request

Table 6.x.3.2-1 describes the information elements for the confirm location service subscription request from the VAL Server towards the location management server.

Table 6.x.3.2-1: Confirm location service subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user |
| Application identifier/VAL service id | M | Application identifier for which is the subscription request/ Identity of the VAL service for which the confirm location service is subscribed. |

#### 6.x.3.3	Confirm location service subscription response

Table 6.x.3.3-1 describes the information elements for the confirm location service subscription response from the location management server to the VAL Server.

Table 6.x.3.3-1: Confirm location service subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user |
| Subscription status | M | It indicates the subscription result |

#### 6.x.3.4	Confirm location request

Table 6.x.3.4-1 describes the information elements for the confirm location request from the location management client to the location management server.

Table 6.x.3.4-1: Confirm location request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of UE whose location information is to be confirmed |
| UE provided location | M | UE provided location taken from functionality within UE (e.g. GPS). It can include vertical accuracy, timestamp, course/direction, longtitude, speed, latitude, horizontal accuracy, altitude. |
| CellID | O | CellID identified in the UE from UE signalling |
| Timestamp | M | Timestamp of the UE provided location |
| ME ID | O | Mobile equipment identifier, e.g. IMEI |
| Application ID | M | Application identifier |

#### 6.x.3.5	Confirm location report

Table 6.x.3.5-1 describes the information elements for the confirm location request from the location management server to the location management client.

Table 6.x.3.5-1: Confirm location report

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of UE whose location information is to be confirmed |
| Confirm location status | M | unknown, verified, mismatch in the same country, mismatch in other country, roaming country mismatch, error unauthorized, error other |
| CellID status | O | Connected to provided cell, not connected to provided cell |
| Timestamp | M | Timestamp of the confirm location check |
| Application ID | M | Application identifier |

#### 6.x.3.6	Confirm location service usage notification

Table 6.x.3.6-1 describes the information elements for the confirm location service usage notification from the location management server to the VAL server.

Table 6.x.3.6-1: Confirm location service usage notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user |
| UE ID | M | The identifier of UE whose location information was confirmed |
| Timestamp | O | Timestamp of the confirm location service notification |

* * * End of Changes * * * *