3GPP TSG-SA WG6 Meeting #61	S6-242070

Jeju Island, South Korea,  20th – 24th May 2024	(revision of S6-242xxx)

Source:	Convida Wireless LLC

Title:	Update solution 07 positioning/ ranging

Spec:	3GPP TR 23.700-72 v0.5.0

Agenda item:	8.1

Document for:	Approval

Contact:	Mladin.Catalina@convidawireless.com

Ly.Quang@convidawireless.com

1. Introduction

This paper proposes an update to the solution on ranging / SL positioning enablement.

2. Reason for Change

This paper proposes an update to the solution on ranging / SL positioning enablement.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72 v0.5.0.

* * * First Change  * * *

## 6.7	Solution #7: Support for SL positioning/ranging enablement

### 6.7.1	Solution description

This solution provides a mechanism for configuring and provisioning via application layer information on the entities to be used and their responsibilities for a ranging / SL positioning service. This solution assumes that the VAL UEs of different capabilities and potential roles (reference, target, client) have instantiated a location enablement client (LMC). In this solution, an SL positioning enabler at LMS is authorized / delegated by SL positioning server to support the provisioning/delivery of ranging services in an area.

Figure 6.7.1-1 shows a high-level procedure with the details.

Figure 6.7.1-1: Support for ranging procedure

1.	VAL server (SL Positioning / Ranging Server) sends a subscription request to LMS to provision and manage ranging operation and configurations. This includes monitoring, discovering of UEs assisting the operation, and delivering the configuration for a target service area, or for a target set of UEs to serve as reference and target. The request can specify the identifiers of UEs and their assigned roles (e.g. reference UE, target UE), or include the UE selection criteria for different roles. This request can also include a list of client UEs that their location needs to be discovered (based on the client UE request).

2.	LMS sends a subscription response to requestor VAL Server.

3.	LMS finds all the LMCs in the target service area, and categorizes them by their capabilities as target UE, Reference UEs, or even Located UEs (if Reference UE location is known or for out of coverage scenarios). The identification of UEs in a target area can happen via existing SEAL LMS procedures.

4.	LMS after getting all UE information at the target area, calculates the relative distances among them, and also may request/receive HDMaps (e.g., from other VAL servers) for the target area. This will help identifying LoS or NLoS situations.

5.	LMS evaluates the need of reference UE, and if reference UE is needed, LMS selects the appropriate UE to server as reference  and configures reference UE with information, thresholds etc.., one time or period ranging, ranging for distance or direction measurement or both.The selection of UE as reference UEs can be based on their capabilities and the selection criteria specified by the VAL server. The LMS can also leverage analytics services (e.g. NWDAF) to obtain predictions of UE information (e.g. UE location prediction, relative distance prediction) to select the appropriate reference UE.

6.	LMS configures LMCs with all the configuration information from step 5 and in particular the expected responsibility in relation ranging/SL positioning application service.

In this step, Client UE gets provisioned also with ranging UEs, so as to be aware of the entity when he needs to request from LMS or from reference UE to get ranging parameter.

7.	The LMCs interact with their VAL clients in the corresponding VAL UEs (of client, reference, target) of to convey the configuration.

8	LMS sends a notification to the VAL server to indicate the reference UE and the feasibility of SL positioning based on steps 4 and 5.

- The VAL server sends a subscription request to LMS to initiate ranging operation for the target and reference UEs. The request includes the client, target and reference UEs, the SL positioning and ranging information (e.g. range, direction, relative position, relative velocity, etc.) requested, reporting interval and events, QoS requirement, and expiration of ranging request.

- The SEAL LMS sends a request to the Client UE to initiate the SL positioning and ranging procedure between the target and reference UEs using the information provided in step 9.

- The Client UE initiates the SL positioning and ranging operation as described in TS 23.586.

- The Client UE sends the SL positioning and ranging results to the SEAL LMS.

- The SEAL LMS sends a notification to th VAL server the requested SL positioning and ranging information. SEAL LMS may also be able to derive and expose additional information from the SL positioning and ranging results such as UEs within a field of view of the target UE.

* * * End of Changes * * *