3GPP TSG-SA WG6 Meeting #61	S6-242109

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	NTT Docomo

Title:	Pseudo-CR on the deployment of the API Invoker as AF on the UE for RNAA

Spec:	3GPP TR 23.946 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1. Introduction

This pCR presents the deployment of the API Invoker as AF on the UE for RNAA.

2. Reason for Change

If readers want to understand how to deploy API Invoker as AF on the UE for RNAA, the current information available can be difficult to comprehend. Therefore, additional information on how to deploy API Invoker as AF on the UE for RNAA is necessary.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.946.

* * * First Change * * * *

## 6		Exemplary Use Cases and Adoption

## Editor's Note: This clause will provide the main representative usage examples for CAPIF and adoption in 3GPP, ETSI, etc.

6.x	 The deployment of the API Invoker as AF on the UE for RNAA

Several RNAA deployments are described in clause 7.5 of 3GPP TS 23.222 [2], and the following is an example of a RNAA deployment in which API invoker is deployed as AF on the UE.

In this deployment, the API Invoker (UE A) requests to obtain the resource (e.g. location) of UE B from the Resource Owner (UE B) via the API exposing function.

Figure 6.x-1 illustrates integrated fundamental deployment of the API Invoker as AF on the UE for RNAA.

Figure 6.x-1: The deployment of the API Invoker as AF on the UE for RNAA

Figure 6.x-2 illustrates the procedure for The deployment of the API Invoker as AF on the UE for RNAA.

Figure 6.x-2: The deployment of the API Invoker as AF on the UE for RNAA

The following procedure is for the deployment of the API Invoker as AF on the UE for RNAA:

1.	API invoker (UE A) onboards to the CAPIF core function.

2.	The API invoker (UE A) authenticates with the CAPIF core function.

3.	The API invoker (UE A) discovers the service API.

4.	The API invoker (UE A) authenticates with the AEF to access the service API prior to invoking the service API or upon the invocation.

5.	The API exposing function obtains the resource owner consent from the resource owner (UE B). The result of the resource owner consent response may be stored in the API exposing function.

6.	The API invoker (UE A) sends service API invocation request to the API exposing function.