3GPP TSG-SA WG6 Meeting #61	S6-242110

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	NTT DOCOMO

Title:	Pseudo-CR on the deployment of the API Invoker as AF on the network for RNAA

Spec:	3GPP TR 23.946 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1. Introduction

This pCR presents the deployment of the API Invoker as AF on the network for RNAA.

2. Reason for Change

If readers want to understand how to deploy API Invoker as AF on the network for RNAA, the current information available can be difficult to comprehend. Therefore, additional information on how to deploy API Invoker as AF on the network for RNAA is necessary.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.946.

* * * First Change * * * *

## 6		Exemplary Use Cases and Adoption

## Editor's Note: This clause will provide the main representative usage examples for CAPIF and adoption in 3GPP, ETSI, etc.

6.x	 The deployment of the API Invoker as AF on the network for RNAA

In this deployment, UE A requests Resource Owner via API Invoker (AF) for consent to obtain the resource (e.g. QoS) of UE A via the API exposing function.

Figure 6.x-1 illustrates integrated fundamental deployment of the API Invoker as AF on the network for RNAA.

Figure 6.x-1: The deployment of the API Invoker as AF on the network for RNAA

Figure 6.x-2 illustrates the procedure for the deployment of the API Invoker as AF on the network for RNAA.

Figure 6.x-2: The procedure for the deployment of the API Invoker as AF on the network for RNAA

The following procedure is for API Invoker deployment on the network as AF for RNAA:

1.	The application on the UE A consumes the service from the AF.

2.	API invoker onboards to the CAPIF core function.

3.	The API invoker authenticates with the CAPIF core function.

4.	The API invoker discovers the service API.

5.	The API invoker authenticates with the AEF to access the service API prior to invoking the service API.

6.	The API exposing function obtains the resource owner consent from the resource owner (UE B). The result of the resource owner consent response may be stored in the API exposing function.

7.	The API invoker sends service API invocation request to the API exposing function.