3GPP TSG-SA WG6 Meeting #61	S6-242113

Jeju Island, Sotuh Korea, 20th - 24th May 2024	(revision of S6-24xxxx)

Source:	Apple, Vodafone

Title:	Pseudo-CR on updating CAPIF Use Case of NEF publishing update

Spec:	3GPP TR 23.946 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

This contribution builds on contribution S6-241358 “Pseudo-CR on CAPIF Use Case of NEF publishing APIs”, accepted at SA6#60.

2. Reason for Change

Highlights that the NEF (API Management Function: AMF) requires knowledge of the appropriate CCF endpoint. Provides clarifications on the information the AMF is required to provide in the register API provider request (including the authorization header as specified in clause C.5 of 3GPP TS 33.122) and updates the JSON scheme accordingly. Highlight that the CCF provides the AMF with the required certificates for each of the offered API provider functions and the necessary identities. Provides clarifications on the role of the API Exposure Function (AEF) profiles and the assignment of the service API identifier. Completes the schema for the CAPIF access control policy and proposes deletion of the associated EN.

3. Proposal

It proposed to accept the changes 6.3 to v0.4.0 TR 23.946 presented in this contribution.

* * * First Change * * *

## 6.4	NEF Publishes an API

The 5GS network exposure function (NEF) can use CAPIF capabilities to publish and manage its APIs, including managing charging. One of several possible deployment arrangements is shown in Figure 6.4-1.

Figure 6.4-1: NEF implements the CAPIF API provider domain functions

Signalling for the NEF to publish its APIs to the CAPIF core function is shown in Figure 6.4-2, followed by example request and response content.

A pre-condition is that the NEF (API Management Function) has been made aware of the appropriate CAPIF Core Function endpoint with which to initiate the registration, as well as having been provided with a token to access the CAPIF Core Function.

Figure 6.4-2: Publish a service API

Details of the signalling flow in Figure 6.4-2 are as follows:

NOTE: The JSON snippets provided in the following clauses are aligned with version 18.5.0 of 3GPP TS 29.222 [3], in which apiVersion is specified as "v1".

1. Register_API_Provider

The API provider includes:

-	Authorization HTTP header (access token)

-	Security information enabling the CCF to validate the registration (regSec)

-	An array (apiProvFuncs) of at least one API provider domain function profiles

-	Optionally API provider domain information (apiProvDomInfo) and the API provider name (apiProvName)

Table 6.4-1: Register API provider request

HTTP POST CAPIF_core_function_URL/{apiRoot}/api-provider-management/v1/registrations Request headers: Authorization: "string" Request body: { "regSec": "string", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "string", }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

The CAPIF core function assigns identities for the API provider domain (apiProvDomId) and each of its functions (apiProvFuncId) and returns them in the response. Furthermore, it provides certificates (apiProvCert) for each of the API provider functions based on each function’s provided public key (apiProvPubKey).

2. 201 Created

The API provider domain has been registered successfully The URI of the created resource is returned in the "Location" HTTP header, including the assigned registration identifier (registrationId).

Table 6.4-2: Register API provider response

201 Response headers: Location: {apiRoot}/api-provider-management/v1/registrations/{registrationId} Response body: { "apiProvDomId": "string", "regSec": "string", "apiProvFuncs": [ { "apiProvFuncId": "string", "regInfo": { "apiProvPubKey": "string", "apiProvCert": "string" }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

3. CAPIF_Publish_Service_API

The API publishing function sends information on the offered APIs by providing the API exposing function profiles array (aefProfiles). The published APIs resource created in response to the request represents all published service APIs of the NEF.

Table 6.4-3: Publish API request

HTTP POST CAPIF_core_function_URL/{apiRoot}/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ "object" ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string", "apiProvName": "string" }

4. 201 Created

The service API has been published successfully The URI of the created resource is returned in the "Location" HTTP header, including the assigned service API identifier (serviceApiId) that is also equal to the API identifier (apiId) in the response body.

Table 6.4-4: Publish API response

201 Response headers: Location: {apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId} Response body: { "apiName": "string", "apiId": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ "object" ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string", "apiProvName": "string" }

5. CAPIF administrator creates policy

Table 6.4-5: CAPIF administrator creates policy

{apiRoot}/access-control-policy/v1/accessControlPolicyList/{serviceApiId} { "apiInvokerPolicies": [ { "apiInvokerId": "string", "allowedTotalInvocations": "integer", "allowedInvocationsPerSecond": "integer", "allowedInvocationTimeRangeList": [ { "startTime": "string", "stopTime": "string" } ] } ] }

6. CAPIF_Access_Control_Policy_API

Table 6.4-6: GET access control policy request

HTTP GET CAPIF_core_function_URL/{apiRoot}/access-control-policy/v1/accessControlPolicyList/{serviceApiId} Request body: { "apiInvokerPolicies": [ { "apiInvokerId": "string", "allowedTotalInvocations": "integer", "allowedInvocationsPerSecond": "integer", "allowedInvocationTimeRangeList": [ { "startTime": "string", "stopTime": "string" } ] } ] }

7. 200 OK

Table 6.4-7: GET access control policy response

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId": "string", "allowedTotalInvocations": "integer", "allowedInvocationsPerSecond": "integer", "allowedInvocationTimeRangeList": [ { "startTime": "string", "stopTime": "string" } ] } ] }

Editor's Note: The tables above showing JSON content are to be populated with example values.

* * * End of Changes * * *