3GPP TSG-SA WG6 Meeting #61	S6-242123

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Samsung

Title:	Pseudo-CR on Updates to the AIML enablement client discovery procedure

Spec:	3GPP TR 23.700-82 V0.4.0

Agenda item:	8.3

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

UE capability information such as memory, computing resource storage etc., is hard to be quantized and standardized. Also as part of the S6-241615 VAL UE capabilty analytics term is updated to application layer AI/ML member client capabiltiy.

2. Reason for Change

This pCR updates the Discovery criteria IE as part of the table 8.7.3-1 to remove the usage of the client capabilites.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 V0.4.0.

* * * First Change * * * *

### 8.7.3	Corresponding APIs

Table 8.7.3-1 shows the request sent by a VAL server to an AIML enablement server for the AIML enablement client discovery procedure.

Table 8.7.3-1: Request for AIML enablement client discovery procedure

| Information element | Status | Description |
| --- | --- | --- |
| VAL server identifier | M | The identifier of the VAL server. |
| Security credentials | M | Security credentials to authenticate and authorize the AIML enabler client. |
| Discovery criteria | M | Discovery criteria for finding suitable AIML enablement clients for AIML operations: |
|  |  | > Requested ML models |
|  |  | > Requested AIML roles: role for data collection, data preparation, training, or inferencing |
|  |  | > Target location (for data collection purposes) |
|  |  | > Application layer AI/ML member capability information (e.g. communication capability (e.g. maximum/mininum number of supported active connections)) that may be dedicated to AIML operations |
|  |  | > Dataset requirements that includes: >> dataset availability, including dataset size, age, features list, and data distribution of each feature >> Data capabilities such as data collection and supported data processing capabilities |
|  |  | > Required service: >> Required VAL service ID or Service type (e.g. Slice/Service Type SST) that the client support >> Required corresponding service permission level (e.g. raw data, processed data, premium resource usage standard resource usage, limited resource usage) |

Editor's Note: Which IEs in Table 8.7.3-1 discovery criteria are optional, and mandatory is FFS.

Table 8.7.3-2 shows the response sent by the AIML enablement server to the VAL server for the AIML enablement client discovery procedure.

* * * End Change * * * *