3GPP TSG-SA WG6 Meeting #61	S6-242141

Jeju Island, South Korea, 20th – 24th May 2024

Source:	China Mobile

Title:	Evaluation for Solution#6

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	zhengshaowen@chinamobile.com

1. Introduction

This paper provides evaluation for Solution #6: Multi-modal flows alignment and monitoring.

2. Reason for Changes

Evaluation for Solution #6 is needed.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v0.3.0.

* * * First Change * * * *

7.6	Solution #6: Multi-modal flows alignment and monitoring

7.6.1	Architecture Impacts

- This solution requires a mechanism for data storage to cache the traffic flows.

7.6.2	Solution description

7.6.2.1	General

- Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

- To avoid the downlink traffic flow delay difference introduced from (R)AN and Internet, the SEALDD/XRApp client could do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred to application client at specific time.

- As shown in figure 7.6.2.1-1, flow#1 and flow#2 are associated flows. They are sent out from VAL server at the same time, while flow#1 arrives at SEALDD/XRApp client earlier than flow#2 due to different routing path. To align those associated flows, the SEALDD/XRApp client holds and buffers the flow#1 for a while. Until all the associated flows arrived(i.e., flow#2), the SEALDD/XRApp client sends out those associated flows to application client at the same time.

Editor’s note: Whether the implementation entity is XRApp or SELADD depends on the the Application enablement architecture.

Figure 7.6.2.1-1: DL traffic flow alignment

7.6.2.2	Procedure of flow alignment

- Figure 7.6.2.2-1illustrates the procedures of multi-modal flows alignment.

- Precondition:

-	The VAL server can discover and select the XRApp server by CAPIF functions.

-	The connection has been established among the UE, XRApp server and VAL server.

SEALDD/XRApp client

5GC

SEALDD/XRApp server

1. multi-modal flows coordination request

VAL server

5. flows coordination

4. multi-modal flows coordination response

connection established

2. multi-modal flows coordination request

3. multi-modal flows coordination response

Application client

Figure 7.6.2.2-1: multi-modal flows alignment

- VAL server sends the multi-modal flows alignment request with VAL ID, UE ID(s), flows description (e.g., Multi-modal Service ID, packet filter description, etc.), flows transmission requirement, flows alignment assistance information. Flows transmission requirement including the delay requirement. Flows alignment assistance information may include the timestamp (e.g., timestamp in the RTP header), number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server sends the multi-modal flows alignment request to the SEALDD/XRApp client identified by the UE ID(s), along with the information received in step1.

If the maximum acceptable duration for traffic flow alignment is not provided, then the XRApp server would determine the maximum acceptable duration for traffic flow alignment based on flows transmission requirement. The server may translate the flows description into SEALDD flow ID.

- The SEALDD/XRApp client sends the multi-modal flows alignment response to the SEALDD/XRApp server.

- The SEALDD/XRApp server sends the multi-modal flows alignment response to the VAL server indicating the result of multi-modal flows alignment.

- When the traffic flow arrived, the SEALDD/XRApp client initiate the multi-modal flows alignment based on the received information. The flows needs to be aligned is detected by the VAL ID, flows description. If number of associated flows are provided, the number of associated flows are used to determine whether all the associated flows arrived. If the timestamp is provided, the flows that are sent at specific time will be aligned. After all associated flows arrived, the SEALDD/XRApp client sends the flows to the application client. If the maximum acceptable time duration is provided, once the maximum acceptable time is reached, the SEALDD/XRApp client will no longer waits for the associated flows even if the associated flow is not arrived yet.

NOTE: Step3 and step5 can be executed in parallel.

7.6.2.3	Procedure of flow alignment monitoring

- Figure 7.6.2.3-1illustrates the procedure of multi-modal flows alignment monitoring.

- Precondition:

SEALDD/XRApp client

5GC

SEALDD/XRApp server

1. multi-modal flows coordination monitoring request

VAL server

5. flows coordination monitoring

7. multi-modal flows coordination monitoring notification

multi-modal flows alignment

2. multi-modal flows coordination monitoring request

4. multi-modal flows coordination monitoring response

6. multi-modal flows coordination monitoring notification

3. multi-modal flows coordination monitoring response

-	The multi-modal flows alignment has been requested and triggered.

Figure 7.6.2.3-1: traffic flow alignment monitoring

- VAL server sends the multi-modal flows alignment monitoring request. VAL ID, UE ID, flows description, monitoring period are provided in the request.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server sends the multi-modal flows alignment monitoring request to the SEALDD/XRApp client.

- The SEALDD/XRApp client sends the multi-modal flows alignment monitoring response to the SEALDD/XRApp server.

4.	The SEALDD/XRApp server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

5.	The XRApp client calculates the end-to-end delay difference based on the arrival time and timestamp(e.g., timestamp in the RTP header) among the flows.

Editor’s note: The detail of the end-to-end delay difference calculation is FFS.

6-7.XRApp client sends the multi-modal flows alignment monitoring result to the XRApp server, then to the VAL server.

7.6.x	Solution evaluation

This solution addresses Key issue #1: KPI measurement and exposure, it proposes multi-modal flows alignment and monitoring to optimize the delay difference of multi-modal flows.

For multi-modal flows alignment, VAL server sends multi-modal flows alignment request to SEALDD/XRApp client via SEALDD/XRApp server, including VAL ID, UE ID(s), flows description, flows transmission requirement and flows alignment assistance information. The SEALDD/XRApp client utilizes these information to identifies multi-modal flows and perform the alignment.

For multi-modal flows monitoring, VAL server sends multi-modal flows alignment monitoring request to SEALDD/XRApp client via SEALDD/XRApp server, including VAL ID, UE ID, flows description and monitoring period. The SEALDD/XRApp client utilizes these information to identifies multi-modal flows and perform the monitoring.

* * * End of Change(s) * * * *