3GPP TSG-SA WG6 Meeting #61	S6-242151

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.435 | CR | 0027 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update about NSI and Target Service Area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSCALE |  |  |  |  |  | Date: |  |  | 2024-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | AS asked in S6-242011 on Clarification related to NSI and Target Service Area, external AF is not aware of Network Slice Instance, so it is proposed to updates the request table. And for the Target Service Area in slice modification notify, it is expected to be geographical coordinates. The topological area (e.g. List of cells/TAs) is not supposed to be exposed to avoid exposing Internal 5G Core information outside the 3GPP operator domain. The initial intention to include the target DNN/DNAI information in the notification is to indicate the change of the EDN. Considering the slice change does not imply the EDN change, it is proposed to remove it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification about the usage of the Network Slice Instance information. Remove the NSI ID in the request from VAL server, correct the Target Service Area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Internal 5G Core information exposed outside the 3GPP operator domain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4, 9.9.3.2, 9.9.3.6, 9.13.3.2, 9.13.3.3, 9.16.3.2, 9.16.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.4	Network Slice related Identifier

The network slice related identifier identifies the network slice, which is mapped to the VAL application.

The identifier of the network slice is either S-NSSAI defined in 5.15.2.1 TS 23.501[16], or External Network Slice Information (ENSI) defined in TS 33.501[14].

Based on the operator’s policy, if the service applies for a certain network slice instance, the identifier of Network Slice instance is used, i.e. NSI ID as defined in TS 23.501 or ENSI. If used, the NSI ID is associated with S-NSSAI.

NOTE: Only when the NSCE server is provided by the MNO, which implies that the NSCE server is within the 3GPP operator domain, can it obtain the Network Slice Instance information based on the operator's policy.

* * * Next Change * * * *

9.9.3	Information flows

9.9.3.1	General

The following information flows are specified for the predictive slice modification support based on 9.9.2.

9.9.3.2	Application service continuity requirement request

Table 9.9.3.2-1 describes information elements for the Application service continuity requirement request from the VAL server to the NSCE server.

Table 9.9.3.2-1: Application service continuity requirement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Service Continuity Requirement | M | The service continuity requirement which can be the expected or predicted migration of the VAL application or a list of VAL UEs within the application to a target area. |
| Slice identifier | O | The slice identifier (S-NSSAI or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Target Service Area | O | The target area can be represented as the geographical coordinates / set of waypoints outside the original service area, where the VAL application/ UE(s) is expected or predicted to move. |
| Application QoS requirements | O | The QoS requirements / KPIs for the VAL service |

9.9.3.3	Application service continuity requirement response

Table 9.9.3.3-1 describes information elements for the Application service continuity requirement response from the NSCE server to the VAL server.

Table 9.9.3.3-1: Application service continuity requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement) |

9.9.3.4	Service continuity negotiation request

Table 9.9.3.4-1 describes information elements for the service continuity negotiation request from the S-NSCE server to the T-NSCE server.

Table 9.9.3.4-1: service continuity negotiation request

| Information element | Status | Description |
| --- | --- | --- |
| S-NSCE server ID | M | The identifier of the source NSCE server |
| VAL service ID | M | The identifier of the VAL service for which the request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the request applies |
| Service Continuity Requirement | M | The service continuity requirement which can be the expected or predicted migration of the VAL application or a list of VAL UEs within the application to a target area. |
| Proposed Trigger Action | O | The proposed slice licecycle change for the target VAL UE or VAL application |
| Slice identifier | M | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Application QoS requirements | O | The QoS requirements / KPIs for the VAL service |

9.9.3.5	Service continuity negotiation response

Table 9.9.3.5-1 describes information elements for the service continuity negotiation response from the T-NSCE server to the S-NSCE server.

Table 9.9.3.5-1: service continuity negotiation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement) |
| Trigger Action | O | The determined trigger action which can be the slice licecycle change for the target VAL UE or VAL application |

9.9.3.6	Slice modification notify

Table 9.9.3.6-1 describes information elements for the slice modification notify message from the NSCE server to the VAL server or the VAL client (via NSCE client).

Table 9.9.3.6-1: slice modification notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The identifier of the VAL application which is expected to be impacted by the slice modification |
| VAL UE ID list | O | The identifiers of the VAL UEs which are expected to be impacted by the slice modification |
| Slice identifier | M | The slice identifier (S-NSSAI or ENSI) which is expected or predicted to modify to extend slice availability to the target service area |
| Target NSCE server ID and address | M | The identifier and address of the target NSCE server |
| Target Service Area | M | The target area can be represented as the geographical coordinates / set of waypoints outside the original service area, for which the modification applies |

* * * Next Change * * * *

9.13.3.2	Inter-PLMN application service continuity requirement request

Table 9.13.3.2-1 and Table 9.13.3.2-2 describe information elements inter-PLMN application service continuity requirement request and response between the VAL server and the NSCE server.

Table 9.13.3.2-1: Inter-PLMN application service continuity requirement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Service Continuity Requirement | M | The service continuity requirement which can be the expected or predicted migration of the VAL application or a list of VAL UEs within the application to a target area. |
| Target PLMN ID | M | PLMN identifier of the target PLMN |
| Slice identifier | O | The slice identifier (S-NSSAI or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Target Service Area | O | The target area can be represented as the geographical coordinates / set of waypoints outside the original service area, where the VAL application/ UE(s) is expected or predicted to move. |
| Application QoS requirements | O | The QoS requirements / KPIs for the VAL service |

Table 9.13.3.2-2: Inter-PLMN application service continuity requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement) |

9.13.3.3	Inter-PLMN slice modification notify

Table 9.13.3.3-1 describes information elements for the inter-PLMN slice modification notify message from the NSCE server to the VAL server or the VAL client (via NSCE client).

Table 9.13.3.3-1: Inter-PLMN slice modification notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The identifier of the VAL application which is impacted by the slice modification |
| VAL UE ID list | O | The identifiers of the VAL UEs which are impacted by the slice modification |
| Slice identifier | M | The slice identifier (S-NSSAI or ENSI) which is used and/or modified to extend slice availability to the target service area |
| PLMN ID | M | PLMN identifier of the PLMN where modification was performed |
| Target Service Area | M | The target area, can be represented as the geographical coordinates / set of waypoints outside the original service area, for which the modification applies. |

* * * Next Change * * * *

9.16.3	Information flows

9.16.3.1	General

The following information flows are specified for slice requirements verification and alignment capability exposure.

9.16.3.2	Slice requirements verification and alignment subscription

Table 9.16.3.2-1 describes information elements for slice requirements verification and alignment subscription.

Table 9.16.3.2-1: Slice requirements verification and alignment subscription

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Slice identifier | M | The slice identifier (S-NSSAI or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Slice Requirements List | M | The list of the slice requirements which need to be aligned. |
| >sliceRequirement | M | The requirement which need to be aligned, this parameter refers to the attribute of serviceProfile defined in TS 28.541 [10] |
| Reporting criteria | O | Indicate the notification reporting criteria, e.g. generated slice requirement is outside the given slice requirement range |
| Automatic enforcement mode | O | Indicates the mode for automatic enforcement of the generated optimal slice requirements to meet the slice requirements. E.g OFF or ON |
| Alignment reporting criteria | O | Indicates the alignment reporting conditions to the VAL server e.g. based on count of UEs |

9.16.3.3	Response of slice requirements verification and alignment subscription

Table 9.16.3.3-1 describes information elements for response of slice requirements verification and alignment subscription.

Table 9.16.3.3-1: Response of slice requirements verification and alignment subscription

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the subscription (positive or negative acknowledgement) |

9.16.3.4	Slice requirements verification and alignment notification

Table 9.16.3.4-1 describes information elements for slice requirements verification and alignment notification.

Table 9.16.3.4-1: Slice requirements verification and alignment notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Slice identifier | M | The slice identifier (S-NSSAI or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Updated Slice requirements info | M | The attributes and values of slice requirements |
| > Slice requirement alignment enforcement | O | Indicates the updated slice requirement is enforced or not. E.g Yes or No |