3GPP TSG-SA WG6 Meeting #61	S6-242156

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0428 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change of the maximum number of simultaneous talkers by an authorized user |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the maximum number of simultaneous talkers an MCPTT group configured for multi-talker control is statically pre-configured per group. There is no option for easily changing that number if operational needs require that, i.e. even not an authorized user is able to change that number at least during a group call. This CR introduces a new procedure which allows an authorized user to increase or lower the number of simultaneous talkers during an ongoing group call to align that number for the duration of the group call with the actual operational needs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new procedure, message flow and configuration data to enable an authorized MCPTT user to change the maximum number of simultaneous talkers at any time during a group call in an MCPTT group configured for multi-talker control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current solution does not meet railway expectations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.9.1.2.x, (new) 10.9.1.3.x, Annex 4, Annex 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 10.9.1.2.x	Maximum number of simultaneous talkers request

Table 10.9.1.2.x-1 describes the information flow maximum number of simultaneous talkers request, from an authorized floor participant user to the floor control server, which is used to change the maximum number of simultaneous talkers in the group.

Table 10.9.1.2.x-1: Maximum number of simultaneous talkers request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the user who requests to change the maximum number of simultaneous talkers |
| Maximum number | M | Modified maximum number of simultaneous talkers |

# * * * * 2nd change * * * *

##### 10.9.1.3.x	Changing the maximum number of simultaneous talkers by an authorized user

Figure 10.9.1.3.x-1 shows the high-level procedure that an authorized MCPTT user changes the maximum number of simultaneous talkers at any time during a group call in an MCPTT group configured for multi-talker control.

Pre-condition:

1.	The participant A is authorised to change the maximum number of simultaneous talkers of the group.

2.	The group supports multi-talker floor control and the maximum number of simultaneous talkers is pre-configured or set to 2 for the group.

Figure 10.9.1.3.x-1: Changing the maximum number of simultaneous talkers by an authorized user

1.	An authorized user wants to increase the number of simultaneous talkers for the group.

2.	Floor participant A sends a change maximum number of talkers request message to change the maximum number of simultaneous talkers.

3.	The floor control server checks whether floor participant A is authorized to change the maximum number of simultaneous talkers.

4.	The floor control server sends floor idle messages towards the floor participants A and C. The users are notified that the floor is idle and requesting the floor is possible.

NOTE:	Reducing the maximum number of simultaous talkers may require that the floor is revoked from floor participants (this scenario is not shown in the figure above).

The modified maximum number of simultaneous talkers in the group is set in the floor control server until the group call is released or an authorized user modifies the maximum number of simultaneous talkers.

# * * * * 3rd change * * * *

# A.4	MCPTT related group configuration data

The general aspects of group configuration are specified in 3GPP TS 23.280 [16].

Parameters specified in table A.4-1 are child parameters of the "MCPTT configuration" parameter specified in table A.4-1 in 3GPP TS 23.280 [16]. Parameters specified in table A.4-2 are child parameters of the "MCPTT configuration" parameter specified in table A.4-2 in 3GPP TS 23.280 [16]. Parameters specified in table A.4-3 are child parameters of the "MCPTT configuration" parameter specified in table A.4-3 in 3GPP TS 23.280 [16].

Table A.4-1: Group configuration data (on and off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-6.4.9-006] of 3GPP TS 22.280 [17] | >> Preferred voice codecs for MCPTT group | Y | Y | Y |
| [R-5.7.2.1.1-013], [R-5.7.2.4.1-008] of 3GPP TS 22.280 [17] | >> Indication whether emergency group call is permitted on the MCPTT group | Y | Y | Y |
| [R-5.7.2.2.1-009] of 3GPP TS 22.280 [17] | >> Indication whether imminent peril group call is permitted on the MCPTT group | Y | Y | Y |
| [R-5.7.2.4.1-012] of 3GPP TS 22.280 [17] | >> Indication whether emergency alert is possible on the MCPTT group | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Media confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Floor control confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Group media protection security material (see NOTE) | Y | N | Y |
| NOTE: Security mechanisms are specified in 3GPP TS 33.180 [19]. |  |  |  |  |

Table A.4-2: Group configuration data (on-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in-progress emergency for an on-network group call | N | Y | Y |
| [R-5.7.2.2.2-004] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in progress imminent-peril group call for an on-network group call | N | Y | Y |
| [R-6.2.4-003], [R-6.4.9-002] of 3GPP TS 22.179 [2] | >> Group call hang timer | N | Y | Y |
| [R-6.4.9-003] of 3GPP TS 22.280 [17] | >> Maximum duration of group calls | N | Y | Y |
| Subclause 10.6.2 | >> Group call model used for group (chat/ pre-arranged) | Y | Y | Y |
| [R-6.2.1-004], [R-6.2.1-007] of 3GPP TS 22.179 [2] | >> Minimum number of affiliated group members acknowledging before start of audio transmission | N | Y | Y |
| [R-6.2.1-007] of 3GPP TS 22.179 [2] | >> List of group members which, if affiliated, have to acknowledge before start of audio transmission |  |  |  |
|  | >>>MCPTT IDs | N | Y | Y |
| [R-6.2.1-006], [R-6.2.1-007] of 3GPP TS 22.179 [2] | >> Geographical area where acknowledgement of all affiliated group members is required before start of audio transmission | N | Y | Y |
| [R-6.2.1-008], [R-6.2.1-009] of 3GPP TS 22.179 [2] | >> Timeout for acknowledgement of required group members | N | Y | Y |
| [R-6.2.1-008], [R-6.2.1-009], [R-6.2.1-012] of 3GPP TS 22.179 [2] | >> Action upon failure to receive acknowledgement from required group members before call timeout (proceed or abandon) | N | Y | Y |
| [R-6.2.1-001a] of 3GPP TS 22.179 [2] | >> Minimum number of group members which must be affiliated to allow the start of the group call | N | Y | Y |
| [R-6.2.1-001b] of 3GPP TS 22.179 [2] | >> List of group members which must be affiliated to allow the start of the group call |  |  |  |
|  | >>>MCPTT IDs | N | Y | Y |
| [R-6.4.9-004] of 3GPP TS 22.280 [17] | >> List of group members with receive-only capability (see NOTE 1) |  |  |  |
|  | >>>MCPTT IDs | N | Y | Y |
| [R-6.4.5-001], [R-6.4.5-003] of 3GPP TS 22.280 [17] | >> Authorisation of a user to request a list of affiliated members of a group | Y | Y | Y |
| [R-5.1.7-002], [R-6.2.2-001], [R-6.6.2.2-006,] [R-6.8.6.2-003] of 3GPP TS 22.280 [17] | >> Priority of the group | N | Y | Y |
| [R-6.8.6.2-003] of 3GPP TS 22.280 [17] | >> Pre-emption capability | Y | Y | Y |
| [R-6.8.6.2-003] of 3GPP TS 22.280 [17] | >> Pre-empted capability | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Group floor control protection security material for multicast use (see NOTE 2) | Y | Y | Y |
| [R-6.2.3.6.2-001] of 3GPP TS 22.179 [2] | >> Audio cut-in policy (enabled/disabled) | N | Y | Y |
| [R-6.2.3.7.2-001] of 3GPP TS 22.179 [2] | >> Support of multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | >> Maximum number of simultaneous talkers applicable for multi-talker control | N | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | >> List of group members which are allowed to change the maximum number of simultaneous talkers |  |  |  |
|  | >>> MCPTT ID | Y | Y | Y |
| Subclause 7.4.2.3.5 and subclause 7.4.2.3.6 | >> Audio mixing is performed in the UE or in the network to support multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-003] of 3GPP TS 22.179 [2], [R-5.2.1-001] of 3GPP TS 22.280 [17] | >> List of group members which are allowed to talk (see NOTE 1, NOTE 4) |  |  |  |
|  | >>> MCPTT ID | Y | Y | Y |
| 3GPP TS 23.283 [20] | >> Indication whether use of LMR E2EE is permitted on the MCPTT group | Y | N | Y |
| 3GPP TS 23.283 [20] | >> LMR specific identity for MCPTT group (see NOTE 3) | Y | N | Y |
| 3GPP TS 23.283 [20] | >> Group to key binding (see NOTE 3) | Y | N | Y |
| NOTE 1: A group member shall not be listed in both the list of group members with receive only capability and the list of group members which are allowed to talk. NOTE 2: The details of security related elements are specified in 3GPP TS 33.180 [19]. NOTE 3: This is an LMR specific parameter with no meaning within MC services. NOTE 4: This parameter is applicable for MCPTT groups with multi-talker control / sets the authorization to talk in pre-arranged groups used in broadcast calls. |  |  |  |  |

Table A.4-3: Group configuration data (off-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in-progress emergency for an off-network group call | Y | N | Y |
| [R-5.7.2.2.2-004] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in progress imminent-peril group call for an off-network group call | Y | N | Y |
| [R-7.4-002] of 3GPP TS 22.179 [2] | >> Group call hang timer | Y | N | Y |
| [R-7.4-003] of 3GPP TS 22.280 [17] | >> Max duration of group calls | Y | N | Y |
| Subclause 10.6.3.9.1 | >> Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | >>> MCPTT group call signalling | Y | N | Y |
|  | >>> MCPTT group call media | Y | N | Y |
|  | >>> MCPTT emergency group call signalling | Y | N | Y |
|  | >>> MCPTT emergency group call media | Y | N | Y |
|  | >>> MCPTT imminent peril group call signalling | Y | N | Y |
|  | >>> MCPTT imminent peril group call media | Y | N | Y |

# A.5	MCPTT service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [16]. The MCPTT service configuration data is stored in the MCPTT server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCPTT service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCPTT service. Data in tables A.5-1and A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCPTT service configuration data (on and off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] of 3GPP TS 22.280 [17] | Levels of group hierarchy for group-broadcast groups (Bc1) | Y | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [17] | Levels of user hierarchy for user-broadcast groups (Bc2) | Y | Y | Y |
| [R-5.8-002] of 3GPP TS 22.280 [17] | Minimum length (Nc3) of an alphanumeric identifier (i.e. alias) assigned by an MCPTT administrator. | Y | N | Y |

Table A.5-2: MCPTT service configuration data (on-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.3.2-002] of 3GPP TS 22.179 [2] | Timeout value for the cancellation of an in-progress emergency for an on-network private call | N | Y | Y |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | Time limit for an in-progress emergency related to an on-network MCPTT group | N | Y | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Max on-network private call (with floor control) duration | N | Y | Y |
| [R-6.2.4-003] of 3GPP TS 22.179 [2] | Hang timer for private calls | N | Y | Y |
| [R-6.7.2-008] of 3GPP TS 22.280 [17] | Max duration of private call (without floor control) | N | Y | Y |
| [R-6.2.3.3.1-001] of 3GPP TS 22.179 [2] | Hierarchy of participant rights to override | N | Y | Y |
| [R-6.2.3.5-002] of 3GPP TS 22.179 [2] | Transmit time limit from a single request to transmit in a group or private call transmission | N | Y | Y |
| [R-6.2.3.5-003], [R-6.2.3.5-004] of 3GPP TS 22.179 [2] | Configuration of warning time before time limit of transmission is reached (on-network) | N | Y | Y |
| [R-6.2.4-005] of 3GPP TS 22.179 [2] | Configuration of warning time before call hang time (on-network) | N | Y | Y |
| [R-6.2.3.2-006] of 3GPP TS 22.179 [2] | Depth of floor control queue | N | Y | Y |
| [R-6.2.3.2-012] of 3GPP TS 22.179 [2] | Max time for a user's floor control request to be queued | N | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Protect confidentiality of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Protect integrity of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Use signalling protection between MCPTT servers (see NOTE 1) | N | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Use floor control protection between MCPTT servers (see NOTE 1) | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Functional alias | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [17] | > Communication priority (see NOTE 2) | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > This functional alias can be taken over | N | Y | Y |
|  | > List of users |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | >> MCPTT ID | N | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Max number immediate forwardings | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [17] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
| Subclause 5.15 of 3GPP TS 22.280 [17] | List of permitted GW MC service ID(s) |  |  |  |
|  | > GW MC service ID | Y | Y | Y |
|  | Ad hoc group call configurations |  |  |  |
| [R-6.15.5.3-005] of 3GPP TS 22.280 [17] | > Support of ad hoc group call (enabled/disabled) (see NOTE 3) | Y | Y | Y |
| [R-6.15.5.3-002] of 3GPP TS 22.280 [17] | > Maximum number of participants allowed to participate in an ad hoc group call | Y | Y | Y |
| [R-6.15.5.3-004] of 3GPP TS 22.280 [17] | > Hang timer for ad hoc group call | N | Y | Y |
|  | > Maximum duration for ad hoc group call | Y | Y | Y |
|  | > List of preferred voice codecs for ad hoc group call | Y | Y | Y |
| Subclause 6.2.3.7.13 of 3GPP TS 22.179 [2] | > Multi-talker floor control supported | Y | Y | Y |
| Subclause 7.4.2.3.5 and subclause 7.4.2.3.6 | > Audio mixing is performed in the UE or in the network to support multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | > Maximum number of simultaneous talkers applicable for multi-talker control | N | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | > List of group members which are allowed to change the maximum number of simultaneous talkers |  |  |  |
|  | >> MCPTT ID | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [19]. NOTE 2: The usage of this parameter by the MCPTT server is up to implementation. NOTE 3: If the support for ad hoc group call is disabled by the MC system then all other configurations related to ad hoc group call are not applicable. |  |  |  |  |

Table A.5-3: MCPTT service configuration data (off-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.3.2-002] of 3GPP TS 22.179 [2] | Timeout value for the cancellation of an in-progress emergency for an off-network private call | Y | N | Y |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | Time limit for an in-progress emergency related to an off-network MCPTT group | Y | N | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Max off-network private call (with floor control) duration | Y | N | Y |
| [R-7.4-002] of 3GPP TS 22.179 [2] [R-7.4-003] of 3GPP TS 22.280 [17] | Hang timer for private calls in off-network | Y | N | Y |
| [R-7.3.3-001], [R-7.3.3-002], [R-7.3.3-003] of 3GPP TS 22.179 [2] | Priority hierarchy for floor control override in off-network | Y | N | Y |
| [R-7.3.5-001], [R-7.3.5-002], [R-7.3.5-003] of 3GPP TS 22.179 [2] | Transmit time limit from a single request to transmit in a group or private call. | Y | N | Y |
| [R-7.3.5-001], [R-7.3.5-004] of 3GPP TS 22.179 [2] | Configuration of warning time before time limit of transmission is reached (off-network) | Y | N | Y |
| [R-7.4-004] of 3GPP TS 22.280 [17] | Configuration of warning time before hang time is reached (off-network) | Y | N | Y |
| [R-7.7-001], [R-7.7-003] of 3GPP TS 22.280 [17] [R-7.7-002] of 3GPP TS 22.179 [2] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCPTT private call signalling | Y | N | Y |
|  | > MCPTT private call media | Y | N | Y |
|  | > MCPTT Emergency private call signalling | Y | N | Y |
|  | > MCPTT Emergency private call media | Y | N | Y |
| [R-7.15-001], [R-7.7-003] of 3GPP TS 22.280 [17] | Configuration of metadata to log | Y | N | Y |

# * * * * End of changes * * * *