3GPP TSG-SA WG6 Meeting #61	S6-242160

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0430 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-talker floor control involving from multiple MCPTT systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Floor request, floor granted, and multi-talker floor taken during an MCPTT session enhanced with multi-talker control and the related media flows are described for a single MCPTT system. This CR adds the related procedure when multiple MCPTT systems are involved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds a new procedure describing how the multi-talker feature is applied when participants are in multiple MCPTT systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current solution does not meet railway expectations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.9.1.3.1c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change * * * *

##### 10.9.1.3.1c	Floor request, floor granted and multi-talker floor taken during an MCPTT session enhanced with multi-talker control for multiple systems

The MCPTT users belong to different to multiple MCPTT systems and participate in an MCPTT group communication. The floor control arbitration resides with the primary MCPTT system and is enhanced with the multi-talker control feature.

Figure 10.9.1.4.x-1 describes the procedure for floor control enhanced for multi-talker control involving multiple MCPTT systems.

Pre-conditions:

1.	The floor control server 1 belongs to the primary MCPTT system.

2.	The MCPTT group is configured to support multi-talker control and audio mixing by the network is applied.

3.	MCPTT user A is in the primary MCPTT system, MCPTT user B and C are the partner MCPTT system.

3.	An MCPTT group call is established between MCPTT clients (client A, client B and client C).

4.	Participants and A and B have the permission to talk to all other participants and the floor is granted to floor participant B in the partner system.

Figure 10.9.1.4.x-1: Floor control enhanced for multi-talker control involving multiple MCPTT systems

1.	Floor participant B (partner MCPTT system) is talking and is sending the voice media.

2.	Floor participant A (primary MCPTT system) wants to send voice media over the session.

3.	Floor participant A (primary MCPTT system) sends a floor request message to the floor control server in the primary MCPTT system which includes the necessary information, e.g. floor priority.

4.	Based on applicable criteria (e.g. floor priority, participant type, allowance to transmit, maximum number of simultaneous talkers) floor control server determines what action (grant, deny, or queue) shall be applied to the request. In this case, the floor request from floor participant A will be accepted. Simultaneous floor requests to transmit are handled in a sequential order. Based on the group configuration repository data, the floor control server may limit the time a floor participant is allowed to talk.

5a.	The floor control server in the primary MCPTT system responds with a floor granted message to floor participant A (primary MCPTT system).

5b.	The floor control server (primary MCPTT system) sends a multi-talker floor taken message to the floor participant B in the partner MCPTT system.

5c.	The floor control server (primary MCPTT system) sends another multi-talker floor taken message to the floor participant C in the partner MCPTT system.

5d. Floor control server (primary MCPTT system) may send a multi-talker floor taken message to floor participant A.

6a.	The floor granted shall cause the user of UE A, where the floor participant A is located, to be notified.

6b.	The multi-talker floor taken shall inform the user of UE B, that the floors are granted to other floor participants, but the floor is not revoked.

6c. The multi-talker floor taken shall inform the user of UE C floor participants list the floor are currently granted to.

7.	Floor participant A starts sending voice media over the session established beforehand, i.e. participants A and B receive and transmit voice media; participant C only receives voice media.

# * * * * End of change * * * *