3GPP TSG-SA WG6 Meeting #61	S6-242247

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Samsung

Title:	Pseudo-CR on update to solution 3 and solution evaluation

Spec:	3GPP TR 23.700-21 V0.3.0

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah, sapan.shah@samsung.com

1. Introduction

This pCR proposes to update solution 3.

2. Reason for Change

This solution adds missing information in the subscription request, adds corresponding API details and lastly provides solution evaluation.

The solution is feasible and can be adapated to final decided architecture option.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 V0.3.0.

* * * First Change * * * *

#### 7.3.1.3	VAL client subscription procedure for spatial anchors in range

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL client requires information about spatial anchors of interest

Figure 7.3.1.3-1: Spatial anchor of interest subscription/notification procedure

1.		A metaverse VAL client subscribes for spatial anchor of interest information with the mobile metaverse enabler client (MMEC), and the subscription includes a VAL client identifier and spatial anchor requirements. The spatial anchor requirements includes spatial anchor of interest discovery filters and a range. The detection filters may include spatial anchor identifiers to detect specific spatial anchors, spatial anchor associated service identifiers to detect spatial anchors associated with a service identifier, location area to detect spatial anchors within a location area, range to detect spatial anchors within a range, an indication to enable persistent search to notify spatial anchors within range of UE’s location, pose of the user along with direction of the movement, optionally product specific information. The subscription may also indicate whether notifications are for spatial anchors becoming in range of the UE, becoming out of range of the UE or both.

NOTE:	The product specific information may contain information like product name, proce, type, manufacturing company and date, expiry date, ratings, etc. Such information is in key-value pair and transperant to the mobile metaverse enabler.

2.	The MMEC sends a spatial anchor of interest subscription request to the mobile metaverse enabler server (MMES). The request includes the MMEC identifier, VAL client identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and discovery filters provided by the VAL client.

3.	Upon receiving the request, the MMES authorizes the MMEC subscription request and performs the necessary actions to detect spatial anchor of interest. To detect spatial anchor of interest within range or becoming in-range or out-of-range of the UE, the MMES starts monitoring the corresponding UE location and pose information of the user who requested to enable the persistent search. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [7] and TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction. The MMES may obtain data analytics by invoking NEF APIs as described in TS 23.288 [10] and TS 23.502 [9]. The MMES may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs becomes in-range or out-of-range of the spatial anchors of interest.

4.	The MMES sends a spatial anchor of interest subscription response to the MMEC. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

5.	Based on the detection filters provided in the subscription request, the MMES monitors the distance between the current UE location and the spatial anchor of interest, and identifies the list of spatial anchors which are within range of the UE.

6.	The MMES sends a spatial anchor of interest notification to the subscribed MMEC; the notification includes the subscription identifier and information about the detection of spatial anchors of interest (e.g., spatial anchor identifier(s), range from the UE, , etc.).

7-	The MMEC provides to the VAL client the information received in the notification about the spatial anchors of interest.

* * * Next Change * * * *

### 7.3.3	Corresponding APIs

Table 7.3.3-1 shows the request sent by a MMEC to a MMES for the UE with SA interest subscription reuqest.

Table 7.3.3-1: UE with SA interest subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Notification endpoint | M | Endpoint where notification is recieved |
| Spatial anchors discovery filter | M | Discovery filters to identify UEs |
| > Spatial anchor identifier(s) | M | Identifier of the spatial anchors that have discovered by the UE |
| > Area of interest | O | Location area of interest within which UEs have discovered spatial anchors |
| > Application specific range | O | Application specific range within which UEs have discovered spatial anchors |
| > Event | O | UE Entering the spatial anchor range, UE leaving the spatial anchors range |

Table 7.3.3-2 shows the response sent by the MMES server to the requester for a spatial anchor discovery response.

Table 7.3.3-2: UE with SA interest subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Subscription identity | O | Identify of the subscription (for successful case) |
| Failure cayse | O | Indicates reason for the failure |

Table 7.3.3-3 shows the notification sent by a MMES server to the requester for the UE with SA interest notification.

Table 7.3.3-1: SA interest notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identify of the subscription (for successful case) |
| List of spatial anchors | M | UE details for each spatial anchors |
| > List of UE(s) | O | List of UEs entering or leaving the spatial anchors range |
| >> event type per UE | O | Indicating UEs entering or leaving the spatial anchors range |

Table 7.3.3-4 shows the request sent by a MMEC to a MMES for the SA of interest subscription reuqest.

Table 7.3.3-4: SA of interest subscription reuqest

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Notification endpoint | M | Endpoint where notification is recieved |
| Spatial anchors discovery filter | M | Discovery filters to identify spatial anchors |
| > Spatial anchor identifier(s) | M | Identifier of the spatial anchors to detect specific spatial anchors |
| > Area of interest | O | Location area of interest within which spatial anchors are interested |
| > Application specific range | O | Application specific range gtom UE within which spatial anchors have discovered spatial anchors |
| > current UE location | O | Current location of the UE |
| > current pose | O | Current post of the user (as defined in clause 6.2.2 of 3GPP TS 26.119 [11]) |
| > product specific information | O | Product specific information in key-value pair – which is transperant to the enabler layer. |

Table 7.3.3-2 shows the response sent by the MMES server to the requester for a spatial anchor discovery response.

Table 7.3.3-2: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Subscription identity | O | Identify of the subscription (for successful case) |
| Failure cayse | O | Indicates reason for the failure |

Table 7.3.3-3 shows the notification sent by a MMES server to the requester for the UE with SA interest notification.

Table 7.3.3-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identify of the subscription (for successful case) |
| List of spatial anchors | M | List of spatial anchors within range of the UE |

* * * Next Change * * * *

### 7.3.4	Solution evaluation

This solution resolves KI #1. This solution is based on architecture option #1. The solution proposes methods to get list of UEs which discovered spatial anchors, and also to get list of spatial anchors within range of UE based on persistent search. The solution is feasible. The solution will be adapted to the chosen architecture.

* * * End of Change * * * *