3GPP TSG-SA WG6 Meeting #61	S6-242256

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Title:	LS on alignment for API provider entity status and API status

Response to:

Release:	Rel-19

Work Item:	FS_CAPIF_Ph3

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG5

Cc:

Contact person:	Jian Zhang

zhangjian369@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

In Rel-19, SA6 has on-going study for further enhancement for Common API Framework (i.e. CAPIF_Ph3). Regarding of CAPIF, it introduces the CAPIF core function (CCF) to support the API publish and API discovery. The API invoker can use CCF to find the service API information, and send the API invocation to API provider entity/API exposing function (AEF). AEF can be any entity that needs to expose its service API information, such as edge application server (EAS) and network exposure function (NEF).

SA6 would like to ask the SA5 the following questions:

Question #1: How to identify the status for API provider entity (e.g. EAS and NEF) to indicate the related instance status is available, unavailable, and potentially available. For example, in SA6 EDGEAPP, the EAS status is instantiable but not to be instantiated (i.e. potentially available) means that the EAS instance is instantiated (i.e., available) after instantiation by management system.

Question #2: Is it possible to sending the request by authorized consumer (e.g. CCF) to management system, to request the potentially available API provider entity to make it available?

Question #3: Is it possible only activate the required API, i.e. it means that not all API of API provider is activated if the API provided entity is instantiated by management system.

Question #4: If the answer for question#3 is yes, how to identify the API status, to indicate the API is potentially available, it can be available after the API is activated.

2	Actions

To SA5

ACTION: 	SA6 kindly request SA5 to provide the answer for the above question.

3	Dates of next TSG SA WG 6 meetings

SA6#61                 20th May – 24th May 2024 	Jeju, Korea

SA6#62                 19th August – 23rd August 2024 	Maastricht, Netherlands