3GPP TSG-SA WG6 Meeting #61	S6-242262

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0074 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification about SEALDD enabled connection establishment based on policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the clause 9.2.2.3 about the SEALDD enabled regular data transmission connection establishment based on policy, there are the following issue need to be further clarified: Issue#1: The established regular data transmission connection is for the VAL application, that has the requirement for data transfer via SEALDD layer. The UE entering the location area is not equivalent to the UE having data transfer requirements, it may cause the unnecessary connections to be established. Issue #2: The SEALDD server should be aware the SEALDD client address and the connection establishment requirements for data traffic transfer before the step 5, but it is not easy to obtain these informations by the SEALDD server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add the related description to capture the possible issue for the policy based establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will cause the unclear use case and uncessary connection establishment in SEALDD-Uu interface for the policy based establishment procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.2.3	SEALDD enabled regular data transmission connection establishment based on policy

The SEALDD servers has Data Delivery (DD) policy being provisioned. Before the application communication between VAL client and VAL server starts, the DD policy is enforced by the SEALDD server to establish the SEALDD connection.

Pre-conditions:

1.	The SEALDD server has DD policies available.

Figure 9.2.2.3-1: Policy enforced by SEALDD server for connectivity

1.	The VAL server subscribes to SEALDD event exposure for connection status using the procedure defined in clause 9.2.2.6.

NOTE 1:	The VAL server can update/delete an existing subscription at the SEALDD Server when required.

2.	When the time for data transmission is about to start, the SEALDD server enforces the policy for connection setup with establishment event to trigger regular data transmission connection establishment. If spatial condition for UE is provided, the SEALDD server also ensures the UE’s location requirement is satisfied when establishing regular data transmission connection (e.g. by using NEF service for monitoring UE location or SEAL location service for UE entering area of interest).

NOTE 2: The established regular data transmission connection is for the VAL application, that has the requirement for data transfer via SEALDD layer. The UE entering the location area is not equivalent to the UE having data transfer requirements, it may cause the unnecessary connections to be established.

3.	If there is a special routing requirement for SEALDD user plane traffic (e.g. running on a specific slice and DNN), the SEALDD server interacts with 3GPP CN to provision service specific parameters with NEF as described in 3GPP TS 23.502 [6], clause 4.15.6.10 and clause 4.15.6.7.

If there are QoS requirements in the DD policy, the SEALDD server also applies QoS to ensure the quality for SEALDD traffic by utilizing NEF/PCF/NRM/EES service for QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4].

If the DD policy specifies failure detection report, the SEALDD server may subscribe to CN analytics (e.g. DN performance analytics) from NEF/NWDAF and further notify data delivery status of application traffic to VAL client (via SEALDD client) and VAL server based on analytics result.

4.	The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-S reference point, then SEALDD server sends SEALDD connection establishment notification to the VAL server with VAL service ID, the IP address and port.

5-6.	The SEALDD server allocates an IP address and port for sending and receiving packet over SEALDD-UU reference point, then SEALDD server sends regular data transmission connection establishment request to the SEALDD client with SEALDD flow ID, VAL service ID, the IP address and port. The request is responded by the SEALDD client. UE IP address (and port) may be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane.

NOTE 3: The SEALDD server should be aware the SEALDD client address and the connection establishment requirements for data traffic transfer before the step 5, it is not easy to obtain these informations by the SEALDD server.

NOTE 4:	Step 4 and step 5 can be done in parallel.

NOTE 5:	Step 5 can be sent via PDU session (if exist) or via application triggering (if no PDU session exists).

7.	The SEALDD client further notifies the VAL client about the SEALDD connection being established.

Upon receiving application traffic from VAL client (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies application traffic based on the VAL service ID and further sends the application traffic to VAL server. The downlink application traffic sent from VAL server to VAL client is processed similarly.