3GPP TSG-SA WG6 Meeting #61	S6-242264

Jeju, KR, 20th– 24th May 2024

Source:	Ericsson

Title:	Key Issue#1. Overall Evaluation

Spec:	3GPP TS 23.700-82 V0.4.0

Agenda Item:	8.3

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

This pCR proivides the overall evaluation of the KI#1.

2. Reason for Change

The overall evaluation of the KI#1 is required to complete the evaluation stage.

3. Conclusions

This pCR proposes the overall evaluation of the KI#1.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.4.0.

* * * First Change * * * *

11.X1	Key issue#1: Support of Architecture Enhancement and Functions for Application Layer AI/ML Services

The open issues studied in the Key Issue#1 are as follows:

- Whether and how to enhance the architecture and related functions to support application layer AI/ML services.

- Whether and how the above architecture enhancement and related functions supporting the management/execution of AI/ML lifecycle operations.

- Whether and how the architecture enhancement and related functions leveraging the existing 5GC capabilities and assistance for the application layer AI/ML services.

- Whether and how to enhance the architecture and the above related functions to support application layer AI/ML services in edge computing scenarios.

- Whether and how to provide the exposure and related functionality to assist the AIML VAL applications for its AIML service operations like service lifecycle management (e.g., assist service creation, service status, service operation).

This clause provides an overall evaluation of the key issue #1. The solutions #1, #11, #12, #13, #23, and #29 cover different aspects for the open issues in the KI#1.

Solution#1 addresses the open issue 1 by introducing the functional model of the ADAE with the ML Model Training and Management Enablement (MTME) function and the functional model of the AIML enablement service for supporting application layer AI/ML services. The corresponding content has been added to clause 7.

Solution#11 and #12 address the open issues 2 and 5 by introducing AIML service lifecycle management and AI/ML model lifecycle management, respectively. Solution#11 provides a generic procedure to enable VAL Server request to start, pause, continue, and finish an AIML service operation via the AIMLE Server. Solution#12 provides a procedure to support AI/ML model lifecycle management for ML model re-training and update if requested by the AIMLE consumer when model performance degradation is observed. Solutions#11 and #12 can be considered as candidate for normative work.

Solution#13 address the open issue 3 by introducing the interaction between the AIMLE Server with ADAES and NEF for analytics and assistance information to support FL member (re)selection. Solution#13 can be considered as candidate for normative work.

Solutons#23 and #29 address the open issue 4 by introducing enhancement of the architecture and related functions to support application layer AI/ML services in edge computing scenarios. Solution#29 provides a generic procedure to enable consumer (e.g., CAS, EAS) request assistance of edge computing process from AIMLE Server. Solution#23 introduces to deploy AIMLE Server to EDN and distributes AIML service to the Edge AIMLE Servers. The procedure provided in Solution#29 can be used by Solution#23 for collecting assistance information from AIMLE Server on the AIML service distribution process. Solutons#23 and #29 can be considered as candidate for normative work.

# *** End of Changes ***