3GPP TSG-SA WG6 Meeting #61	S6-242280

Jeju, KR, 20th – 24th May 2024

Source:	Ericsson

Title:	Solution#14. Solution Completion and Evaluation

Spec:	3GPP TS 23.700-82 V0.4.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This pCR proivides the completion and the solution evaluation for Solution #14 AI/ML policies provisioning and management.

2. Reason for Change

The Solution#14 requires the resolution of ENs and solution evaluation

3. Conclusions

This pCR proposes the resolution of ENs and solution evaluation for Solution 10.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.4.0.

* * * First Change * * * *

#### 8.14.2.1	AI/ML policies provisioning and management

Figure 8.14.2-1: AI/ML policies provisioning and management procedure

1.	The VAL server sends an AI/ML service request with policies provisioning and management information as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enablement Server performs an authorization check of the VAL server.

3.	If the VAL server is authorised, the AI/ML Enablement Server stores the AI/ML policies and applies the provisioned policies to ongoing and further the AI/ML operations (e.g., ML model training). The AI/ML application QoS adjustment policies are applied for the selected AI/ML members as per the member selection and re-selection policy after obtaining the identifiers of the AI/ML members that are selected as a result of the enforcement of the policy. The AI/ML Enablement server can use the SEALDD or the NEF services (AFSessionWithQoS) to configure the QoS. When the AI/ML members are de-selected, the QoS adjustment is reversed as they no longer participate in the AI/ML operation.

4.	AI/ML Enablement Server provides the response to the VAL server with a status via AI/ML member selection policies provisioning and management response defined in clause 8.14.3.2.

NOTE:	The service operation defined in clause 8.14.2.1 can be utilized for AI/ML member selection policies management (i.e., update/delete) functionality.

* * * Next Change* * * *

#### 8.14.3.1	AI/ML service request with policies provisioning and management information

Table 8.14.3.1-1 describes the information flow from the VAL server to the AI/ML Enablement Server as a request that contains the AI/ML policies provisioning and management information.

Table 8.14.3.1-1: AI/ML policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of member selection and re-selection polices | O (see NOTE) | Identifies the list of member selection and re-selection policies, e.g., the AI/ML member shall be re-selected when the PLR is above the given value or leaves the certain area. See details in Table 8.14.3.1-2. |
| List of AI/ML traffic QoS adjustment policies | O (see NOTE) | Identifies the list of QoS adjustment policies, e.g., the AI/ML traffic QoS shall be adjusted when the conditions are met. See details in Table 8.14.3.1-3. |
| AI/ML data transfer policy for AI/ML member re-selection | O (see NOTE) | Identifies the AI/ML data transfer policy that shall be applied for the AI/ML data transfer from de-selected to the selected AI/ML member during the re-selection procedure. See details in Table 8.14.3.1-4. |
| NOTE: At least one of these IEs shall be provided. |  |  |

NOTE 1:	Whether additional AI/ML policies are needed to be specified in the normative phase.

Table 8.14.3.1-2: Member selection and re-selection policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., select member, not select member, or re-select member. |
| List of AI/ML traffic QoS conditions | O (see NOTE) | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |
| List of AI/ML member mobility conditions | O (see NOTE) | Provides the list of mobility conditions for the AI/ML member associated with the Action, e.g., low mobility, high mobility, stationary. |
| List of AI/ML member participation configuration conditions | O (see NOTE) | Provides the list of the AI/ML member participation configuration conditions associated with the Action. The AI/ML member participation configurations provided by the AI/ML members, as in Table 8.10.3.1-1, are matched with the provided configuration conditions for member selection and re-selection. |
| List of AI/ML compute capability conditions | O (see NOTE) | Provides the list of the AI/ML compute capability conditions associated with the Action. |
| NOTE: At least one of these IEs shall be provided. |  |  |

NOTE 2:	The details of the AI/ML compute capability conditions can be specified in the normative phase.

Table 8.14.3.1-3: AI/ML traffic QoS adjustment policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., adjust AIML traffic QoS. |
| List of the required AI/ML traffic QoS values | M | Represents the required QoS values that shall be fulfilled when the conditions are met. |
| List of AI/ML traffic QoS conditions | M | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |

Table 8.14.3.1-4: AI/ML data transfer policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., initiate background AI/ML data transfer or configure a temporary AI/ML data transfer with QoS during AI/ML member re-selection procedure. |
| List of the required AI/ML traffic QoS values | O (see NOTE) | Represents the required network QoS values that shall be enforced for the AI/ML data transfer with QoS. This IE is applicable for the "configure a temporary AI/ML data transfer with QoS". |
| Background data transmission details and policy guidance | O (see NOTE) | Provides the background data transmission details and policy guidance as described in clause 14.3.2.58 of 3GPP TS 23.434 [7]. This IE is applicable for the "initiate background AI/ML data transfer" action. |
| NOTE: Only one of these information elements shall be provided. |  |  |

NOTE:	The AI/ML data transfer policies can be used for e.g., AI/ML model distribution and transfer. In detail, for example, the model transfer process mentioned in step 2 of Solution #5 can consider using AI/ML data transfer policies. The AI/ML data transfer policies can also be used by the solutions for KI #5 on in-time transfer of AI/ML models.

* * * Next Change* * * *

### 8.14.4	Architecture Impacts

This solution is based on the architecture in clause 7 with added functionality for the AIML Policy Provisioning and Management.

### 8.14.5	Corresponding APIs

This clause provides a summary on the corresponding API for solution #14.

-	AIML Policies Provisioning and Management service operations can be a part of AIMLE Client Discovery and Selection API (request/response model; API provider: AIMLE server; known consumer: VAL server and/or AIMLE Client).

### 8.14.6	Solution evaluation

This solution addresses Key Issue#7 and introduces the capability for the VAL server and/or AIMLE Client to provision and manage the AI/ML policies for AIMLE Client selection and re-selection, data transfer. This solution is ML mode agnostic (i.e., applicable for FL, DML, ML, Transfer and/or Split Learnings) and important for scenarios where the availability of AIMLE Clients changes (e.g., due to VAL UE mobility, network QoS, and/or energy status).

This solution can be considered as a candidate for normative work and doesn't introduce any dependency to exisiting 3GPP network functionality.

# *** End of Changes ***