3GPP TSG-SA WG6 Meeting #61	S6-242311

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Nokia

Title:	Terminology alignment

Spec:	3GPP TR 23.700-22 v0.2.0

Agenda item:	8.7

Document for:	Approval

Contact:	Diego Preciado (diego.preciado_rojas@nokia.com)

1. Introduction

In S6-240579 and S6-240580, the term "resource owner client" is changed to "resource owner function". Resource owner function indicated as equivalent to the resource owner user agent in IETF RFC 6749. In this study the termonology is mixed. This pCR proposes a terminology alignment.

2. Reason for Change

Terminology alignment between TS 23.222 and TR 23.700-22.

3. Proposal

It is proposed to agree the new key issue for 3GPP TR 23.700-22 V0.2.0.

* * * First Change * * * *

## 5.2	Key issue #2: Supporting Single Sign-On

### 5.2.1	Description

For the Authorization code flow in RNAA, CAPIF-8 reference point (between User Agent/ROF and Authorization Server/CCF) is used to support end-user interactions with the resource owner e.g., obtaining permission from resource owner (also known as consent) and user authentication. According to 3GPP TS 23.222 clause 6.4.16, the CAPIF-8 reference point between CCF and ROF is kept out of Release 18.

The RO is authenticating (via CAPIF-8) to CCF as part of the interaction between API invoker and authorization function (which part of CCF).

Single sign-on (SSO) is an authentication method that enables users to securely authenticate with multiple applications by using just one set of credentials. In CAPIF RNAA context, Single Sign-On (SSO) can enable an Resource Owner to use one authentication procedure to authenticate multiple API invokers to use one or more AEFs exposing resources related to the resource owner. Specifically, to implement Single Sign-On (SSO) in RNAA, a Resource Owner needs to determine in advance which API invokers can be authorized with the authentication information for Single Sign-On (SSO), and then they provide to API invokers this Single Sign-On (SSO) information. Therefore, how to manage and provide information on API invokers subject to Single sign-on (SSO) authentication for the Resource Owner should be considered.

* * * Next Change * * * *

## 5.3	Key issue #3: RNAA architecture enhancements

### 5.3.1	Description

Clause 6.2.3 of 3GPP TS 23.222 [2] specifies the high-level functional architecture for CAPIF supporting Resource owner-aware northbound API access (RNAA). The security procedures for CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3]. As per the architecture and security procedures the role and responsibilities of Resource Owner Function to support RNAA are not fully specified in Release 18.

It is required to study the role and responsibilities of Resource Owner Function and its interactions with CAPIF entities considering the requirements specified in 3GPP TS 22.261 [4] as shown below:

The 5G system shall be able to: - provide a third-party with secure access to APIs (e.g. triggered by an application that is visible to the 5G system), by authenticating and authorizing both the third-party and the UE using the third-party’s service. - provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE. - allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party. - preserve the confidentiality of the UE’s external identity (e.g. MSISDN) against the third-party. - provide a third-party with information to identify networks and APIs on those networks.

### 5.3.2	Open issues

Open issues to study:

1.	Further enhancements to CAPIF architecture considering Resource Owner Function functionalities and its interactions with CAPIF entities (e.g., CAPIF-8 related interactions).

NOTE:	The security aspects for the architecture enhancements need to be coordinated with SA3.

* * * End of Changes * * *