3GPP TSG-SA6 Meeting #61	S6-242341

# Jeju, Korea (Republic Of), 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0120 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE to UE single hop relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of Rel-18 3GPP SA2 completed the architecture work to support UE-to-UE single hope relay. This CR proposes the needed changes to the mission critical architecture to enable MCx over single hop relay for off network UEs |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are to add off network service requirements and to add a new functional refernce model for MC UE-to-UE single hop relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Single hop relay will not be supported, and requirements not fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new), 5.3.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Version 1 – Updated to include service requirements and new functional architecture Initial CR |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.x		5G ProSe UE-to-UE relay service requirements

### 4.X.1	General

The MC service shall support the capabilities for 5G ProSe UE-to-UE single hop relay. For this matter, 5G ProSe Layer-2 and 5G ProSe Layer-3 UE-to-UE relaying techniques can be utilized, as described in 3GPP TS 23.304 [17].

A 5G ProSe UE-to-UE relay supporting MC service UE provides means of connectivity and relaying of MC traffic from a MC service UE to another MC service UE(s) via a MC UE Relay. For this matter, the 5G ProSe UE-to-UE Relay Discovery service allows the MC service remote UE to discover a potential UE-to-UE relay supporting MC service in its proximity as described in 3GPP TS 23.304 [17]. Upon its discovery, the 5G ProSe Direct UE-to-UE Relay Communication functionality is utilized to achieve communication to provide the MC service between MC UEs, relaying MC traffic via the UE-to-UE relay UE over the NR PC5 reference point.

### 4.X.2	5G ProSe UE-to-UE relay service requirements

In order to enable 5G ProSe UE-to-UE relaying capabilities – whether based on Layer-3 or Layer-2 UE-to-UE relaying techniques, the MC system provides the appropriate parameters and configurations to the MC service UE(s).

As defined in 3GPP TS 23.304 [17], among these parameters are: Relay Service Code(s) (RSCs) which can be associated to a certain MC service group, User Info, ProSe Layer-2 Group ID and ProSe Group IP multicast address. Moreover, the MC service group ID is resolved to the ProSe Layer-2 Group ID and ProSe Group IP multicast address, which are utilized within the 5G ProSe Relay Discovery and 5G ProSe Direct Communication procedures, as described in 3GPP TS 23.304 [17]. Furthermore, the RSCs are utilized to restrict the necessary UE-to-UE relay service and related procedures within members of a certain MC service group.

* * * Second Change * * * *

### 5.3.5	Off-network single hop relay functional model

Figure 5.3.5-1 shows the functional model for off-network operation.

Figure 5.3.5-1: Functional model for MC service off-network single hop relay operation

For a specific MC service, the description of off-network operation is contained in the corresponding MC service TS.

Editors Note: Additional work is needed to finalize the information flows and information elements to support this functionality.

* * * End Changes * * * *