3GPP TSG-SA WG6 Meeting #61	S6-242404

Jeju, Republic of Korea, 20th – 24th May 2024	(revision of S6-242029)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0058 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Service Continuity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are duplicate name: Clause 8.9.2.1.4 name is “PIN Service Continuity Update procedure” Clause 8.9.2.1.6.2.3 name is “PIN service continuity update” Clause 8.9.2.1.6 name is “PIN Service Continuity subscription”, Clause 8.9.2.1.6.2.3 “PIN service continuity update” is used for PIN Service Continuity subscription update. Another examples like 8.5.9.2.3 PIN status update, 8.5.14.2.3 PIN connectivity update, 8.7.2.1.4.2.3 PIN service switch update, all for subscription update. But in clause 8.9.2.1.4 “PIN Service Continuity Update procedure”, step 1 is PIN Configuration Service Continuity Update request, step 5 is PIN Configuration Service Continuity Update response. So the name of clause 8.9.2.1.4 is wrong, which should be “PIN Configuration Service Continuity Update procedure”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix clause 8.9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bad TS quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2.1.1, 8.9.2.1.2, 8.9.2.1.4, 8.9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.9.2.1.1	General

Following procedures are supported for service continuity in a PEGC relocation scenario:

-	PIN Management PEGC Service Continuity procedure;

-	PIN Management PEGC Configuration procedure;

-	PIN Configuration Service Continuity Update procedure;

-	PIN Management PEGC Discovery procedure;

-	PIN Service Continuity subscription procedure.

* * * Start of Change 2 * * *

8.9.2.1.2	PIN Management PEGC Service Continuity procedure

Pre-conditions:

1.	PIN Element has an application session that goes through a PIN Gateway Client; and

2.	PIN Gateway Client needs to be replaced (e.g., PEGC will become unavailable or PEGC is unreachable).

Figure 8.9.2.1.2-1: PIN PEGC Service Continuity procedure

1.	The PIN Gateway Client or the PIN Client sends a PIN Management PEGC Service Continuity request to the PIN Management Client for a specific service. The PIN Management PEGC Service Continuity request includes security credentials, PIN Client identifier(s), PIN Gateway identifier, and Service identifier.

2	Upon receiving the request, the PIN Management Client validates the request and checks if the request is authorized for the given PIN Client(s), PIN Gateway Client and service. If the request is authorized and valid, the PIN Management Client determines if a target PIN Gateway Client meets the requirements for handling the application traffic for the provided service.

If the PIN Management Client identifes a target PIN Gateway Client that meets the requirements, the PIN Management Client sends PIN Management PEGC Configuration request to the target PIN Gateway Client to configure the target PIN Gateway for handling service traffic from the PIN Client(s) as in clause 8.9.2.1.3. If the service is provided by an Application Server, the PIN Management Client send a PIN Configuration Service Continuity Update request to the PIN Server if needed to inform of the service continuity changes in the PIN as in clause 8.9.2.1.4.

3.	If the procesing of the request was successful, the PIN Management Client sends a PIN Management PEGC Service Continuity response to indicate that the request processing was successfull and includes target PIN Gateway Client information. Otherwise, the PIN Management Client sends a PIN PEGC Service Continuity response indicating that the request processing failed and can include appropriate reasons.

Upon receiving the PIN Management PEGC Service Continuity response, if the response was received by the PIN Client, the PIN Client validates if the request was successful and can inform the application client of the target PIN Gateway Client provided in the response to establish connectivity. If the response was received by the PIN Gateway Client, the PIN Gateway Client notifies each PIN Client included in the PIN Management PEGC Service Continuity request about the target PIN Gateway Client change as in clause 8.9.2.1.5.

* * * Start of Change 3 * * *

8.9.2.1.4	PIN Configuration Service Continuity Update procedure

The PIN Configuration Service Continuity Update procedure allows the PEMC to inform the PIN Server about a service continuity request received at the PEMC for a PINE, and it allows the PIN Server to inform the related AS of the same if such functionality is supported by the AS.

The PIN Server may provide, in the response, policy information to the PEMC about service continuity for the concerned PINE.

Pre-conditions:

1.	PINE has an application session that goes through a PEGC; and

2.	PEGC needs to be replaced (e.g., PEGC will become unavailable or PEGC is unreachable).

3.	The AS is successfully subscribed for PIN service continuity notifications with the PIN Server.

Figure 8.9.2.1.4-1: PIN Configuration Service Continuity Update procedure

1.	The PEMC sends a PIN Configuration Service Continuity Update request to the PIN Server. The PIN Configuration Service Continuity Update request includes the requestor identifier, security credentials, PINE identifier(s), source and target PIN Gateway identifiers, and service identifier (e.g., application client identifier, application server identifier, application session identifier, application session identifier), and the PIN ID.

2.	Upon receiving the request, the PIN Server validates the request and checks if the PEMC is authorized to request service continuity update. If the request is authorized and valid, if the PIN Server has not authorized the PINE for service continuity, the PIN Server validates if the PINE is authorized and determines policy information about service continuity for the PINE. If the PIN Server has already authorized the PINE for service continuity, the PIN Server updates the configuration for the PINE and service indicated in the request.

3.	If the PIN Server needs to update the AS with service continuity information, the PIN Server sends a PIN Service Continuity notification to the AS. The PIN Service Continuity notification includes PIN identifier, PINE identifier(s), PEGC identifier, service identifier and may include an application session identifier and terminating endpoints on the PINE and PEGC.

4. 	Upon receiving the notification, the AS validates the notification. If the notification is valid, the AS updates the service continuity configuration related to the PINE and service indicated in the notification.

5.	If the processing of the PIN Configuration Service Continuity Update request was successful, the PIN Server sends a PIN Configuration Service Continuity Update response to indicate that the request processing was successful and may include policy information about service continuity for the PINE. Otherwise, the PIN Server sends a PIN Configuration Service Continuity Update response indicating that the request processing failed and can include appropriate reasons.

Upon receiving the PIN Configuration Service Continuity Update response, the PEMC uses the information provided in the response to inform the PINE(s) or the PEGC from which the service continuity request originated as in clause 8.9.2.1.2.

* * * Start of Change 4 * * *

8.9.3.1	General

The following information flows are specified for Service Continuity:

-	PIN Management PEGC Service Continuity request and response

-	PIN Management PEGC Configuration request and response

-	PIN Configuration Service Continuity Update request and response

-	PIN Management PEGC Discovery request and response

-	PIN service continuity subscribe

-	PIN service continuity notify

-	PIN service continuity update

-	PIN service continuity unsubscribe

* * * End of Change * * *