3GPP TSG-SA WG6 Meeting #61	S6-242528

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242315)

Source:	KPN N.V.

Title:	Solution #10 and #14 Update

Spec:	3GPP TR 23.700-82 v0.4.0

Agenda item:	8.3

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl), Anthony Pagès (anthony.pages@tno.nl)

1. Introduction

This contribution updates the list of AI/ML member participation configuration conditions in Table 8.14.3.1-2 by introducing a slice type as an alternative to the VAL service ID for AI/ML member participation configuration provisioning, as well as for AI/ML member selection and reselection.

2. Reason for Change

This contribution enables the utilization of service types (such as slice type) as an alternative to the VAL Service ID for AI/ML member participation configuration provisioning by the AI/ML member. Additionally, it enhances clarity in Table 8.14.3.1-2 (Member selection and re-selection policy) by explicitly outlining the AI/ML member participation configuration conditions in a dedicated table, rather than solely referencing Table 8.10.3.1-1.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.4.0.

* * * First Change * * * *

## 8.14	Solution #14: AI/ML policies provisioning and management

### 8.14.1	General

The following clauses specify procedures, information flows and APIs for Key issue#7 to provision and manage the AI/ML policies for the AI/ML Enablement services.

The AI/ML data transfer policies allow the VAL server to provide requirements for the AI/ML data (e.g., the current status of the trained ML model) from the de-selected to the selected client. The VAL server may have a network QoS requirements for AI/ML data transfer to use Background Data Transfer to save cost for the AI/ML data transfer.

The AI/ML data transfer policies can be used for both time-sensitive and non-time-sensitive AI/ML data transfer. For example:

- VAL server performs the time-sensitive learning that should be completed as soon as possible. The VAL server can request the AI/ML Enablement server to configure a temporary AI/ML data transfer session with QoS during AI/ML member re-selection procedure, in order to transfer the current trained data within the minimal time.

- The VAL server performs the non-time-sensitive learning and would like to reduce the cost of the AI/ML operation. The VAL server can request the AI/ML Enablement server to use background data transfer with lowest costs as BDT policy guidance.

### 8.14.2	Procedures

#### 8.14.2.1	AI/ML policies provisioning and management

Figure 8.14.2-1: AI/ML policies provisioning and management procedure

1.	The VAL server sends an AI/ML service request with policies provisioning and management information as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enablement Server performs an authorization check of the VAL server.

3.	If the VAL server is authorised, the AI/ML Enablement Server stores the AI/ML policies and applies the provisioned policies to ongoing and further the AI/ML operations (e.g., ML model training). The AI/ML application QoS adjustment policies are applied for the selected AI/ML members as per the member selection and re-selection policy after obtaining the identifiers of the AI/ML members that are selected as a result of the enforcement of the policy. The AI/ML Enablement server can use the SEALDD or the NEF services (AFSessionWithQoS) to configure the QoS. When the AI/ML members are de-selected, the QoS adjustment is reversed as they no longer participate in the AI/ML operation.

4.	AI/ML Enablement Server provides the response to the VAL server with a status via AI/ML member selection policies provisioning and management response defined in clause 8.14.3.2.

NOTE:	The service operation defined in clause 8.14.2.1 can be utilized for AI/ML member selection policies management (i.e., update/delete) functionality.

Editor's Note:	The related service operation request (e.g., member selection) is to be updated in the evaluation stage.

### 8.14.3	Information flows

#### 8.14.3.1	AI/ML service request with policies provisioning and management information

Table 8.14.3.1-1 describes the information flow from the VAL server to the AI/ML Enablement Server as a request that contains the AI/ML policies provisioning and management information.

Table 8.14.3.1-1: AI/ML policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of member selection and re-selection polices | O (see NOTE) | Identifies the list of member selection and re-selection policies, e.g., the AI/ML member shall be re-selected when the PLR is above the given value or leaves the certain area. See details in Table 8.14.3.1-2. |
| List of AI/ML traffic QoS adjustment policies | O (see NOTE) | Identifies the list of QoS adjustment policies, e.g., the AI/ML traffic QoS shall be adjusted when the conditions are met. See details in Table 8.14.3.1-3. |
| AI/ML data transfer policy for AI/ML member re-selection | O (see NOTE) | Identifies the AI/ML data transfer policy that shall be applied for the AI/ML data transfer from de-selected to the selected AI/ML member during the re-selection procedure. See details in Table 8.14.3.1-4. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Editor's note: Whether additional AI/ML policies are needed is FFS.

Table 8.14.3.1-2: Member selection and re-selection policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., select member, not select member, or re-select member. |
| List of AI/ML traffic QoS conditions | O (see NOTE) | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |
| List of AI/ML member mobility conditions | O (see NOTE) | Provides the list of mobility conditions for the AI/ML member associated with the Action, e.g., low mobility, high mobility, stationary. |
| List of AI/ML member participation conditions | O (see NOTE) | Provides the list of the AI/ML member participation conditions associated with the Action. The AI/ML member participation configurations provided by the AI/ML members, as in Table 8.10.3.1-1, are matched with the provided conditions detailed in Table 8.14.3.1-5 for member selection and re-selection. |
| List of AI/ML compute capability conditions | O (see NOTE) | Provides the list of the AI/ML compute capability conditions associated with the Action. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Editor's note: The AI/ML compute capability conditions are FFS.

Table 8.14.3.1-3: AI/ML traffic QoS adjustment policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., adjust AIML traffic QoS. |
| List of the required AI/ML traffic QoS values | M | Represents the required QoS values that shall be fulfilled when the conditions are met. |
| List of AI/ML traffic QoS conditions | M | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |

Table 8.14.3.1-4: AI/ML data transfer policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., initiate background AI/ML data transfer or configure a temporary AI/ML data transfer with QoS during AI/ML member re-selection procedure. |
| List of the required AI/ML traffic QoS values | O (see NOTE) | Represents the required network QoS values that shall be enforced for the AI/ML data transfer with QoS. This IE is applicable for the "configure a temporary AI/ML data transfer with QoS". |
| Background data transmission details and policy guidance | O (see NOTE) | Provides the background data transmission details and policy guidance as described in clause 14.3.2.58 of 3GPP TS 23.434 [7]. This IE is applicable for the "initiate background AI/ML data transfer" action. |
| NOTE: Only one of these information elements shall be provided. |  |  |

NOTE:	The AI/ML data transfer policies can be used for e.g., AI/ML model distribution and transfer. In detail, for example, the model transfer process mentioned in step 2 of Solution #5 can consider using AI/ML data transfer policies. The AI/ML data transfer policies can also be used by the solutions for KI #5 on in-time transfer of AI/ML models.

Table 8.14.3.1-5: List of AI/ML member participation conditions

| Information element | Status | Description |
| --- | --- | --- |
| List of conditions | M | List of conditions for AI/MLmember selection and re-selection. |
| > VAL service ID | O | The VAL service ID required to be supported by the AI/ML member. |
| > List of time-schedule conditions | O | Provides the required list of time-schedule conditions, e.g., the AI/ML member is required to be available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |
| > List of location-based conditions | O | Provides the required list of location-based conditions, e.g., the AI/ML member is required to be available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| > Mode of member participation | O | Indicates the required AI/ML member availability for the participation based on the modes like training (e.g., FL, DML, ML, and/or Split Learning), aggregator, coordinator and/or inference. |
| > Service exposure level | O | Indicates required level of service exposed by the AI/ML member for the corresponding VAL service ID e.g., premium resource usage, limited resource usage, raw data, processed data. |

#### 8.14.3.2	AI/ML service response with policies provisioning and management information

Table 8.14.3.2-1 describes the information flow from the AI/ML Enablement Server to the VAL server as a response that contains the AI/ML policies provisioning and management result.

Table 8.14.3.2-1: AI/ML member selection policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * End of Changes * * * *