3GPP TSG-SA WG6 Meeting #61	S6-242559

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242293)

Source:	Samsung

Title:	Pseudo-CR for solution to KI#3

Spec:	3GPP 23.700-01 v0.3.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This contribution proposes solution for KI#3.

2. Reason for Change

KI#3 is agreed with following open issues:

To support the discontinuous satellite coverage in application enabler, the following aspects need to be studied:

-	Whether and how the information on discontinuous satellite coverage needs to be exposed to the application layer (e.g. semantics, service-based interfaces, etc.) on the AS/AF side.

-	Whether and how the application enabler enforces policies (e.g., based on the time of the day, the UE location, etc.) on the discontinuous coverage information exposure towards the vertical applications.

Existing solution in clause 7.2.3.1.1 is a procedure for procedure for UE providing UE unavailability period information to AF. However, there are scenarios Satellite coverage availability information (SCAI) is not yet configured on the UE or not supported by the UE. The proposed solution is a procedure for AS handling UE unavailability period information taking support of 5GC to determine the unavailability of UE.

Excerpt from TS 23.501 clause 5.4.1.4	Support of Unavailability Period:

7)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF at the start of Unavailability Period considers the remaining time in the Unavailability Period (if available) when constructing the "Loss of Connectivity" event report towards the NEF and the remaining time in the Unavailability Period is reported to the respective subscribed AF.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-01 v0.3.0.

* * * First Change * * * *

##### 7.2.3.1.2	Procedure for AS handling UE unavailability period information

Pre-condition:

-	Satellite coverage availability information (SCAI) is not yet configured on the UE or not supported by the UE.

-	5GC is aware of SCAI and UE location.

Figure 7.2.3.1.2-1: Procedure for AS handling UE unavailability period information

1.	UE does not support SCAI provisioning or UE has not received SCAI configuration from the 5GC. Hence, cannot provide UE unavailability period to the AS for operating gracefully under a discontinuous coverage pattern.

2.	AS (e.g. SEAL server) sends get UE unavailability request to the 5GC, including the UE/User identity and service identity for which UE has satellite access. AS can subscribe with 5GC to loss of coverage event subscription to get notified about the unavailability period or unavailability duration when certain events occur e.g. start of unavailability period or before certain duration in a particular location, as described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

3.	5GC (e.g. NEF) checks if UE supports satellite access and then determines UE unavailability e.g. based on SCAI information and UE location.

4.	5GC sends get UE unavailability response including the UE unavailability information to the AS. Or when an event is triggered in the 5GC corresponding to the subscription in step 2, 5GC notifies to AS the unavailability period information e.g. duration/start time, as described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

5.	AS (e.g. SEAL server) sends the unavailability period information to the AS (e.g. VAL server), if required.

6.	AS (e.g. SEAL server) uses the UE unavailability information for adjusting the application behaviour e.g. to trigger the downlink data delivery, bulky data exchanges can be deferred and planned during longer connectivity intervals, giving precedence to data suiting short connectivity intervals.

* * * End of Change * * * *