3GPP TSG-SA WG6 Meeting #61	S6-242582

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242102)

Source:	Huawei, Hisilicon

Title:	Solution on Satellite edge computing EES onboard

Spec:	3GPP TR 23.700-01 v0.3.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes a solution on Satellite edge computing EES onboard.

2. Reason for Change

In Key issue#2, EAS and EES are deployed in the same MEO or LEO satellite, in such case, the UE need to perioricly interact with ECS and EES for EES and EAS information, which causes the frequent interruption.

In time#1, the SAT#1's service area can cover the UE, thus the UE can conncet to the EES#1. However, in time#2,the SAT#1 moved which cannot cover UE#1 thus the UE cannot obtain the service from EES#1, then the service interuption may happen. As this time the UE need to discover a new EES which its service area covers the UE due to the UE cannot obtain service from EES#1 at this time.

Using currently mechnism the UE need to periodically discover the new EES and new EAS when the old EES and old EAS' Satellite moved out of UE' location which cause the periodically servce interruption.

With this solution, the UE can determine the EES and EAS based on the UE (predicted/expected) location, Prediction expiration time, EES Satellite ID and EES ephemeris information, and the UE does not need to send the service provisioning again obtain the list of EES which is more effiency compared to current mechanism.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v0.3.0.

* * * First Change * * * *

7.X.1	Solution #X: Satellite edge computing considering EAS onboard

7.X.1.1	Solution description

7.X.1.1.1	General

Figure 7.2.1.1.1-1: EAS and EES onboard satellite deployment

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. EASs and their registered EES are deployed on one or more satellites

As for the EAS and EES on MEO or LEO satellites however the ECS is on the ground, in the service provisioning procedure, the ECS can determine a list of EES based on the UE (predicted/expected) location, Prediction expiration time, Satellite ID and EES ephemeris information and ECS provide list of EES information in the service provisioning response message. Upon receiving a list of EES information, the EEC can dertermine the EAS based on the UE (predicted/expected) location, Prediction expiration time, EAS Satellite ID and EAS ephemeris information, then the UE may sequentially connect to corresponding EESs based on ephemeris information of the EESs. When an EES is about to leave the UE’s location, the UE may connect to a next EES in advance to obtain an edge computing service, and does not need to perform EES discovery again. In doing so, the service interruption caused by periodic EES discovery can be avoided.

Furthermore, the EES can indicate the ECS whether the EAS and EES are deployed in the same Satellite, if the EES and EAS are deployed in the same satellite, then the ECS can provide EAS information (EAS endpoint and EAS ephemeris information) along with the EES information. Upone receiving the EAS information, the UE can connect to the EAS and the EAS discovery procedure does not need to be performed.

In addition, when there is only one SAT which provide EES and EAS, then the EEC can connect to the EES when EES related Satellite service area covers UE location, similary the UE can connect to the EAS when EAS related Satellite service area covers UE location.

* * * NEXT Change * * * *

7.X.1.2	Corresponding APIs

7.X.1.2.1 	EES Profile enhancement for EES onboard

According to clause 7.X.1.2 service provisioning for EES onboard satellite the following change is needed in TS 23.558.

- TS 23.558 Clause 8.2.6 EES Profile EES mobility on board LEO or MEO Satellite

TS 23.558 Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| … | … | … |
| SAT ID | O | The Satellite ID corresponding to the EES. |
| > ephemeris information | O | The ephemeris information of the satellite corresponding to the EES |
| > codeployed EAS information | O | Indicates that the EAS is co-deployed with the EES in same Satellite. |
| >> EAS endpoint |  | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information is provided to the EEC to connect to the EAS |
| … NOTE 2: The assignment of the trajectory ID for the EES can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculed for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |