3GPP TSG-SA WG6 Meeting #61	S6-242294

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Samsung

Title:	Pseudo-CR for solution to KI#7

Spec:	3GPP 23.700-01 v0.3.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This contribution proposes solution for KI#7.

2. Reason for Change

KI#7 is agreed with following open issues:

To support utilization of S&F events information by application enablers, the following aspects need to be studied:

-	What are the S&F events information that can be utilized by the application enablers?

-	How to use the S&F events information to optimise the application enablement behaviour?

-	Whether and how the S&F Satellite operation information can be exposed to the 3rd party/VAL server.

The proposed solution is about leveraging S&F events information at the application enablement for the optimized service delivery. In order to leverage S&F events information and handle S&F data delivery at AF, the AF needs to subscribe for S&F information from 5GC and UE needs to register for S&F mode of operation.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-01 v0.3.0.

* * * First Change * * * *

### 7.2.x	Solution #x: Leveraging S&F events information at application enablement

#### 7.2.x.1	Solution description

This solution maps to KI#7. This solution is about leveraging S&F events information at the application enablement for the optimized service delivery. In this scenario, AS (e.g. SEAL server) is considered to be deployed on the ground.

##### 7.2.x.1.1	AS subscribing to S&F events information with 5GC

Figure 7.2.x.1.1-1: Procedure for AS subscribing to S&F events information with 5GC

1.	AS (e.g. SEAL server) sends subscribe S&F information request to 5GC (e.g. NEF) e.g. for getting feeder link and service link availability information, expected data delivery time.

NOTE:	The request may include UE information if AS (e.g. SEAL server) is subscribing to events related to a specific UE.

2.	5GC then authenticates and authorizes the AS requesting to receive S&F information.

3.	5GC sends success or failure response to the AS related to the request made in step 1. If S&F information is available then the success response may also include the S&F information e.g. for getting feeder link and service link availability information, expected data delivery time.

4.	5GC monitors the occurrence of any event related to S&F.

5.	Upon occurrence of any event related to S&F, 5GC sends S&F information e.g. for getting feeder link and service link availability information, expected data delivery time towards AS.

##### 7.2.x.1.2	UE registration to S&F operation mode

Figure 7.2.x.1.2-1: Procedure for UE registration to S&F operation mode

1.	UE sends register S&F mode request to AS (e.g. SEAL server) including the user and service identity information.

NOTE:	AS (e.g. SEAL server) may subsequently register S&F mode request from a UE to 5GC.

2.	AS (e.g. SEAL server) sends register S&F mode response to UE including success or failure result. If successful, the response may include S&F information e.g. scheduling, supported QoS, supported data size.

3.	UE then stores and leverages S&F information (e.g. scheduling, supported QoS, supported data size) received from AS to optimize application/service behavior e.g. adjust the frequency/timing of sending data, adjust the data size.

##### 7.2.x.1.3	S&F data delivery handling at AS

Pre-condition: AS (SEAL server) has subscription to S&F information with 5GC as described in 7.2.x.1.1

Figure 7.2.x.1.3-1: Procedure for S&F data delivery handling at AS (e.g. SEAL server)

1.	 VAL server sends send data request to AS (e.g. SEAL server) including the target UE/user and service identity information.

2.	AS (e.g. SEAL server) checks if UE has registered for S&F operation mode and may store the received data request. AS (e.g. SEAL server) may optimize application/service behavior e.g. adjust the frequency/timing of sending data, adjust the data size.

3.	AS (e.g. SEAL server) provides send data response to VAL sever including the estimated delivery time information.

NOTE:	AS (e.g. SEAL server) may send an acknowledgement to the VAL server after the data delivery request at step 1 is eventually delivered to the target UE/user.

#### 7.2.x.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts (if any) of the solution and possible new SA6 capabilities and interfaces.

#### 7.2.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *