3GPP TSG-SA WG6 Meeting #61	S6-242595

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242229)

Source:	China Mobile

Title:	pCR on update of solution#6

Spec:	3GPP TR 23.700-92 v0.5.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR updates the Solution#6 in 3GPP TR 23.700-92 v0.5.0. Some technical errors existing in this solution, e.g. when the eMMTel Enabler establishing the 3PCC, it does not know the media information of both caller and callee. These technical errors are needed to be solved. This pCR will also fix some editorials.

2. Reason for Change

updates the Solution#6 in 3GPP TR 23.700-92 v0.5.0.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.5.0.

* * * First Change * * * *

## 8.6	Solution #6: Third-Party Call service

### 8.6.1	Description

This solution resolves Key Issue #1 about Third-Party Call service via eMMTel Enabler Server.

This solution uses the architecture of eMMTel Enabler as specified clause 8.1as the basis and has no requirement to enhance the architecture.

IMS capability exposure is being discussed in SA2, IMS Core network exposures create/update/release an IMS session with data channel capability via N33/DC4/MDC2/MDC3. In this procedure, the eMMTel Enabler Server provides Third-Party Call service from service granularity which use SA2’s atomic interface to provide high-level call service APIs to hide the complexity of underlying the IMS core capabilities, the eMMTel Enabler Server handles all the necessary API requests/responses related to the session creation on behalf of the application without further interaction with the application until the IMS session with Data Channel media is established successfully or failed. the eMMTel Enabler Server acts as a middle layer which can provide some other value-added services (e.g. provides the virtual number service) between Third-Party DC AS and IMS Core network as specified in 3GPP TR 23.700-77 [19].

### 8.6.2	Procedures

#### 8.6.2.1	Service Enabler Description

Figure 8.6.2.1-1: Reference Architecture for the Nemes Services

The eMMTel Enabler Server provides the following services to the application:

-	Control operations of Third-Party Call with data channel media, such as initiating, modifying, terminating.

-	Subscribe and unsubscribe to Third-Party Call events of the specific subscriber, and

-	May include some other value-added services, e.g. virtual number.

8.6.2.2	Establishing a Third-Party Call

Figure 8.6.2.2-1 below illustrates how to establish a Third-Party Call between two users. the eMMTel Enabler Server acts as a middle layer between the Third-Party Call application and IMS Core network, according to the session creation API invoked by the eMMTel Enabler Server, the IMS network initiates an IMS session with data channel media.

Figure 8.6.2.2-1: Establishing a Third-Party Call

1.	The Third-Party Call application invokes the session creation API of the eMMTel Enabler Server to create a Call which may include data channel media. The request includes information elements as specified in Table 8.6.2.2-1.

Table 8.6.2.2-1: Information elements in call request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID | M | The identifier of the caller. |
| Terminating ID | M | The identifier of the callee. |
| Media information | M | The Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call |
| Application Profile requested | O | The DC application profile expected to be used in this call |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

2.	The eMMTel Enabler Server verifies that the sender is authorized to establish a Third-Party Call.

3.	The eMMTel Enabler Server invokes the API provided by IMS Core network to initiate a call between the UE A and UE B and hides the detailed interactions in the underlying network. The detail prodecure as specified in clause 6.6 of 3GPP TR 23.700-77 [19].

NOTE:	This procedure also allows the eMMTel Enabler Server to establish a Third-Party Call between a DCMTSI Client and an Application without IMS capability. In this case, the Application without IMS capability and the eMMTel Enabler Server serves this Application acts as a UE in the procedure specified in clause 6.6 of 3GPP TR 23.700-77 [19].

Editor's note 1:	The SA6 specific interfaces and the elements of the interface depend on the ongoing study of SA2 FS_NG_RTC_Ph2.

4.	The eMMTel Enabler Server sends a corresponding call session establishment response, e.g the call is established successfully or not, to the sender.

Table 8.6.2.1-2: Information elements in call response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the session. |
| Session ID (see NOTE 1) | O | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Reason (see NOTE 2) | O | Indicates the reason for failure.This IE only be present when the Registration result is failure. |
| NOTE 1: This IE shall only be present when the Set virtual number is Success. NOTE 2: This IE shall only be present when the Set virtual number is Failure. |  |  |

5.	UE A and UE B begin to talk with each other with interaction over data channel.

8.6.2.3	Terminating a Third-Party Call

Figure 8.6.2.3-1 below illustrate how to terminate a Third-Party Call between two users. The terminating procedure in Figure 8.6.2.3-1 is triggered by the Third-Party Call application invoking a release session API provided by the eMMTel Enabler Server.

Figure 8.6.2.3-1: Terminating a Third-Party Call

1.	The 3rd Party call application sends a release request to the eMMTel Enabler Server. The request message includes information elements as specified in Table 8.6.2.3-1.

Table 8.6.2.3-1: Information elements in Call session release request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Call Release Cause | O | The reason for call Release |

2.	The eMMTel Enabler Server verifies that the sender is authorized to release a Third-Party Call.

3.	The eMMTel Enabler Server invokes the API provided by IMS Core network to release the call to the UE A and UE B. The detail procedure as specified in clause 6.6 of 3GPP TR 23.700-77 [19]

Editor's note:	The SA6 specific interfaces and the elements of the interface depend on the ongoing study of SA2 FS_NG_RTC_Ph2.

4.	The eMMTel Enabler Server return a release response to the Third-Party Call application.

8.6.2.4	Modifying a Third-Party Call without data channel media to add data channel media

Figure 8.6.2.4-1 below illustrates how to modify an established Call without data channel media between two users to add data channel media. It’s assumed that a Call with audio and video media has established between two users.

Figure 8.6.2.4-1: Modifying a Call to add data channel media

1.	The 3rd Party call application invokes the session modification API of the eMMTel Enabler Server to add data channel media into the specific session.

2.	The eMMTel Enabler Server verifies that the sender is authorized to modify a Third-Party Call.

3.	The eMMTel Enabler Server invokes the API provided by IMS Core network to modify the call to add Data Channel media between the UE A and UE B.

Editor's note:	The SA6 specific interfaces and the elements of the interface depend on the ongoing study of SA2 FS_NG_RTC_Ph2.

4.	The eMMTel Enabler Server sends a corresponding call session modification response to the sender.

5.	UE A and UE B begin to talk with each other with interaction over data channel.

### 8.6.3	Impacts on Existing Nodes and Functionality

In this solution, the procedures use the basic capability exposured by IMS core which is under studied by SA2.