3GPP TSG-SA WG6 Meeting #61	S6-242673

Jeju, KR, 20th – 24th May 2024

Source:	Ericsson, Samsung

Title:	Adaptive reporting configuration for location services

Spec:	3GPP TS 23.700-72 V0.5.0

Agenda item:	8.1

Document for:	Approval

1. Introduction

The location reporting configuration requires the application to provide the details about the reporting types and characteristics; however, it is hard for a regular application to determine the detailed reporting characteristics due to lack of information about the UE moving statistic and predictions that may cause non-effective network usage and report generation (e.g., too many reports that are not needed or too rare reports).

2. Reason for Change

The typical reporting configuration for location services includes the reporting type (e.g., periodic or event-triggered), the reporting periodicity for the periodic reporting and trigging events for the event triggered reporting.

The typical reporting configuration require the application to foresee the required situations that are possible for the UE (e.g., the collision detection for UAS or V2X) to configure the location reporting. The application may configure too frequent periodic notification that will overflow the network with the reports or configure big periodicity for the location that leads for low accuracy. Thus, it is beneficial for the application to ask the SEAL server to select the reporting configuration based on VAL service ID, VAL server ID, UE moving statistic and prediction.

3. Conclusions

This pCR proposes a solution to address the adaptive reporting configuration for location services.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-72 V0.5.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |
| Sol #2 | X |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |
| Sol #5 |  |  |  | X |  |  |
| Sol #6 |  |  | X |  |  |  |
| Sol #7 |  |  |  |  | X |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #X |  |  |  | X |  |  |

* * * Next Change (all texts are new) * * * *

## 6.x	Solution #X: Adaptive Location Configuration and Reporting based on UE Application and Movement statistic and prediction

### 6.x.1	Architecture Impacts

This solution is based on architecture of SEAL as described in 3GPP TS 23.434 [23434].

### 6.x.2	Solution description

The typical reporting configuration for location services includes the reporting type (e.g., periodic or event-triggered), the reporting periodicy for the periodic reporting and trigging events for the event triggered reporting.

The typical reporing configuration require the application to forsee the required situations that are possible for the UE (e.g., the collision detection for UAS or V2X) to configure the location reporting. The application may configure too frequent periodic notification that will overflow the network with the reports or configure big periodicy for the location that leads for low accuracy. Thus, it is befefitial for the application to ask the SEAL server to select the reporting configuration based on VAL service ID, VAL server ID, UE moving statistic and prediction.

In this solution, the following definition is uses:

-	Adaptive reporting configuration is the reporting configuration provided by SEAL LM server based on VAL service ID, VAL server ID, and/or UE Moving statistic and prediction.

This solution improves the SEAL LM service defined in 3GPP TS 23.434 [23434], i.e.:

-	Introduction of Adaptive reporting configuration type that allows the SEAL LM server to configure the reporing based on VAL service ID, VAL server ID, and/or UE Moving statistic and prediction.

This solution addresses KI#4 to support:

-	Whether and how to expose the location data information per the e.g. time/area/space granularity in application enabled layer.

#### 6.x.2.2	Impact to existing SEAL location management procedures

The SEAL procedures and information flows in 3GPP TS 23.434 [23434] can be enhanced (highlighted in bold italics) as follows.

#### 9.3.2.4 Location reporting trigger Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure. Table 9.3.2.4-1: Location reporting trigger Information element; Status; Description. Requestor Identity; M (see NOTE 1); Identity of the requesting authorized VAL user or VAL UE or VAL server. Identity; M (see NOTE 1); Identity of the requested VAL user or VAL UE. VAL service ID; O; Identity of the VAL service for which the location reporting trigger is set.. Immediate Report Indicator; O (see NOTE 2); Indicates whether an immediate location report is required. Requested location information; O (see NOTE 2); Identifies what location information is requested. This information element may be represented as VAL service area ID(s) when this information flow from VAL server to the location management server.. Triggering criteria; O (see NOTE 2); Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE as specified in table 9.3.2.4-2.. Minimum time between consecutive reports; O see (NOTE 2); Defaults to 0 if absent otherwise indicates the interval time between consecutive reports. Endpoint information; O; Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required.. NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present.. Table 9.3.2.4-2: Location reporting Triggering criteria Information element; Status; Description. Frequency of reporting; O; It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered.. Adaptive reporting; O; It indicates the request for an adaptive location reporting.. Reporting periodicity; O; If the Frequency of reporting is periodic, the reporting periodicity shall be provided.. Location change condition; O (see NOTE); It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates, SAI, ECGI, RAT, VAL service area. If the Frequency of reporting is event-triggered, this IE shall be provided.. Inside or outside indication; O (see NOTE); It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations represented by the VAL service area ID(s).. Reporting schedule; O; It indicates the requested reporting schedule, e.g., days of the week and/or time period for the location reporting.. NOTE: Triggering criteria based on the VAL service area represented by the VAL service area ID(s) may be provided when this information flow is from the VAL server to the location management server.. ### 9.3.5 Client-triggered or VAL server-triggered location reporting procedure Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting. Figure 9.3.5-1: Client-triggered location reporting procedure 1. Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers as specified in table 9.3.2.4-1 e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included. NOTE: Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger corresponding to loscation management client 1. 2. Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. a. If the adaptive reporting is requested in step 1, the location management server interacts with NWDAF (e.g., UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [23288]) to obtain the location management client 1 moving statistic and prediction and determines the reporting configuration based on VAL service ID, VAL server ID, and the retrieved location management client 1 moving statistic and prediction. b. Depending on the information specified by the location reporting trigger or determined reporting configuration, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. 3. Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server. Editor's note: Once SEAL LMS updates the location reporting trigger configuration based on adaptive indication, it is FFS how VAL server recieves and accepts the updated configuration.

### 6.x.3	Solution evaluation

This solution addresses KI#4 about whether and how to expose the location data information per the e.g. time/area/space granularity in application enabled layer.

This solution improves the SEAL Location Management service to provide the assistance to the VAL server in the location reporting configuration at the LM server. The solution provides the simplification of the LM-S reference point for exposure of the location information, enables the adaptive location reporting based on the UE moving trend, as well as may reduce the energy consumption of the UE via the optimization of the invoking frequency of the On-demand location reporting procedure.

The impacted entities are VAL server and LM server.

# *** End of Changes ***