3GPP TSG-SA6 Meeting #61	S6-242719

# Jeju, Korea (Republic Of), 20th May 2024 - 24th May 2024				was 2074, 2660

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0635 | rev | 2 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EES capability to influence traffic routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | VODAFONE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2024-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Several procedures in TS 23.558 include the EES influencing traffic routing, for example applying AF traffic influence with N6 routing information of a T-EAS. However, the ability of an EES to influence traffic is included in NOTE text and not normative text in 3 places (2 in Rel-17 and one more in Rel-18, Rel-19) and therefore might be missing from implementations. TR 21.801 (specification drafting rules) says the following regarding NOTEs: 6.5.1 Notes and examples integrated in the text Notes and examples integrated in the text of a 3GPP TS or 3GPP TR shall only be used for giving additional information intended to assist the understanding or use of the 3GPP TS or 3GPP TR. They shall not contain provisions to which it is necessary to conform in order to be able to claim compliance with a 3G TS. Also, the purpose of the EES influencing traffic routing is not included in the existing NOTEs but is explained in S6-211111 that added the NOTEs in Rel-17 and S6-223623 that added the note in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Made the ability for an EES to influence traffic routing to itself normative. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EES unable to influence traffic routing to itself. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.2.2, 8.3.3.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

##### 8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

If the ECS is not provisioned with any EDN configuration information or is unable to determine the EES information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

NOTE 4:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

After the EEC establishes a connection to an EES using information received in step 3, the EES can issue an AF request to influence traffic routing as specified in 3GPP TS 23.501 [2] clause 5.6.7 in order to influence the user plane path to this connected EES.

NOTE 5: For example, using a particular DNAI to reach the data network containing the EES might be necessary to meet AC service KPIs.

*** Next Change ***

###### 8.3.3.2.3.3	Notify

Figure 8.3.3.2.3.3-1 illustrates the service provisioning notification procedure between the EEC and the ECS.

Pre-conditions:

1.	The EEC has subscribed with the ECS for the provisioning information as specified in clause 8.3.3.2.3.2.

Figure 8.3.3.2.3.3-1: Service provisioning notification

1.	An event occurs at the ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC. If UE's location information is not available, the ECS may obtain the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) were provided by the EEC during subscription creation, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. If AC profiles(s) were not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 1:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 2:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

2.	The ECS sends a provisioning notification to the EEC with the list of EDN configuration information determined in step 1.

If the EDN configuration information in the service provisioning notification includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area, which can be discovered using the UE Registration Procedure.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

After the EEC establishes a connection to an EES using information received in step 2, the EES can issue an AF request to influence traffic routing as specified in 3GPP TS 23.501 [2] clause 5.6.7 in order to influence the user plane path to this connected EES.

NOTE 3: For example, using a particular DNAI to reach the data network containing the EES might be necessary to meet AC service KPIs.

*** End of Changes ***