3GPP TSG-SA WG6 Meeting #61	S6-242728

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242633,

S6-242246)

Source:	Samsung

Title:	Pseudo-CR on solution for KI 7

Spec:	3GPP TR 23.700-21 V0.3.0

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah, sapan.shah@samsung.com

1. Introduction

This contribution provides solution to KI 7.

2. Reason for Change

SA6 has agreed KI 6 with following open issue:

1)	To study whether and how to enhance architecture and procedures (e.g. EDGEAPP) to support metaverse service requiring multiple devices?

A metaverse service (e.g. a game service) requires haptic device associated with the UE/user in order for user to consume the service (e.g. to play the game). Current discovery mechanism (e.g. EAS discovery mechanism) do not support such application server requirements to be considered in the discovery procedure.

This pCR proposes to enhance EAS profile as defined in 3GPP TS 23.558 to enable EAS to provide required associated devices infomration. This pCR further proposes to enhance EAS discovery procedure as defined in 3GPP TS 23.558 to enable EEC to provide associated devices infomration.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 V0.3.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #7 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |
| Sol #3 | X |  |  |  |  |
| Sol #5 |  |  | X |  |  |
| Sol #8 |  |  |  | X |  |
| Sol #x |  |  |  |  | X |

* * * Next Change * * * *

## 7.x	Solution #x: Support for metaverse services requiring multiple devices

### 7.x.1	Solution description

This solution maps to KI# 7. A metaverse service (e.g. a game service) requires haptic device associated with the UE/user in order for user to consume the service (e.g. to play the game).

Editor’s note:	Whether EDGEAPP procedure enhacements are needed or not is FFS.

### 7.x.2	Architecture Impacts

This clause describes the architecture impacts to the EDGEAPP architecture as defined in 3GPP TS 23.335 [13]. Figure 7.x.2-1 illustrates the associated devices (e.g.  tactile gloves or VR glasses) are connected with the application client of the UE.

Figure 7.x.2-1: Associated devices connected with AC of the UE

The associated devices (UE-2 and UE-3) are connected to the application client of the UE using 3GPP or non-3GPP connection. The associated devices do not have EEC.

NOTE 1:	For simplicity, other elements of the EDGEAPP architecture are not shown in above figure as there are no impacts.

NOTE 2:	The application client in UE-1 is aware of the associated devices using application specific ways which is out of scope of SA6.

### 7.x.3	Corresponding APIs

Editor's Note:	This clause provides an corresponding APIS for this solution.

### 7.x.4	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution addressing KI#x.

This solution resolved open issue 1 of the KI 7. The archtiecture proposes to have application level connection between application client of the UE and other associated devices as shown in clause 7.X.2.

* * * End of Change * * * *