3GPP TSG-SA WG6 Meeting #61	S6-242730

Jeju, Republic of Korea, 20th – 24th May 2024	(revision of S6-242658, S6-242026)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0179 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Service API discovery involving multiple CCFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, CAPIF |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The Query information parameter in Service API discover request (Clause 8.7.2.1) is mandatory, and the Query information parameter in Interconnection service API discover request (Clause 8.25.2.3) is mandatory. And in 5.2.2.2.2 of TS 29.222, “To discover service APIs available at the CAPIF core function, the consumer (e.g. API invoker) shall send an HTTP GET message with the API invoker Identifier or CAPIF core function Identifier and query parameters to the CAPIF core function as specified in clause 8.1.2.2.3.1.” But now the description is “may include query information” in procedure step 1 (Clause 8.25.3.2). The description is “The identity of the CCF-A and the query information are included.” in procedure step 1 (Clause 8.25.3.3) which is correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For consistent with TS 29.222, correct the description in procedure step 1 of Clause 8.25.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The procedure description for query information is inconsistent with corresponding IE status and the procedure in TS 29.222. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.25.3.2	Service API discovery involving multiple CCFs

This subclause describes a procedure for service API discovery involving multiple CCFs

Pre-condition:

1.	CCF-A and CCF-B connect to each other, and either belong to the single trust domain of the same CAPIF provider or trust domains of different CAPIF providers.

2.	When CCF-A and CCF-B belong to trust domains of different CAPIF providers, the two CAPIF providers have business agreement for service API sharing.

Figure 8.25.3.2-1: Service API discovery y involving multiple CCFs

1.	The API invoker sends a service API discover request to the CCF-A. It includes the API invoker identity and  query information.

2.	The CCF-A verifies the identity of the API invoker and retrieves the stored service API(s) information and service API categories. The information of CCF-B with the service API category matching the discovery criteria is returned to API invoker in the service API discover response.

NOTE:	The remaining steps are only applied when the service API category is included in the interconnection API publish request as described in subclause 8.25.2.1.

3.	The API invoker sends an service API discover request to the CCF-B. The identity of API invoker is included. The query information is also provided.

4.	Upon receiving the service API discover request, the CCF-B verifies the identity of the API invoker. The CCF-B retrieves the stored service API(s) information as per the query information in the service API discover request. Further, the CCF-B applies the discovery policy and performs filtering of service APIs information which matches the discovery criteria.

5.	The CCF-B sends an service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

* * * End of Changes * * * *