3GPP TSG-SA WG6 Meeting #61	S6-242757

Jeju Island, South Korea, 20th - 24th May 2024	(revision of S6-242357, 2328, 2239)

Source:	Apple, AT&T

Title:	Backend For Frontend reference points

Spec:	3GPP TR 23.700-22 v0.2.0

Agenda item:	8.7

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

This paper proposes to highlight the required capabilities of the CAPIF-8 & CAPIF-X reference point introduced through solution 1 of 23.700-22, clause 6.2 on Backend For Frontend (BFF) based on the discussion in contribution S6-242238.

2. Reason for Change

Address the editor’s notes in clause 6.2.1 regarding the required capabilities for CAPIF-8 & CAPIF-X.

3. Proposal

It is proposed to update solution 1 of TR 23.700-22 v0.2.0 to provide the required capabilities of CAPIF-8 & CAPIF-X.

* * * First Change * * *

## 6.2	Solution #1: Backend For Frontend

### 6.2.1	Solution description

This solution relates to KI#3 on RNAA architecture enhancements. Specifically, it proposes the Backend For Frontend (BFF) pattern (as described, for example, in clause 6.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5]) to be included as an architectural option.

In 3GPP TS 23.222 [2] clause 7.5, the RNAA deployment option in which the API invoker is deployed on a UE is presented. A UE can be considered as hosting public clients according to the OAuth 2.0 Authorization Framework [6]. Such public clients lack the ability to maintain the confidentiality of secrets such as their client credentials or even access tokens provided to them. Regarding this vulnerability, as an example, clause 5.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5] highlights the danger of malicious JavaScript in relation to browser-based applications (highlighted as an example of a user-agent-based application public client in the OAuth 2.0 Authorization Framework [6]). This IETF draft presents the BFF application architecture pattern as an approach in which applications are built to rely on a backend component for handling OAuth responsibilities and then all requests are proxied through this backend component. Such an architectural approach is applicable to CAPIF as described in the next sub-clause.

NOTE:	The security aspects of the CAPIF architectural variant described in this solution are out of scope of the present document.

### 6.2.2	Architecture Impacts

In relation to the OAuth 2.0 Authorization Framework [6] the CAPIF resource owner function is responsible for interactions with the resource owner, which is anticipated to be in a similar manner to the way in which interactions between the resource owner’s user agent are described by the OAuth framework.

With OAuth there is also a client (or client application) that interacts with the user-agent and authorization server in order to gain access to information exposed by a resource server.

The OAuth resource server is considered as the CAPIF API exposing function, aligned with clause 6.5.3.1 of 3GPP TS 33.122 [3].

The OAuth client is considered as the CAPIF API invoker (aligned with clause 6.5.3.1 of 3GPP TS 33.122 [3]), where that API invoker can be UE hosted as previously highlighted.

Then in this solution (as presented in Figure 6.2.2-1) it is proposed to support an option in which the API invoker hosted by a UE (API invoker frontend) interacts via a new reference point (CAPIF-X) with a “helper” API invoker (API invoker backend) that is placed server-side (network-side) and is considered alongside the Backend For Frontend component in the Application Architecture presented in clause 6.1.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5].

According to that IETF draft, the BFF:

1.	Interacts with the authorization server as a confidential client

2.	Manages access and refresh tokens and does not share those with the client, i.e., the tokens are inaccessible to the client

3.	Proxies all requests from the client to the resource server, augmenting them with the correct access token before forwarding them to the resource server

In the IETF draft, an OAuth user-agent-based application is considered in which the client application runs within the user-agent. On the other hand, for an OAuth native application, the user-agent and client application are presented as separate entities, which is also the case the architectural solution proposal in Figure 6.2.2-1.

In the proposed architecture for CAPIF supporting BFF, the API invoker is split between a frontend and backend component. The backend acts as a proxy on behalf of the frontend. Through the API invoker backend, the API invoker frontend can leverage the capabilities of CAPIF including service discovery and authorization prior to accessing the service API over CAPIF-1(e) and service API invocation over CAPIF-2(e). However, it is the API invoker backend that onboards to the CAPIF core function (CCF) and mutually authenticates with it.

Figure 6.2.2-1: High level functional architecture for CAPIF supporting BFF

NOTE:	Impacts to the reference points with their assocaited procedures between the API provider domain functions and the CAPIF core function are not anticipated (and hence have not been depicted in Figure 6.2.2-1).

Figure 6.2.2-2: BFF based procedure for service API access

Figure 6.2.2-2 illustrates the BFF based procedure.

Pre-condition:

- The API invoker frontend and API invoker backend have an established secure session.

Firstly, in step 1, the UE hosted API invoker (API invoker frontend) initiates the authorization flow, where the API invoker’s objective is to gain access to particular resources offered through the service APIs exposed by the API provider domain.

In step 2, the API invoker backend initiates the authorization flow towards the CCF authorization function (CCF(AuthF)). The request is redirected via the UE hosted User agent.

In step 3, mutual authentication is performed between the CCF(AuthF) and the Resource owner via the Resource owner function and any required user consent obtained.

Once mutual authentication has been successfully perform, in step 4 the CCF(AuthF) provides indication of that granted authorization to the API invoker backend using redirection via the User agent.

Next in step 5, the API invoker backend requests access to the API exposing function’s service APIs from the CCF(AuthF) using the authorization obtained in step 4.

The API invoker backend maintains an association between its session with API invoker frontend and access information it obtains in step 6 from the CCF(AuthF).

The API invoker backend then provides authorization granted notification to the API invoker frontend that service API access is permitted via the User agent in step 7. The access security token itself is not passed to the API invoker frontend, but is stored by the User agent.

When the API invoker frontend subsequently initiates a service API request (to the AEF via API invoker backend), it does so via the User agent. The User agent then adds the access security token (that it obtained from the CCF(AuthF) in step 8) to the request.

The API invoker backend then uses its granted service API access that it received from the authorization function (that is associated with the API invoker frontend to API invoker backend session) to make the request towards the service API (step 10).

The response from the service API (step 10) is then forwarded towards the API invoker frontend by the API invoker backend (step 11).

### 6.2.3	Corresponding APIs

The following API calls are described in the context of those described in the BFF IETF RFC draft, draft-ietf-oauth-browser-based-apps-17 [5].

Editor's note:	Mapping of the API calls to existing CAPIF capabilities is FFS.

CAPIF-X (API invoker frontend – API invoker backend)

- Initiate authorization request, from API invoker frontend to API invoker backend (step 1, clause 6.2.2)

- Access granted indication, from API invoker backend to API invoker frontend via User agent (step 7, clause 6.2.2)

- API service request, from API invoker frontend to API invoker backend (step 8, clause 6.2.2)

- API service response, from API invoker backend to API invoker frontend (step 11, clause 6.2.2)

CAPIF-8 (ROF – CCF(AuthF))

- Granting authorization and user consent, from CCF(AuthF) to ROF (step 3, clause 6.2.2)

CAPIF-1 (API invoker backend – CCF)

- Authorization request, from API invoker backend to CCF(AuthF) redirected via User agent (step 2, clause 6.2.2)

- Authorization response from CCF(AuthF) to API invoker backend, redirected via User agent (step 4, clause 6.2.2)

- API invoker backend makes authorization access request to CCF(AuthF) (step 5, clause 6.2.2)

- CCF(AuthF) provides authorization access grant to API invoker backend (step 6, clause 6.2.2)

Editor's note:	The CAPIF-X related procedures are FFS.

* * * End of Changes * * *