3GPP TSG-SA WG6 Meeting #61	S6-242759

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242731)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0662 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing ACR failure cause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EEC services notification is missing “failed” ACR execution status and correpsonding failure cause IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following are the changes proposed: 1. Add missing ACR execution status in the EEC services notification 2. Add failure cause IE 3. Clarify in the related procedure that AC may trigger DNS query |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AC and EEC interaction specification would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.3.14, 8.14.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.14.3.14	EEC services notification

Table 8.14.3.14-1 describes information elements for the EEC services notification sent by the EEC to the AC.

Table 8.14.3.14-1: EEC services notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the notification. |
| Notification details | M | Information related to the notification |
| > EAS discovery | O | Indicates that the notification relates to EAS discovery |
| >> List of EAS profiles | M | EAS profiles associated with the notification. Each element is described in clause 8.2.4 |
| >>> Lifetime | O | Time interval or duration during which the information in the EAS profile is valid and supposed to be cached in the AC (e.g. time-to-live value for an EAS Endpoint) |
| > EAS dynamic information | O | Indicates that the notification relates to EAS dynamic information subscription |
| >> List of EAS profile | M | EAS profiles associated with the notification. Each element is described in clause 8.2.4 |
| >>> Lifetime | O | Time interval or duration during which the information in the EAS profile is valid and supposed to be cached in the AC (e.g. time-to-live value for an EAS Endpoint) |
| > ACR | O | Indicates that the notification relates to ACR |
| >> List of S-EAS information | M | ACR operation information and information of the EAS to which the notification relates. |
| >>> EASID | M | Identifier of the S-EAS to which the subscription relates. |
| >>> EAS endpoint | M | Endpoint of the S-EAS to which the subscription relates. |
| >>> ACR detection | O | Indicates that the EEC detected a need for ACR |
| >>> ACR decision | O | EEC’s decision information related to an ACR detection |
| >>> ACR execution status | O | Indicates the status of ACR execution |
| >>>> Status | M | ACR status such as initiated, in-progress, complete, failed |
| >>>> T-EAS information | O (NOTE) | Information of the T-EAS corresponding to the S-EAS |
| >>>>> EASID | M | Identifier of the T-EAS to which an ACR has been performed. |
| >>>>> EAS endpoint | M | Endpoint address of the T-EAS to which an ACR has been performed. |
| >>>> Failure cause | O (NOTE) | ACR failure reason e.g. due to unavailability of suitable T-EES or T-EAS |
| NOTE: “T-EAS information” IE shall be present only when the value of “Status” IE is other than failed. And “Failure cause” IE shall be present only when the value of “Status” is failed. |  |  |

* * * Next Change * * * *

##### 8.14.2.5.3	EEC services notification

Pre-conditions:

1.	The AC has subscribed to the EEC.

Figure 8.14.2.5.3-1: EEC services notification procedure

1.	An event occurs at the EEC that satisfies the trigger conditions for notifying a AC e.g., EEC detects a need for Application Context Relocation, unavailability of suitable T-EES and/or unavailability of suitable T-EAS.

NOTE:	Upon receiving a EEC services notification indicating no suitable T-EES/T-EAS was available, the AC may decide to initiate DNS query to obtain CAS information to prepare and move the application context to the CAS.

2.	The EEC sends an EEC services notification to the AC with relevant information related to the event triggered in step 1.

* * * End of Changes * * *