3GPP TSG-SA WG6 Meeting #62	S6-244059

Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

Source:	Motorola Solutions

Title:	Discussion paper on reference architecture to support recording for MC Services.

Document for:	DISCUSSION

1) Discussion

Recording reference points and interface is proposed as part TS 23.280. This proposal leverages reference point, interfaces defined in TS 23.280 and proposes recording reference architecture for MCX services (MCPTT, MCVIDEO, MCDATA).

1.1	Recording reference architecture for MCPTT Service.

- At start of MCPTT call session, MCPTT Server (based on configuration or criteria) shall initiate MCPTT call recording session with recording server.

  - SIP Recording procedures (Session Recording Protocol - RFC-7866) to used.

- Recording server shall honour new recording session and shall start new recording session.

- As MCPTT call session progresses, Session information (media, floor arbitration messages) is sent from MCPTT Server to recording server.

  - For sending MCPTT session metadata information, procedures documented in RFC-7865 (Session Initiation Protocol (SIP) Recording Metadata) to be used.

  - MCPTT session meta data information to be sent as XML content in SIP recording messages.

  - Session RTP media (Continuous stream of MCPTT session audio) shall be recorded.

- Once the MCPTT call session ends, MCPTT server shall communicate recording session end (complete) information to recording server.

- On receipt of recording server session completion message, recording server shall stop the recording and shall transfer recorded information (media+ call metadata content) to mass storge for later retrieval.

1.2	Recording reference architecture for MCVIDEO Service.

- At start of MCVIDEO call session, MCVIDEO Server (based on configuration or criteria) shall initiate MCVIDEO call recording session with recording server.

  - SIP Recording procedures (Session Recording Protocol - RFC-7866) to used.

- Recording server shall honour new incoming recording session and shall start new recording session.

- As MCVIDEO call session progresses, session information (voice, video, floor arbitration messages etc.) is sent from MCVIDEO server to recording server.

  - For sending MCVIDEO session metadata information, procedures documented in RFC-7865 (Session Initiation Protocol SIP Recording Metadata) to be used.

  - MCVIDEO session meta data information to be sent as XML content in SIP recording messages.

  - Session content to be recorded.

    - Audio (Continuous stream of MCVIDEO session audio)

    - Video (Continuous stream of MCVIDEO session video)

- Once the MCVIDEO call session ends, MCVIDEO server shall communicate recording session end (complete) information to recording server.

- On receipt of recording server session completion message, recording server shall stop the recording and shall transfer recorded information (voice, video, call metadata content) to mass storge for later retrieval.

1.3	Recording reference architecture for MCDATA Service.

- On completion of delivery of MCDATA FDS/SDS message (with or without acknowledgement) towards the target MC users, MCDATA server shall initiate sending of MCDATA conversation information to recording server.

  - Metadata, file attachment over HTTPS or

  - Metadata, file transfer over SFTP to be used.

- On receipt of Metadata, file attachments, recording server shall push this information to Recording mass storage server (for later retrieval).

NOTE:

Metadata, file attachment specific to MCDATA conversation can be pushed immediately (once the conversation ends) or can be transferred periodically based on the periodicity configured in the MCDATA server (this aspect will be implementation specific).

2) Need for content transfer interface for reference architecture.

- Current TS 23.280 captures reference point REC-6 (between recording server and MC service servers), this reference point only addresses Subscription/Notification information.

- To capture/transfer of actual MC service specific information (voice, video, data, metadata, attachments, etc.) from MC service server to recording server, no reference point is defined currently in TS 23.280. This proposal is to emphasize the need for such new reference point.

3) Conclusion

- This paper recommends

- SIP REC based recording support for MCPTT call sessions.

- SIP REC based recording support for MCVIDEO call sessions.

- SFTP based file transfer for supporting MCDATA.

3.1 Why SIP Recording

- SIP REC is industry standard for supporting voice, video communication recording.

- SIP REC procedures integrate well with MCPTT, MCVIDEO session procedures as MC Service procedures, proposed recording procedures works on same application protocol (SIP).

- SIP REC supports high quality data capture, in compliance with legal requirements, suits mission critical requirements, use cases.