3GPP TSG-SA WG6 Meeting #63	S6-244068

Hyderabad, India, 14th – 18th October 2024

Source:	InterDigital Inc., Samsung, Ericsson

Title:	Pseudo-CR on Split AIML operation node registration

Spec:	3GPP TS 23.482 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This CR brings normative text on split AIML operation node registration.

2. Reason for Change

The following agreement has been approved in TR 23.700-82 on Split AI/ML operation.

The following principles will be considered in the normative phase for KI #5.

Split AI/ML operation:

- Solution #19 will be considered for AIML split operation pipeline discovery (request/response), events subscription/notification and split an operation pipeline creation/registration (request/response). AIML split operation profile provides the information for exposing and consuming split AI/ML analytics.

- Solution #28 will be considered for AIML split operation pipeline assistance information subscription/notification. AIML split operation assistance information provides the information for exposing and consuming split AI/ML analytics.

NOTE 1: Possible merge or alignment of the subscribe/notify model and AIML split operation information in Solution #19 and Solution #28 will be considered in the normative phase.

-	Solution #23 can rely on solution #19 and #28 to manage and expose created AIML split operation pipeline information in edge scenarios. Further discussions will happen in the normative phase to understand how the capabilities of the CES and EES can be enhanced for creating AIML split operation pipelines in edge areas.

Solution #19 and Solution #28 have a complementary relationship; Solution #23 addresses different aspects of split AIML operation related to edge and cloud.

3. Conclusions

This pCR provides normative text for KI#5 Solution #19 and brings procedure and information elements for split AIML operation node registration.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.2.0.

* * * First Change * * * *

## 8.x	Support for Split AI/ML Operation

### 8.x.1	General

Split AI/ML operation (e.g., split operation) procedures enable applications consuming services from the AI/ML application enablement layer to discover and create instance(s) of split operation pipeline and addresses aspects related to consumption of split operation results.

Split operation node registration enables a VAL server to register with the AIMLE server to indicate split operation capabilities. Registered VAL servers can be notified by the AIMLE server for participating in an instance of a split operation pipeline.

* * * Next Change * * * *

### 8.x.2	Procedure

#### 8.x.2.1	General

Following procedures are supported for split AI/ML operation:

-	Split operation node registration, registration update and de-registration request-response procedures;

* * * Next Change * * * *

#### 8.x.2.4	Split operation node registration

##### 8.x.2.4.1	General

Clause 8.x.2.4.2 illustrate the split operation node registration procedure.

Clause 8.x.2.4.3 illustrates the split operation node registration update procedure.

Clause 8.x.2.4.4 illustrates the split operation node de-registration procedure.

##### 8.x.2.4.2	Split operation node registration

Figure 8.x.2.4.2-1 illustrates the procedure for VAL server to register with the AIMLE server to indicate split operation capabilities.

Pre-conditions:

- The VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

- The VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.4.2-1: Split operation node registration

- The VAL server sends a split operation node register request to the AIMLE server to indicate the split operation capabilities of the VAL server. The request includes information defined in Table 8.x.3.6-1.

Editor’s note:	Whether an AIMLE client can register as a processing node is FFS.

- Upon receiving the request from the VAL server, the AIMLE server validates if the requestor is authorized for the request, the AIMLE server stores the registration information.

3.	The AIMLE server sends a split operation node register response to the requestor. If the AIMLE server has registered the requestor, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation node registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.4.3	Split operation node registration update

Figure 8.x.2.5-1 illustrates the procedure for VAL server to update a split operation node registration with the AIMLE server.

Pre-conditions:

- The VAL server has registered with the AIMLE Server.

Figure 8.x.2.4.3-1: Split operation node registration update

1.	The VAL server sends a split operation node registration update request to the AIMLE server to update an existing split operation registration. The request includes information defined in Table 8.x.3.8-1.

- Upon receiving the request from the VAL server, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the registration information.

- The AIMLE server sends a split operation node registration update response to the requestor. If the AIMLE server has successfully updated the registration, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.4.4	Split operation node de-registration

Figure 8.x.2.4.4-1 illustrates the procedure for VAL server to de-register for split operation with the AIMLE server.

Pre-conditions:

- The VAL server has registered with the AIMLE Server.

Figure 8.x.2.4.4-1: Split operation node de-registration

1.	The VAL server sends a split operation node de-register request to the AIMLE server. The request includes information defined in Table 8.x.3.10-1.

2.	Upon receiving the request from the VAL server, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server terminates the registration for split operation.

3.	The AIMLE server sends a split operation node de-register response to the requestor. If the AIMLE server has successfully terminated the registration for split operation node, the response includes an indication of success. If the registration for split operation was not terminated, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	General

The following information flows are specified for operation splitting.

* * * Next Change * * * *

#### 8.x.3.6	Split operation node register request

Table 8.x.3.6-1 shows the request sent by a VAL server to the AIMLE server for split operation node register request.

Table 8.x.3.6-1: Split operation node register request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Security credentials | M | The security credentials of the requestor. |
| Node information | M | Information about the VAL server node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | M | Split operation capabilities of the VAL server. |
| > Model information | M | Information about ML model capabilities of the VAL server for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Expiration time | O | The proposed expiration time of the registration. |

#### 8.x.3.7	Split operation node register response

Table 8.x.3.7-1 shows the response sent by an AIMLE server after processing of split operation node register request.

Table 8.x.3.7-1: Split operation node register response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > Registration identifier | M | The identifier of the registration. |
| > Expiration time | O | The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

#### 8.x.3.8	Split operation node registration update request

Table 8.x.3.8-1 shows the request sent by a VAL server to the AIMLE server for split operation node registration update request.

Table 8.x.3.8-1: Split operation node registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the registration. |
| Security credentials | M | The security credentials of the requestor. |
| Node information | O | Information about the VAL server node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | O | Split operation capabilities of the VAL server. |
| > Model information | O | Information about ML model capabilities of the VAL server for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Expiration time | O | The proposed expiration time of the registration. |

#### 8.x.3.9	Split operation node registration udpate response

Table 8.x.3.9-1 shows the response sent by an AIMLE server after processing of split operation node registration update request.

Table 8.x.3.9-1: Split operation node registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > Expiration time | O | The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

#### 8.x.3.10	Split operation node de-register request

Table 8.x.3.10-1 shows the request sent by a VAL server to the AIMLE server for split operation de-register request.

Table 8.x.3.10-1: Split operation node de-register request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the registration. |
| Security credentials | M | The security credentials of the requestor. |

#### 8.x.3.11	Split operation node de-register response

Table 8.x.3.11-1 shows the response sent by an AIMLE server after processing of split operation de-register request.

Table 8.x.3.11-1: Split operation de-register response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

* * * End of Changes * * * *