3GPP TSG-SA6 Meeting #63	S6-244239

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0336 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Application QoS coordination for Mobile Metaverse Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution provides a new feature related to the concluded solution #11 of TR 23.700-21. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The change includes mainly the introduction of new procedure in SEAL NRM capability related to “QoS/resource management for network-assisted UE-to-UE/VN group communications” for capturing the procedure for supporting mobile metaverse use cases (covering interactions among VAL UEs and VAL servers in both physical and virtual world). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New capability based on mobile metaverse use cases will not be added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses: 14.3.5.1, 14.3.5.X (new), 14.3.5.X.1 (new), 14.3.5.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.5	QoS/resource management for network-assisted UE-to-UE/VN group communications

#### 14.3.5.1	General

This feature provides the SEAL NRM support for coordinated QoS/resource management for network assisted UE-to-UE communications. Such capability may be required for guaranteeing end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements) in network assisted UE to UE communications and may accommodate various vertical-specific application services, e.g.:

-	Network-assisted Command and Control (C2) communications in UASAPP [TS 23.255], where the UAV controller navigates its UAV over the 5GS;

-	Teleoperated Driving (ToD) in eV2XAPP [TS 23.286], where the a V2X UE acting as server may remotely control a further V2X UE over the 5GS;

-	Network-assisted Device-to-Device communications in Factory of the Future (FF) use cases, such as control-to-control communications.

-	5G LAN-Type communication within a 5G VN group as specified in 3GPP TS 23.501 [10].

-	Application QoS coordination for Mobile Metaverse Services.

* * * Second Change * * * *

#### 14.3.5.X	Application QoS coordination for Mobile Metaverse Services

#### 14.3.5.X.1	General

This clause provides a mechanism for application session QoS coordination for mobile metaverse (MM) services.

It is assumed that the MM service consists of the following VAL sessions which can be present for multi-user interactions. The sessions may include VAL UE1 and VAL UE2 and digital counterparts of UE1 and UE2 instantiated in one or more DNs (called as virtual or avatar VAL UEs).

-	VAL session #1: VAL client 1 sends to VAL server 1 (including virtual VAL UE 1 at DN side) sensor data / measurements on the physical environment related to VAL UE1 over VAL-UU interface. VAL server (virtual VAL UE1) sends back haptic feedback to VAL UE1 (for UE1 and/or UE2 and the environment).

-	VAL session #2: VAL client 2 sends to VAL server 2 (including virtual VAL UE 2 at DN side) sensor data / measurements on the physical environment related to VAL UE2 over VAL-UU interface. VAL server (virtual VAL UE2) sends back haptic feedback to UE2 (for UE1 and/or UE2 and the environment).

-	VAL session #3: Exchange of service related / feedback data between VAL server 1 and 2 for interactions between virtual UE 1 and 2 (for example micro-transactions such as avatar updates/modifications or environment changes).

-	VAL session #4: Sensor data / measurements are exchanged between VAL UEs over VAL-PC5 (communication can be over side link) for traffic related to the MM service.

#### 14.3.5.X.2	Procedure on application QoS coordination for Mobile Metaverse Services

Figure 14.3.5.X.1-1 illustrates the procedure where the NRM server is supporting application QoS coordination for the VAL sessions within a MM Service.

Pre-conditions:

1.	The NRM clients are connected to the NRM server.

2.	The VAL servers are connected to the NRM server.

3.	The VAL sessions corresponding to the Mobile Metaverse (MM) Service (e.g., VAL client 1 to VAL server 1, VAL client 2 to VAL server 2) are established.

Figure 14.3.5.X.1-1 application QoS coordination for VAL MM services

- The NRM client 1 (acting on behalf of VAL client 1) sends to the NRM server an MM-specific QoS management request to trigger the application QoS coordination between the physical and virtual VAL UEs within MM service. This request may also include the end-to-end QoS requirement for the MM service.

- The NRM server decomposes the MM service performance requirement (end to end application QoS target) to per VAL session requirement, where in this step the NRM server identifies the VAL sessions for which the application QoS parameters need to be configured or updated.

- The NRM server configures the application QoS parameters per VAL session, for the identified VAL sessions to ensure meeting the end-to-end QoS requirements for the MM service.

- The NRM server sends the MM-specific QoS management response to the NRM client 1 to provide the configuration information for the QoS attributes for the VAL sessions related to VAL UE 1.

- The NRM server sends a QoS configuration notification to the VAL servers 1 and 2 as well as to the NRM client 2 to provide the configuration information for the QoS attributes for the corresponding VAL sessions.

* * * Third Change * * * *