3GPP TSG-SA WG6 Meeting #63	S6-244274

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0208 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align CAPIF API publish and discover with SA3 SNAAPP enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI, SNAAPP |  |  |  |  |  | Date: |  |  | 2024-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 defines three optional OAuth 2.0 flows (Client credential flow (according to RFC 6749 [4]), Authorization code flow (according to RFC 6749 [4]), or Authorization code flow with PKCE (according to RFC 7636 [11])) for RNAA authorization are specified in TS 33.122. A service APIs or an API invoker may support only one or two of the optional flows. Currently, the CCF is not aware the OAuth 2.0 flows per each service API, then a OAuth 2.0 flow maybe negotiated between the CCF and the API invoker while the AEF/service API does not support this negotiated OAuth 2.0 flow. For example: API invoker supports the Client credential flow and Authorization code flow, but the Service API A supports Authorization code flow and Authorization code flow with PKCE. Then if Client credential flow is selected by the CCF due to not aware of the service API capabilities from the AEF, then failure occurs. So, in order to avoid such failure, the service API publishing and discovery procedures need updates to reflect the fact that a service API does not have to support all the flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the service API publish and discover information flow to support the OAuth 2.0 flows negotiation between the CCF and API invoker per service API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure may occur if the CCF and API invoker do not know the supported OAuth 2.0 flows of a particular service API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1, 8.7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ 1st changes *************************/

8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API type, service API status (e.g. active, inactive), communication category, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), and Network Slice Info (optional), supported OAuth Flows (optional). |
| Shareable information | O (see NOTE) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API is not allowed to be shared. |  |  |

The Service KPIs is defined as below:

Table 8.3.2.1-2: Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |

# /************************ 1st changes *************************/

8.7.2.2	Service API discover response

Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker.

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including service API information such as service API name, API provider name (optional), service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version, data format, Service KPIs (optional), and Network Slice Info (optional), supported OAuth Flows (optional). |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. |  |  |