3GPP TSG-SA WG6 Meeting #63	S6-244299

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Ericsson

Title:	Discussion Paper on Rel-19 Application Data Analytics Enablement Service

Agenda Item:	10

Contact:	Jing Yue (jing.yue@ericsson.com)

# 1	Decision/action requested

Discuss the possible ways and guidance on enhancing the ADAES functionality.

# 2	References

[1]	S6-240552 "Reply LS on A-DCCF, A-ADRF and ADAES services"

[2]	3GPP TS 23.436 "Functional architecture and information flows for Application Data Analytics Enablement Service"

[3]	S6-240019 "LS on A-DCCF, A-ADRF and ADAES services"

# 3	Rationale

Release-18 normative work on Application Data Analytics Enablement Service (ADAES) in SA6 has introduced data analytic services for vertical and edge applications.

As agreed in S6-240552 [1], SA6 will consider working on the definition of clear metrics for Analytics/Data Output. Continuous developing of the ADAES services in Release-19 is under TEI19 work item, includes improvements, corrections, and clarifications for the A-ADRF, A-DCCF, and ADAES services in TS 23.436 [2].

However, the detail content of analytics inputs/outputs and data collection are still unclear in TS 23.436 [2]. Thus, it is important to have a discussion on a way to enhance ADAES to specify metrics related to analytics inputs/outputs, data collection, and data storage in Release-19 work.

# 4	Discussion

## 4.0	ADAES-related functiionality

SA6 introduced ADAES in Release-18. Continuous developing of the ADAES-related corrections and improvements is under TEI19 WI. However, the detail content of the analytics inputs/outputs, data collection, and data storage.

Thus, introduction of a small WID in SA6 for TEI19_ADAES would provide the following advantages:

-	Allow the clarification of metrics related to the analytics inputs/outputs of the analytics introduced in TS 23.436 [2].

-	Enable the enhance of IEs in the request/response of data collection from Data Producer and/or A-ADRF (or via A-DCCF).

-	Complete the definition of IEs in the data/analytics storage subscription request to A-ADRF.

## 4.1	ADAES analytics

The inputs and outputs of analytics introduced in TS 23.436 [2] are described in corresponding information flows clauses for each analytics. For example, the inputs and outputs of the edge analytics are given in as the analytics request and analytics notification in clauses 8.8.3.2 and s 8.8.3.7 of TS 23.436 [2] respectively.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS endpoint for which the analytics subscription applies. |
| Destination EES information | O (NOTE) | This identifier shows the destination EES information including destination EES ID and destination EES endpoint for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| NOTE: At least one of these shall be present. |  |  |

Table 8.8.3.7-1: Edge analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Output | M | The predictive or statistical parameter, which can be stats or prediction related to the edge performance/load for the edge platform or EES/EAS for a given area/time and based on the event type. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

Observation 1: There is no clear definition for some IEs in the inputs/outputs of the edge load analytics. In the analytics request, general information such as Analytics ID and Analytics type are given. However, detailed definition of analytics output is missing, e.g., the content of required outputs for edge performance or edge load are unclear. Correspondingly, the outputs for the analytics are unclear. The same issue also exists in the other analytics which are introduced in TS 23.436 [2]:

- application performance analytics;

- slice-specific application performance analytics;

- UE-to-UE application performance analytics;

- location accuracy analytics;

- service API analytics;

- slice usage pattern analytics.

No clear content defined for the inputs/outputs of the analytics may cause implementation problems, such as the questions from CT3 in S6-240019 [3]: “Does the specific content of the Analytics/Data Output need to be defined? What specific metric information should be included in each Analytics/Data Output?”

Proposal 1: Enhance the analytics introduced in TS 23.436 [2] by specifying metrics related to analytics inputs/outputs.

## 4.2	Data collection

The data collection request to and response from Data Producer and/or A-ADRF are introduced in TS 23.436 [2]. For example, the inputs and outputs of the location accuracy data collection from A-ADRF are given in as the data collection request and response in clauses 8.5.3.4 and 8.5.3.5 of TS 23.436 [2] respectively.

Table 8.5.3.4-1: Location accuracy data request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Analytics ID | M | The identifier of the analytics event |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Reporting configuration | O | The configuration for data reporting. This requirement may include e.g. the frequency of reporting (periodic), the reporting periodicity in case of periodic, and reporting thresholds, whether data abstraction is needed or not. |
| Data collection requirements | O | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

Table 8.5.3.5-1: Location accuracy data response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Analytics Output | M | The reported analytics for the location accuracy, which can be in form of offline stats/historical data for a specific VAL service or for particular UE(s) or group of UEs |

Observation 2: There is no clear definition on content and metrics for the inputs/outputs of the data collection. In the analytics request, general information such as Analytics ID is given. However, detail requirement on data to be collected is missing, e.g., the required data is offline stats or historical data, the data is for a specific VAL service or for particular UE(s), are unclear. Correspondingly, the outputs for the data collection are unclear. The same issue also exists in the other clauses in TS 23.436 [2], e.g., data collection in clause 8.8.3.6 for edge data, etc.

Proposal 2: Enhance the data collection introduced in TS 23.436 [2] via enhance the IEs in the request/response of data collection from Data Producer and/or A-ADRF (or via A-DCCF).

## 4.3	Data storage

The data/analytics storage to A-ADRF is defined in TS 23.436 [2]. For example, the data storage subscription request to A-ADRF is given in clauses 8.10.3.2 of TS 23.436 [2].

Table 8.10.3.2-1: Data storage subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (ADAE server, A-DCCF). |
| Data Producer ID | O | The identifier of the data producer (e.g. ADAE server, A-DCCF). |
| Analytics ID | O | The identifier of the analytics event. This IE shall be provided when Analytics Producer ID is present. |
| Analytics Type | O | The type of analytics for the event, which include category (e.g. statistics or predictive analytics), and may include mode (e.g. offline or online), and indication on ML-enabled analytics. This IE shall be provided when Analytics Producer ID is present. |
| Target Data Producer Profile Criteria | O | Characteristics of the data producers to be used. This IE shall be provided when Data Producer ID is present. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time Validity | O | The time validity of the subscription request to the A-ADRF. |
| Storage Handling information | O | The information for storage handling. |
| > Lifetime | O | A lifetime for how long the data or analytics should be stored. |
| > Alert Indicator | O | To indicate that a notification alerting the consumer be sent prior to data deletion from the A-ADRF. |

Observation 3: The data storage subscription request to A-ADRF is incomplete. There is an editor’s note left under the table 8.10.3.2-1about “what IEs be contained in the request and response tables 8.10.3.2-1 and 8.2.4.5-1 is FFS”.

Proposal 3: Enhance the data storage introduced in TS 23.436 [2] by completing the definition of IEs in the data storage subscription request to A-ADRF.

## 4.4	Detailed proposal

Based on the background information, it is proposed.

Proposal 1: Enhance the analytics introduced in TS 23.436 [2] by specifying metrics related to analytics inputs/outputs.

Proposal 2: Enhance the data collection introduced in TS 23.436 [2] via enhance the IEs in the request/response of data collection from Data Producer and/or A-ADRF (or via A-DCCF).

Proposal 3: Enhance the data storage introduced in TS 23.436 [2] by completing the definition of IEs in the data storage subscription request to A-ADRF.

# 5	Conclusion

This discussion paper provided the background information and the related justifications to the identified Issue of ADAES services documentation in Release-19.

Ericsson will be happy to bring the small WID according to Proposals described in section 4.4 of this DP to resolve the identified Issue and improve the documentation of ADAES services. The target completion plenary for the work of this small WID is TSG#107 (March 2025).

Ericsson is opened to other alternatives that resolve the identified issues.