3GPP TSG-SA WG6 Meeting #63	S6-244312

Hyderabad, India, 14th – 18th October 2024

Source:	Ericsson

Title:	Spatial Anchor Analytics Information

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This CR brings normative text related to the Spatial anchor analytics information procedure.

2. Reason for Change

The spatial anchor usage analytics information was studied and concluded for normative work. To derive the analytics based on the spatial anchor usage information, an ADAE service is added.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.2.0.

* * * First Change * * * *

#### 8.3.x	Spatial anchor analytics information

##### 8.3.x.1	General

Spatial anchor analytics information enables a VAL server to request analytics related to spatial anchor(s) with the SAn server.

##### 8.3.x1.2	Spatial anchor analytics information(Sub-Notify)

Figure 8.3.x1.2-1 depicts the procedure for subscribing to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.3.x1.2-1: Spatial anchor analytics information subscription

1.	A consumer(e.g. ADAES) sends a spatial anchor analytics information subscription request to the SA server). The request can include the VAL server identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor analytics filters like spatial anchor identifiers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and analytics sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and sends a spatial anchor analytics subscription response to the consumer.

3.	The SAn server samples the information related to spatial anchor-related analytics based on the obtained data and the stored spatial anchor data as per the analytics sampling interval. It provides response to the consumer as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information, spatial anchor services session time per user.

#### 8.3.x2.2	Spatial anchor analytics information(Request-Response)

Figure 8.3.x2.2 depicts the procedure to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.3.x2.2-1 Spatial anchor analytics

1)	The consumer(e.g. ADAES) sends a request message to the SAn server to get the spatial anchor analytics information as per the Table 8.3.x.3.1-1.

2)	The SAn server authorizes the consumer. If the requestor is authorized, then the SAn server server provides the spatial anchor analytics information.  The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information, spatial anchor services session time per user.

* * * End of Changes * * * *

#### 8.3.x.3	Information flows

##### 8.3.x.3.1	Spatial anchor analytics information subscription request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor analytics information subscription request from the consumer to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor analytics information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor analytics filters | O | Indicates the spatial anchor filter for the analytics information |
| > Spatial anchor identifiers | O | Indicates the spatial anchor identities |
| > Spatial anchor location information | O | Indicates the spatial anchor location information |
| > UE position information | O | Indicates the UE position information (e.g. location information, pose) |
| > proximity information | O | Indicates the proximity information (e.g. radial distance) |
| > Application service information | O | Indicates the application service information(e.g. application identifier) |
| Analytics sampling interval | O | Indicates the interval to sample the spatial anchor(s) related information to generate analytics |
| Reporting interval | O | Indicates the periodic reporting interval of analytics information |
| notification endpoint | O | Indicates the endpoint(e.g., IP address, FQDN, URI) to send the analytics information notification |

##### 8.3.x.3.2	Spatial anchor analytics information subscription response

Table 8.3.x.3.2-1 describes information elements for the spatial anchor analytics information subscription response from the SAn server to the consumer.

Table 8.3.x.3.2-1: Spatial anchor analytics information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Spatial anchor Analytics subscription ID | M | Identity for the spatial anchor analytics subscription |
| Failure response | O | Indicates that the create spatial anchor group request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.3.x.3.3	Spatial anchor analytics information notification

Table 8.3.x.3.3-1 describes information elements for the spatial anchor analytics information notification from the SAn server to the consumer.

Table 8.3.x.3.3-1: Spatial anchor analytics information notification

| Information element | Status | Description |
| --- | --- | --- |
| > Spatial anchor Analytics subscription ID | M | Identity for the spatial anchor analytics subscription |
| > Spatial anchor identifiers | M | Indicates the spatial anchor identities |
| > Spatial anchor usage information | M | Indicates spatial anchor usage information (e.g. number of times spatial anchor discovered and accessed, spatial anchor services accessed information, spatial anchor services session time per user) |
| > Spatial anchor density per location | M | Indicates the density of the spatial anchor in the given area. |
| > User density per spatial anchor | M | Indicates the density of users within proximity to a specific spatial anchor. |

* * * End of Changes * * * *

#### 8.3.x3.1	Spatial anchor analytics information (subscription-notify)

Figure 8.3.x3.1-1 depicts the procedure for subscribing to receive spatial anchor analytics information for metaverse applications. The service is provided by ADAE  server.

Figure 8.3.x3.1-1: Spatial anchor analytics subscription

1)	The consumer(e.g.VAL server) sends a subscription request to the ADAES to get notifications about the spatial anchor analytics information. The request includes analytics filters, periodicity of notification and analytics sampling interval as per the Table

2)	The ADAES server authorizes the consumer. If the requestor is authorized, then the ADAES sends a successful response along with the subscription identity.

3-4)	Based on the analytics sampling interval, the ADAES fetches the information related to the spatial anchor, UE from data producers like 5GS and spatial Anchor server. It uses the spatial anchor information analytics procedure defined in clause 8.3.x1.1 or clause 8.3.x2.1 to collect the data related to the usage of spatial anchor.  It can use TS 23.502 NEF services and TS 23.436 ADAE analytics services to fetch details like UE communication pattern, UE location information, UE mobility pattern, UE session information for the spatial anchor services, service API analytics information, and application layer performance analytics. Based on the collected data, the ADAE server derives the analytics related to the spatial anchor. The generated analytics can be predicted or statistical based on the requested analytics type.

5.)	The ADAES sends the notification to the subscribed consumers(e.g VAL server)  providing the spatial anchor analytics details as described in Table 8.3.x3.2.3.

#### 8.3.x3.2	Information flows

##### 8.3.x3.2.1	Spatial anchor analytics information subscription request

Table 8.3.x3.2.1-1 describes information elements for the spatial anchor analytics information subscription request from the VAL server to the SAn server.

Table 8.3.x3.2.1-1: Spatial anchor analytics information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example “Spatial anchor usage analytics”. |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| Spatial anchor analytics filters | M | Indicates the spatial anchor filter for the analytics information |
| > Spatial anchor identifiers | O | Indicates the spatial anchor identities |
| > Spatial anchor location information | O | Indicates the spatial anchor location information |
| > UE position information | O | Indicates the UE position information (e.g. location information, pose) |
| > proximity information | O | Indicates the proximity information (e.g. radial distance) |
| > Application service information | O | Indicates the application service information(e.g. application identifier) |
| Analytics sampling interval | O | Indicates the interval to sample the spatial anchor(s) related information to generate analytics |
| Time Validity and horizon | O | Time validity of the subscription request and time horizon for the predictions. |
| Area of interest | O | Geographical or topological area for which the subscription applies. |
| Notification reception information | O | Indicates the endpoint(e.g., IP address, FQDN, URI) to send the analytics information notification |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement include the type(e.g. immediate, periodic, event-triggered) and frequency of reporting(e.g, the reporting periodicity in case of periodic, and reporting thresholds for event-triggered). |

##### 8.3.x3.2.2	Spatial anchor analytics information subscription response

Table 8.3.x3.2.2-1 describes information elements for the spatial anchor analytics information subscription response from the SAn server to the VAL server.

Table 8.3.x3.2.2-1: Spatial anchor analytics information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Spatial anchor Analytics subscription ID | M | Identity for the spatial anchor analytics subscription |
| Failure response | O | Indicates that the create spatial anchor group request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.3.x3.2.3	Spatial anchor analytics information notification

Table 8.3.x3.2.3-1 describes information elements for the spatial anchor analytics information notification from the SAn server to the VAL server.

Table 8.3.x3.2.3-1: Spatial anchor analytics information notification

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor Analytics subscription ID | M | Identity for the spatial anchor analytics subscription |
| Consumer ID | M | The information to determine the identity of the subscribing entity (consumer) |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example “Spatial anchor usage analytics”. |
| Analytics Output | M | Stats or predictions indicating the spatial anchor usage information (e.g. number of times spatial anchor discovered and accessed, spatial anchor services accessed information, spatial anchor services session time per user), Spatial anchor density per location(e.g. Indicates the density of the spatial anchor in the given area), User density per spatial anchor(e.g. density of users within proximity to a specific spatial anchor) |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| Area of validity | O | Geographical or topological area for which the analytics apply. |

* * * End of Changes * * * *