3GPP TSG-SA WG6 Meeting #63	S6-244566

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244312)

Source:	Ericsson

Title:	Spatial Anchor Analytics Information

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This CR brings normative text related to the Spatial anchor analytics information procedure.

2. Reason for Change

The spatial anchor usage analytics information was studied and concluded for normative work. To derive the analytics based on the spatial anchor usage information, an ADAE service is added.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.2.0.

* * * First Change * * * *

## 8.x	Spatial anchor analytics information

### 8.x.1	General

The lifecycle management of the AIML services is an essential requirement for the applications to manage the AIML services like model training, inference, discovery etc.

### 8.x.2	Spatial anchor analytics information(Sub-Notify)

Figure 8.x.2.1-1 depicts the procedure for subscribing to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.x.2.1-1: Spatial anchor analytics information subscription

1.	A consumer(e.g. VAL server) sends a spatial anchor analytics information subscription request to the SA server). The request can include the VAL server identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor analytics filters like spatial anchor identifiers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and analytics sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and sends a spatial anchor analytics subscription response to the consumer.

3.	The SAn server samples the information related to spatial anchor-related analytics based on the obtained data and the stored spatial anchor data as per the analytics sampling interval. It provides response to the consumer as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information, spatial anchor services session time per user.

### 8.x.3	Spatial anchor analytics information(Request-Response)

Figure 8.x.3.1-1 depicts the procedure to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.x.3.1-1 Spatial anchor analytics information

1)	The consumer(e.g. VAL server) sends a request message to the SAn server to get the spatial anchor analytics information as per Table 8.x.4.2-1.

2)	The SAn server authorizes the consumer. If the requestor is authorized, then the SAn server server provides the spatial anchor analytics information.  The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information, spatial anchor services session time per user.

* * * End of Changes * * * *

### 8.x.4	Information flows

#### 8.x.4.1	Spatial anchor analytics information subscription request

Table 8.x.4.1-1 describes information elements for the spatial anchor analytics information subscription request from the consumer to the SAn server.

Table 8.x.4.1-1: Spatial anchor analytics information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor analytics filters | O | Indicates the spatial anchor filter for the analytics information |
| > Spatial anchor identifiers | O | Indicates the spatial anchor identities |
| > Spatial anchor location information | O | Indicates the spatial anchor location information |
| > UE position information | O | Indicates the UE position information (e.g. location information, pose) |
| > proximity information | O | Indicates the proximity information (e.g. radial distance) |
| > Application service information | O | Indicates the application service information(e.g. application identifier) |
| Analytics sampling interval | O | Indicates the interval to sample the spatial anchor(s) related information to generate analytics |
| Reporting interval | O | Indicates the periodic reporting interval of analytics information |
| notification endpoint | O | Indicates the endpoint(e.g., IP address, FQDN, URI) to send the analytics information notification |

#### 8.x.4.2	Spatial anchor analytics information subscription response

Table 8.x.4.2-1 describes information elements for the spatial anchor analytics information subscription response from the SAn server to the consumer.

Table 8.x.4.2-1: Spatial anchor analytics information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Spatial anchor Analytics subscription ID | M | Identity for the spatial anchor analytics subscription |
| Failure response | O | Indicates that the create spatial anchor group request has failed. |
| > Cause | O | The cause for the request failure. |

#### 8.x.4.3	Spatial anchor analytics information notification

Table 8.x.4.3-1 describes information elements for the spatial anchor analytics information notification from the SAn server to the consumer.

Table 8.x.4.3-1: Spatial anchor analytics information notification

| Information element | Status | Description |
| --- | --- | --- |
| > Spatial anchor Analytics subscription ID | M | Identity for the spatial anchor analytics subscription |
| > Spatial anchor identifiers | M | Indicates the spatial anchor identities |
| > Spatial anchor usage information | M | Indicates spatial anchor usage information (e.g. number of times spatial anchor discovered and accessed, spatial anchor services accessed information, spatial anchor services session time per user) |
| > Spatial anchor density per location | M | Indicates the density of the spatial anchor in the given area. |
| > User density per spatial anchor | M | Indicates the density of users within proximity to a specific spatial anchor. |

* * * End of Changes * * * *