3GPP TSG-SA WG6 Meeting #63	S6-244571

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244186)

Source:	Samsung, ETRI, Convida Wireless LLC

Title:	Pseudo-CR on enhancements to spatial map subscription procedures

Spec:	3GPP TS 23.437 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This contribution provides IE tables for spatial map subscription procedures.

2. Reason for Change

Following changes are made:

1) Added IE tables for Subscribe Spatial map, Notify Spatial map, Unsubscribe Spatial map procedures.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.2.0).

* * * First Change * * * *

### 9.3.6	Subscribe spatial map event

Figure 9.3.6-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to subscribe the spatial map event. The service is provided by SEAL SM server. Spatial map consumer can get notification information when the the SM server detects subscribed event.

Figure 9.3.6-1: Subscribe spatial map event procedure

1)		The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, a spatial map ID, event to subscribe which triggers the notification. The request message may also include a list of spatial map layers with its corresponding layer IDs to subscribe spatial map layering information related events. The requestor (VAL server or SEAL SM client) may subscribe to one or more of the following events:

a)	"changes of objects" events to indicate the SM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for "changes of objects" event may include added or removed objects, position or direction changes of the spatial map objects.

b)	"layers modification" events to indicate the SM server to notify the requestor when any layer in the spatial map is added or updated or removed.

c)	"spatial map employed" events to indicate the SM server to notify the requestor when the SEAL SM server has sufficient information to map all the stationary and moving objects related to area of interest the spatial map and the spatial map is employed for localization. This event is used when the result IE in the Create spatial map response indicates "in-progress".

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then server add the requestor into the subscriber list of the event, SM server starts monitoring the events.

3)	The SEAL SM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.

#### 9.3.6.1	Information flows for Subscribe spatial map

Table 9.3.6.1-1 shows the subscribe spatial map event request sent by VAL server (or SEAL SM client) to SEAL SM server to subscribe for spatial map event(s).

Table 9.3.6.1-1: Subscribe spatial map event request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Expiry time | O | Proposed subscription expiry time |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to subscribe |
| List of events | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | List of events to subscribed. Possible values are: - Change of objects - Layers modification - spatial map employed |
| > specified objects | O | List of specified objects for "change of objects" event |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g. Spatial map is employed and ready for localization service. |
| >> Changes in objects | O (NOTE) | Triggers notification if any or specified (or specified type of) objects are added, removed, or changed their position or direction, etc. |
| >> Layer IDs | O (NOTE) | List of layers. Triggers notification if any or specified spatial map layer change is detected |
| > Notification intervals | O | Specifies when to notify the detected event, Possible values are: - Event based (e.g., promptly); Or - Periodically (with minimum time between consecutive notifications) - intervals (i.e. minimum time between consecutive notifications) |
| > interval time | O | Time between consecutive notifications (applicable only if notification intervals is set to intervals. |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

Editor’s Note:	Details about "layers" event is FFS.

Editor’s Note:	Additional events (e.g. change of access control rules) is FFS.

Table 9.3.6.1-2 shows the subscribe spatial map event response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the subscribe spatial map event request.

Table 9.3.1.1-2: Subscribe spatial map event response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the subscribe spatial map event request |
| Subscription Id (NOTE 1) | O | Identity of the subscription |
| Expire time | O | Subscription expire time |
| Failure cause | O | Indicates reason for the failure. |
| NOTE 1: This IE is mandatory when Result IE indicates success. |  |  |

### 9.3.7	Notify spatial map event

Figure 9.3.7-1 depicts the notify spatial map event operation between the authorized spatial map consumer (VAL server or SEAL SM client) and SEAL SM server.

Figure 9.3.7-1: Notify spatial map event

1)		The SEAL SM server detects an event that triggers the notification to the subscribers for the corresponding event.

a)	If the subscribed event is "changes of objects", the SM server detects any changes of the spatial map objects.

b)	If the subscribed event is "Layers modification", the SM server detects any changes of the spatial map layers information.

a)	If the subscribed event is "spatial map employed", the SM server detects when the SEAL SM server has sufficient information to map all the stationary and moving objects related to area of interest the spatial map and the spatial map is employed and ready for localization.

SM server determines whether the detected event matchs triggering criteria and notifies subscribers accordingly.

2)	The SEAL SM server notifies the VAL servers (or SEAL SM clients) which subscribed the corresponding event. The notification includes a subscription ID and event-specific information. If the event is "changes of objects", spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map etc. If the event is "Layer modification", the notification message includes the list of all layers for the spatial map.

#### 9.3.7.1	Information flows for Notify spatial map event

Table 9.3.7.1-1 shows the spatial map notification sent by SEAL SM server to VAL server (or SEAL SM client) to notify about spatial map event(s).

Table 9.3.7.1-1: Spatial map event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| > Spatial map ID | M | ID of the spatial map for which the event is detected |
| > Event type | M | Type of event detected. One of the event as specified in Table 9.3.6.1-1. |
| >> Changed object | O (NOTE) | Information on the object which triggered the notification |
| >>> Object ID | O | ID of the changed object |
| >>> Type of change | O | Specifies the detected type of change for the object (e.g., added, removed, or changed position or direction) |
| >>> Changed object information | O | A container to carry the changed object specific information |
| >> list of layers (NOTE 2) | O | Information about layers for the spatial map. |
| NOTE 1: This IE is applicable for "Change of objects" event type. NOTE 2: This IE is applicable for "Layer modification" event type. |  |  |

Editor’s Note: Detils on "Changed object information" is FFS.

### 9.3.8	Unsubscribe spatial map

Figure 9.3.8-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to unsubscribe from the spatial map event. The service is provided by SEAL SM server.

Figure 9.3.8-1: Unsubscribe spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to unsubscribe from spatial map event. The request includes requestor ID, security credentialsand the subscription ID for the event.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then SM server removes the requestor’s subscription ID from the subscriber list for the spatial map event.

3)	The SEAL SM server sends unsubscribe spatial map event response message with the result. If successful, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 9.3.8.1	Information flows for Unsubscribe spatial map

Table 9.3.8.1-1 shows the unsubscribe spatial map event request sent by VAL server (or SEAL SM client) to SEAL SM server to subscribe for spatial map event(s).

Table 9.3.8.1-1: Unsubscribe spatial map event request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Subscription ID | M | Subscription ID for the event |

Table 9.3.8.1-2 shows the unsubscribe spatial map event response sent by SEAL SM server to VAL server (or SEAL SM client) as the response to the unsubscribe spatial map event request.

Table 9.3.8.1-2: Unsubscribe spatial map event response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the subscribe spatial map event request |
| Failure cause | O | Indicates reason for the failure. |

* * * End of Change * * * *