3GPP TSG-SA WG6 Meeting #63	S6-244581

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244190)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0061 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN element discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2024-09-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion of the KI#6 of the TR 23.700-21, solution#12 needs to be conisder for normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new procedure to discover PIN element from another PIN element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Metaverse applications on the UE (acting as a PIN element) will not be able to discover another PIN element. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.2.x (NEW), 8.5.8.3.1, 8.5.8.3.x (NEW), 8.5.8.3.y (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.5.8.2.x	PINE discovery within PIN

Figure 8.5.8.2.x.2-1 illustrates procedure of PIN client requests to discover another PIN element, based on request/response model.

Pre-conditions:

1.	The UE (PIN client) has already discovered PIN as specified in clause 8.5.7;

2.	The UE (PIN client) has already joined the PIN as specified in clause 8.5.8.2.

3.	The PIN client has been authorized to communicate with the PEMC and already has application layer connection with PEMC;

4.	Dynamic profile information about the PIN is available at the PEMC and PIN server.

Figure 8.5.8.2.x.2-1: discovery PIN element

1.	The PIN client of a PIN element sends request the PIN Management client to discover another PIN elements. The request includes PIN element identifier, the security credentials of the PIN client received during authorization procedure and may include the UE identifier (such as GPSI), PIN client ID, UE location, PIN ID as an identifier of the PIN and services/capabilities which is required by the requesting PIN element and time duration till when the PIN element is required.

2.	The PEMC authorizes the PIN client of a PIN element and discovers list of PIN elements based on provided parameters in the request and on information of PIN Element registered in the PIN. The PIN elements registered in the PIN may be known at the PIN Management Client (e.g., in the PIN dynamic profile) or otherwise may be obtained by the PIN Management Client from the PIN Server.

3.	The PIN Management client sends response back to the PIN client. The response contains list of PIN elements along with PIN client profiles for each discovered PIN elements, in case of success. The response includes failure cause in case of failure.

Editor’s note:	Enhancement to PIN profile regarding task ofliading capability information is FFS.

NOTE:	If multiple PIN elements are discovered, then how to select the best PIN element to offload the task is up to application level implementation.

* * * Next Change * * * *

##### 8.5.8.3.1	General

The following information flows are specified for PIN creation:

-	PIN Management PINE join into PIN request and response;

-	PIN Management PINE leave from PIN request and response;

-	PIN Management PINE discovery request and response;

* * * Next Change * * * *

##### 8.5.8.3.x	PIN Management PINE discovery request

Table 8.5.8.3.x-1 shows the request sent by a PIN client of a PIN element to a PEMC for the PIN element discovery request.

Table 8.5.8.3.x-1: PIN Management PINE discovery request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to join in. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |
| Discovery filters | M | Provides discovery filters to discover PIN elements |
| > UE location | O | The location information of the UE. The UE location is described in clause 7.2.7. |
| > required PIN services | O | Specifies PIN services as required by the PIN element (as specified in Table 8.2.2.1-1). |
| > time duration | O | Specifies time duration till when the service from the PIN element is required |

##### 8.5.8.3.y	PIN Management PINE discovery response

Table 8.5.x.3.3-1 shows the response sent by the PEMC to the requester for a PIN element discovery response.

Table 8.5.8.3.y-1: PIN Management PINE discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Discovered PIN elements List | O | List of discovered PIN element(s). Each element includes the information described below |
| > PIN client ID | M | Identifier of the PIN client. |
| > PIN client profile | M | PIN client profile |

* * * End of Change * * * *