3GPP TSG-SA WG6 Meeting #63	S6-244608

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244297)

Source:	Ericsson

Title:	Support of VFL at Enablement Layer

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedure for supporting VFL to TS 23.482.

2. Reason for Change

Solution#18 is recommended for normative work on procedure to support Vertical FL (VFL). See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for VFL to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for VFL to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FL		Federated Learning

VFL	Vertical FL

* * * Next Change * * * *

## 8.x	Support Vertical FL

This clause describes procedure for supporting VFL among application layer multiple UEs.

### 8.x.1	General

The following clauses specify procedures, information flows and APIs to support VFL among Application Layer multiple UEs.

### 8.x.2	Procedure for supporting VFL

Pre-conditions:

-	VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server.

-	The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains.

-	The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications.

Figure 8.x.2-1: Procedure for supporting VFL

  - An AIMLE server receives an ML model training assist request (e.g. AIMLE client discovery and/or selection requet as introduced in clauses 8.8, 8.9, and 8.13) or ML model training request from a VAL server as described in clause 8.3.2. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between domains), the procedure continues to step 2.

2.	The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through running the step 2 in the AIMLE client discovery procedure in clause 8.8.2.1). The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for ML model training capability evaluation as described in clause 8.y.2.

3.	The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 2 and parameters received in step 1 (Table 8.3.3.1-1). The criteria used by the AIMLE Server include:

-	Available data and minimum number of data samples for the same sample among the VFL members.

-	Feature alignment of the sample/datasets with data labels among the VFL members.

-	Available time of the VFL members for support the VFL training operations.

-	Capability and minimum number of the VFL members for the VFL training operations.

- AIML model information for the VFL members and for the AIMLE Server.

4.	The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responses the VFL members with the updated information (e.g. gradients). The informaiton from the AIMLE server can be used to update the model parameters maintained at each VFL member.

4a.	The AIMLE server may report to the VAL server with the intermediate training status, the VAL server may adjust its request on the ML model training.

Editor’s Note:	Clarification on step 4a is needed.

Editor’s Note:	The procedure for AIMLE Server coordinates the selected VFL members to perform VFL training is FFS.

Editor’s Note:	Whether and how the AIMLE server monitoring the dynamic changes of the VFL member(s) during the VFL process is FFS.

- The AIMLE server sends an ML model training assist response (e.g. AIMLE client discovery and/or selection response/notification as introduced in clauses 8.8, 8.9, and 8.13) or an ML model training response to the VAL server.

## 8.y	Training Capability Evaluation

This clause describes procedure for supporting training capability evaluation for FL (e.g. HFL, VFL).

### 8.y.1	General

The following clauses specify procedures, information flows and APIs to support training capability evaluation for FL (e.g. HFL, VFL).

### 8.y.2	Procedure for training capability evaluation

Pre-conditions:

-	AIMLE server determines to use FL (e.g. HFL, VFL) training.

-	AIMLE Server knows the information of the FL (e.g. HFL, VFL) members (e.g., AIMLE Client which is deployed to an UE).

Figure 8.y.2-1: Procedure for training capability evaluation

1.	The AIMLE Server sends ML model training capability evaluation request to the FL members (AIMLE Clients which are deployed on UEs). The request message includes information as described in Table 8.y.3.1-1.

2.	The FL members evaluate their capability and availability to join the FL training process. The FL members (AIMLE Clients which are deployed on UEs) run the test task contained in the request in step 1 and determine if it can join the FL process. For VFL, as part of the test task, data alignment between the datasets of the different domains are determined. The VAL server may also provide data labels for the data alignment.

NOTE:	The procedures for data collection from UE need to take user consent into account.

3.	The FL members (AIMLE Clients which are deployed on UEs) send ML model training capability evaluation response to the AIMLE Server. The response message contains the information as described in Table 8.y.3.2-1.

### 8.y.3	Information flows

#### 8.y.3.1	ML model training capability evaluation request

Table 8.y.3.1-1 shows the request sent by AIMLE server to FL (e.g. HFL, VFL) members (AIMLE Clients which are deployed on UEs) for ML model training capability evaluation.

Table 8.y.3.1-1: ML model training capability evaluation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Available time | O | Requirement on available time for supporting FL operations. |
| Test task | O | The task for test ML model training capability. |
| AI/ML model and model parameter(s) | O | Information about the AI/ML model and model parameters for use in FL training. |
| Requirement on dataset | O | Requirements on dataset for FL training. |
| >Feature ID(s) | O (NOTE 1) | Identifier(s) of the required feature of the dataset from the UE. |
| >Domain(s) | O (NOTE 1) | Domain(s) of the datasets at the UE. |
| >Data source | O (NOTE 2) | Data source for the FL training. |
| NOTE 1: If Requirement on dataset is provided, at least one of these IEs shall be present when for VFL. NOTE 2: If Requirement on dataset is provided, the IE shall be present when for HFL. |  |  |

NOTE:	The detail content of the Test task and AI/ML model is up to implementation.

#### 8.y.3.2	ML model training capability evaluation response

Table 8.y.3.2-1 shows the response sent by FL members (AIMLE Clients which are deployed on UEs) to AIMLE server for the ML model training capability evaluation.

Table 8.y.3.2-1: ML model training capability evaluation response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the evaluaton: success, fail. - Success means join the FL training process. - Fail means not join the FL training process. |
| Test result | O | The test result of the ML model training capapbility evaluation. The "test result" shall be provided when the "status" is "success". |
| Fail reason | O | The reason of the ML model training capapbility evaluation fail. The "fail reason" shall be provided when the "status" is "fail". |

* * * Next Change * * * *

## 9.z	ML model training capability evaluation API

### 9.z.1	General

Table 9.z.1-1 illustrates the API for ML model training capability evaluation. This API enables the AIMLE Server to communicate with the AIMLE client(s) for ML model training capability evaluation.

Table 9.z.1-1: Aimles_MLModelTrainingCapabilityEva API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_MLModelTrainingCapabilityEva | Request | Request/Response | AIMLE Server |

### 9.z.2	Aimles_MLModelTrainingCapabilityEva_Request operation

API operation name: Aimles_MLModelTrainingCapabilityEva_Request

Description: The consumer requests for ML Model training capability evaluation.

Inputs: See clause 8.y.3.1.

Outputs: See clause 8.y.3.2.

See clause 8.y.2 for details of usage of this operation.

# *** End of Changes ***