3GPP TSG-SA6 Meeting #63	S6-244617

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0337 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for sidelink positioning management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A new feature is added to SEAL LMS capabilties based on concluded solution #7 of TR 23.700-72. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper introduces the new capabilities to clause 9.3.x about the SL positioning enablement. This includes the procedure and information flows for the new feature based on concluded solution #7 of TR 23.700-72. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The capability based on the concluded solution will not be added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses: 9.3.x (new), 9.3.x.1 (new). 9.3.2.X (new), 9.3.2.Y (new), 9.3.2.Z (new), 9.3.2.XY (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.x	Support for sidelink positioning / ranging management

#### 9.3.x.1	Procedure of SL positioning /ranging management service

The procedure provides a mechanism for configuring and provisioning information on the entities to be used and their responsibilities for a SL positioning / ranging service. In this procedure, an SL positioning enabler capability at LMS is authorized / delegated by SL positioning server to support the provisioning/delivery of ranging services in an area.

Figure 9.3.x.1-1 shows a high-level procedure with the details.

Figure 9.3.x.1-1: Support for SL positioning procedure

1.	The VAL server (SL Positioning / Ranging Server) sends an SL positioning management subscription request to SEAL LMS to provision and manage SL positioning / ranging operation and configurations.

2.	The SEAL LMS authorizes the VAL subscription request.

3.	The SEAL LMS sends an SL positioning management subscription response to the requestor VAL Server.

4.	The SEAL LMS discovers all the SEAL LMCs in the target service area, and categorizes them by their capabilities as target UE, Reference UEs, or even Located UEs (if Reference UE location is known or for out of coverage scenarios). The identification of UEs in a target area happens via existing SEAL LMS procedures (as in clauses 9.3.10 and 9.3.12).

5.	The SEAL LMS after getting all UE information at the target area, calculates the relative distances among them, and may request/receive HDMaps (e.g., from other VAL servers) for the target area.

Then, SEAL LMS evaluates the need of reference UE (based the information in step 4), and if reference UE is needed (e.g. in NLoS situation), the SEAL LMS selects the appropriate VAL UE to serve as reference and configures the VAL UEs (target, reference, client) with configuration information (thresholds, reporting configurations).

The LMS may also leverage analytics services (e.g. NWDAF) to obtain VAL UE location related predictions (e.g. UE location prediction, relative distance prediction) to discover reference UEs.

NOTE: The selection of UE as reference UEs can be based on their capabilities and the selection criteria specified by the VAL server.

6.	The SEAL LMS sends a SL positioning management notify message to the VAL server to indicate the roles of the VAL UEs and acknowledging that the SL positioning provisioning is performed.

7.	The SEAL LMS configures the SEAL LMCs with the configuration information applicable to the corresponding VAL UEs based on step 5, and in particular the expected role (acting as reference UE, target UE) in relation ranging/SL positioning service. Based on this configuration, the LM clients interacts each other using off-network location management procedures as described in clause 9.5.

* * * Second Change * * * *

#### 9.3.2.X	SL positioning management subscription request

Table 9.3.2.X-1 describes the information flow from the VAL server to the location management server for SL positioning management subscription request.

Table 9.3.2.X-1: SL positioning management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server. |
| VAL service ID | M | Identity of the VAL service (SL positioning service) for which the location information is subscribed. |
| VAL service area | O | The geographical area /zone or topological area (e.g. cells) for which the subscription applies. |
| VAL UE/user Identities list | O | List of VAL users or VAL UEs participating at the SL positioning service (reference and target VAL UEs). |
| VAL UE selection criteria | O | Selection criteria of UEs of different roles (e.g. reference UE). |
| Reporting configuration | M | It indicates the threshold and interval time between consecutive reports. |
| > Triggering criteria | O | If reporting is expected to happen based on certain conditions in future instance, this IE provides information on the conditions to trigger reporting. |
| SL positioning parameters | O | The SL positioning parameters to be reported (e.g. distance, direction or both), their frequency, and the permissions of the VAL UEs in the target service area. |

#### 9.3.2.Y	SL positioning management subscription response

Table 9.3.2.Y-1 describes the information flow from the location management server to the VAL server for SL positioning management subscription response.

Table 9.3.2.Y-1: SL positioning management subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result which is success or failure. |
| Subscription identity | O (see NOTE) | If subscription is successful, identifies the subscription |
| NOTE: This IE shall be present if subscription status is set to success. |  |  |

#### 9.3.2.Z	SL positioning management notification

Table 9.3.2.Z-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.Z-1: SL positioning management notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription. |
| Reference and target UE ID (s) and address | M | Identifies the list of UEs to serve as reference and target UEs. |
| List of Client UE IDs and addresses | O | The identities of the client UEs which are allowed to get ranging information for other UEs. |
| Triggering event | O | Identity of the event that triggered the sending of the notification (success or failure). |
| Reason | O | It indicates the failure reason for the notification report (e.g., "reference UE cannot be found"). |

#### 9.3.2.XY	SL positioning configuration command

Table 9.3.2.XY-1 describes the information flow from the location management server to the location management clients or from LMC to VAL client.

Table 9.3.2.XY-1: SL positioning configuration command

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requestor SEAL server or the subscriber VAL server |
| VAL service ID | M | Identity of the VAL service (SL positioning service) |
| VAL service area | M | The geographical area /zone or topological area (e.g. cells) for which the subscription applies. |
| VAL UE/user Identities list | M | List of VAL users or VAL UEs participating at the SL positioning service (reference and target VAL UEs). |
| SL positioning / Ranging configuration parameters | M | The configuration information for the SL positioning service. This includes the roles of the VAL UEs (reference or target) in the SL positioning service. |
| Reporting configuration | M | Indicates the threshold and interval time between consecutive reports expected from the target VAL UEs. |
| >Triggering criteria | O | If reporting is expected to happen based on certain conditions in future instance, this IE provides information on the conditions to trigger some reporting. |

* * * Third Change * * * *

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting Notify_Adaptive_Configuration | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_SLPositioningManagement | Subscribe_SLPositioningManagement | VAL server | Subscribe/Notify |
|  | Notify_SLPositioningManagement | VAL server |  |

* * * Fourth Change * * * *

### 9.4.x	SS_SLPositioningManagement API

#### 9.4.x.1	General

API description: This API enables the VAL server to subscribe to SEAL LMS for configuring VAL UEs for a SL positioning / ranging service.

#### 9.4.x.2	Subscribe_ SLPositioningManagement

API operation name: Subscribe_ SL_Positioning_Management

Description: Subscription to SL positioning management.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.X

Outputs: 9.3.2.Y.

See subclause 9.3.x.1 for the details of usage of this API operation.

#### 9.4.x.3	Notify_SLPositioningManagement

API operation name: Notify_ SL_Positioning_Management

Description: Notifying the VAL server about the success or failure of the SL positioning service configuration.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.z

Outputs: None.

See subclause 9.3.x.4 for the details of usage of this API operation.